IEEE P802.11
Wireless LANs

| Group Addressed (GA) GAS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-02-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |
| Michael Montemurro | BlackBerry Ltd | 4701 Tahoe Blvd, Mississauga, ON., L4W 0B4, CANADA | +1-289-261-4183 | mmontemurro@blackberry.com |

Abstract

This document proposes text changes to update the P802.11aq D7.0 for Group Addressed GAS functionality

- Need signalling for GA-GAS

9.4.1.34,

9.4.2.93,

9.6.8.12 – 9.6.8.15,

11.25.3,

11.25.4,

11.25.7,

11.44.4,

11.44.5,

11.44.7,

Annex C,

Annex R.2

Group-addressed GAS  Model:

- Define GA-GAS Action type for GAS Request/Response that are sent as group-addresssd frames

- Add an optional element to signal that a non-AP STA supports GA-GAS responses in a unicast GAS request.

  - Include element in Initial GAS Request frames

- Rule-out GAS Comeback later responses from GA-GAS response frames.

  - AP cannot respond with a non-zero Comeback delay in GA-GAS Initial response

- PAD-capable STAs support GA-GAS request/response

- Dialog token:

  - List STA MAC – dialog token pair with query result.

- Need to define protected dual versions for GA-GAS (if PMF is enabled, the AP shall use Protected Dual for Response)

- Behavior:

  - Non-AP STA

    - Supports GA-GAS if PAD capable

    - Can either send a unicast or group-addressed GAS

      - For a unicast request, will receive

        - A unicast response

        - A unicast come-back later

        - A GA response

      - For a group-addressed request will receive for each AP on the channel

        - A unicast response

        - A unicast come-back later

        - A GA response

        - No response at all

  - AP:

    - Supports GA-GAS if PAD capable

    - Receives either a unicast or group-address GAS Request

      - For a received unicast request

        - Processes unicast request and transmits a unicast response (which may have zero length content, if match is not found)

        - Processes unicast request and transmits a come-back later

        - Processes unicast request, aggregates the response with other compatible responses, and sends an GA-GAS response containing all aggregated responses.

      - For a received group-addressed request:

        - Processes group-addressed request, finds no match and discards request

        - Processes group-addressed request, finds a match, and responds with a unicast response

        - Processes group-addressed request, finds a match, and responds with a unicast come-back later

        - Processes group-addressed request, finds a match, aggregates the response with other compatible responses, and transmits a GA-GAS response containing all aggregated responses.

Adopt the following marked changes updating the clause numbering as appropriate:

6.3.73.2 MLME-GAS.request

6.3.73.2.1 Function

This primitive requests the information of a specific advertisement service from another STA and requests the STA to provide GAS.

## 6.3.73.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.request (

STAAddress,

DialogToken,

AdvertisementProtocolID,

Query,

QueryFailureTimeout,

Protected,

Multi-band,

GAmode,

GASextensions,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| STAAddress | MacAddress | Any valid individual MacAddress. For a group address GAS frame, this is the broadcast address. | Specifies the address of the MAC entity to which query is transmitted or the broadcast address. |
| DialogToken | Integer | 0–255 | The dialog token to identify the GAS transaction. |
| … | … | … | … |
| Multi-band | Multi-band element | As defined in 9.4.2.138 | Specifies the frequency band and channel number to which the GAS transaction applies. The parameter is absent if the GAS transaction applies to the same frequency band and channel where the frame is transmitted. |
| GAmode | Boolean | true, false | If true, the request is sent using a group address GAS frame. Otherwise, the request is sent using a unicast GAS frame. |
| GASextensions | GAS extensions element | As defined in 9.4.2.218 | Specifies the GAS extensions for a transmitting STA that supports group address GAS frames. The parameter is absent if the transmitting STA does not support group address GAS. |
| VendorSpecificInfo | A set of elements | As defined in 9.4.2.26 | Zero or more elements. |

6.3.73.2.3 When generated

This primitive is generated by the SME to request a specific Advertisement Service from another STA.

6.3.73.2.4 Effect of receipt

The STA operates according to the procedures defined in 11.25.3 and 11.25a.

6.3.73.3 MLME-GAS.confirm

6.3.73.3.1 Function

This primitive reports the status code and Query Response from an Advertisement Server to the requesting STA.

6.3.73.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.confirm (

PeerSTAAddress,

DialogToken,

ResultCode,

ResponseInfo,

Protected,

Multi-band,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress. | Specifies the address of the peer MAC entity from which Query Response is received . |
| DialogToken | Integer | 0–255 | The dialog token to identify the GAS transaction. |
| … | … | … | … |

The mapping of Status Code received in the GAS Response frame is mapped to the corresponding Result

Code in Table 9-46.

6.3.73.3.3 When generated

This primitive is generated by the MLME as a response to the MLME-GAS.request primitive indicating the

result of that request.

The primitive is generated when the requesting STA receives a query response in a (Protected) GAS Initial

Response frame or one or more (Protected) GAS Comeback Response frames or a GA GAS Response frame.

6.3.73.3.4 Effect of receipt

The STA operates according to the procedures defined in 11.25.3 and 11.25a.

6.3.73.4 MLME-GAS.indication

6.3.73.4.1 Function

This primitive reports to the STA’s SME about the GAS Request.

6.3.73.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.indication (

PeerSTAAddress,

DialogToken,

AdvertisementProtocolID,

Query,

Protected,

Multi-band,

GAS extensions.

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress. | Specifies the address of the peer MAC entity from which the query was received |
| DialogToken | Integer | 0–255 | The dialog token to identify the GAS transaction. |
| … | … | … | … |
| GASextensions | GAS extensions element | As defined in 9.4.2.218 | Specifies the GAS extensions for a requesting STA that supports group address GAS frames. |

6.3.73.4.3 When generated

This primitive is generated by the MLME as a result of receipt of a GAS request from STA.

6.3.73.4.4 Effect of receipt

The SME is notified of the request from the STA.

The SME operates according to the procedures defined in 11.25.3 and 11.25a.

The SME generates an MLME-GAS.response primitive within a dot11GASResponseTimeout.

6.3.73.5 MLME-GAS.response

6.3.73.5.1 Function

This primitive responds to the request for an advertisement service by a specified STA MAC entity.

6.3.73.5.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.response (

STAAddress,

DialogToken,

ResultCode,

ResponseInfo,

Protected,

Multi-band,

GAmode,

GAS extensions,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| STAAddress | MacAddress | Any valid individual MacAddress. For a group address GAS frame, this is the broadcast address. | Specifies the address of the MAC entity to which the query response information is transmitted or the broadcast address. |
| DialogToken | Integer | 0–255 | The dialog token to identify the GAS transaction. |
| … | … | … | … |
| Multi-band | Multi-band element | As defined in 9.4.2.138 | Specifies the frequency band and channel number to which the GAS transaction applies. The parameter is absent if the GAS transaction applies to the same frequency band and channel where the frame is transmitted. |
| GAmode | Boolean | true, false | Specifies whether the response is sent using a group address GAS frame. If true, the response is sent using a group address GAS frame. Otherwise, the response is sent using a unicast GAS frame. |
| GAS extensions | GAS extensions element | As defined in 9.4.2.218 | Specifies the GAS extensions for one or more receiving STAs that support group address GAS frames. The parameter is absent if this frame is a group address GAS frame or if the transmitting STA does not support group address GAS. |
| VendorSpecificInfo | A set of elements | As defined in 9.4.2.26 | Zero or more elements. |

6.3.73.5.3 When generated

This primitive is generated by the MLME as a result of an MLME-GAS.indication primitive.

6.3.73.5.4 Effect of receipt

This primitive causes the MAC entity at the STA to send a (Protected) GAS Initial Response frame to the requesting STA and optionally one or more (Protected) GAS Comeback Response frames or one GA GAS Response frame.

##### 9.4 Management and Extension frame body components

- Fields that are not elements(#1459)

      - Status Code field

Change Table 9-46 by inserting the following row (ignoring the header row) in the table, maintaining numerical order

Table 9-46— Status Code field

| Status code | Name | Meaning |
| --- | --- | --- |
| <ANA> | GAS_FRAGMENT_NOT_AVAILABLE | The requested GAS fragment is not available |

###### 9.4.2 Elements

###### 9.4.2.1 General

Change Table 9-77 by inserting the following rows (ignoring the header row) in the table, maintaining numerical order:

Table 9-77— Element IDs

| Element | Element ID | Element ID Extension | Extensible | Fragmentable |
| --- | --- | --- | --- | --- |
| Service Hint (see 9.4.2.217) | 255 | 15 |  |  |
| Service Hash (see 9.4.2.218) | 255 | 16 |  |  |
| GAS Extentions (See 9.4.2.218) | 255 | <ANA> | Yes | Yes |

Insert new subclauses as follows after the Service Hint element sub-clause, using the clause numbering shown and maintaining numbering order:

###### GAS Extensions

The GAS Extensions element is defined in Figure 9-xxx1. When present in the GAS Initial Request frame, the GAS Extensions element indicates the STA capability to receive a group-addressed GAS Initial response frame.

|  | Element ID | Length | Element ID Extension | GAS Flags | Maximum Channel Time | Fragment ID | Number of Response Map Duples | Response Map Duples |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 1 | 1 | 1 | 0 or 1 | 0 or 1 | Variable |

###### Figure 9-xxx1—GAS Extensions element format

The GAS Flags field is a one octet field that is defined in Figure 9-xxx2.

|  | B0 | B1 | B2 | B3 B7 B7 |  |
| --- | --- | --- | --- | --- | --- |
|  | Group-addressed GAS | Fragment ID Flag | Response Map Flag | Reserved |  |
| Octets: | 1 |  |  |  |  |

Figure 9-xx2 GAS Flags field

Group-addressed GAS sub-field is set to 1 to indicate that the STA support group-addressed GAS and is set to 0 otherwise. The Fragment ID Flag sub-field is set to 1 to indicate that Fragment ID field is present in the element and is set to 0 otherwise. The Response Map Flag sub-field is set to 1 to indicate that Number of Response Map Duples and the Response Map Duples fields are present in the element and is set to 0 otherwise.

The Maximum Channel Time field is a one octet field that indicates the maximum amount of time the STA will or needs to remain on the channel to receive  a GAS Initial Response, a GAS Comeback Response, or Group-addressed GAS Response, expressed as a multiple of 10 TUs beginning from the end of the PPDU carrying this element. The field has a valid range of 5-255.

The Fragment Id field is a 1 octet field in the GAS Comeback Request to indicate which fragment the STA is requesting. It is only present when the Fragment ID Flag sub-field is set to 1 in the GAS flags field.

The Number of Duples field is a one octet field that indicates the number of Response Map duple fields. It is only present when the Response Map Flag subfield is set to 1 in the GAS Flags field

The Response Map Duples field contains one or more Response Map Duple subfields. The format of the Response Map Duple sub-field is shown in Figure 9-xxx3. The Response Map Duples field is included in a Group-addressed GAS Response frame.

|  | Requestor MAC Address |  | Requestor Dialog Token |
| --- | --- | --- | --- |
| Octets: | 6 | 1 |  |

###### Figure 9-xxx3—Response Map Duple subfield format

The Response Map duple contains a Requestor MAC Address and  Requestor Dialog Token subfield. An AP or PCP includes one or more Response Map Duples in a Group-addressed GAS Response frame

## 9.6.8.1 Public Action frames

Insert new rows at the end of the table and update the “Reserved” field as follows:

Table 9-307—Public Action field values

| Public Action field value | Description |
| --- | --- |
| <ANA> | Group Addressed GAS Request |
| <ANA> | Group Addressed GAS Response |
| 255 | Reserved |

- GAS Initial Request frame format

The GAS Initial Request frame is a Public Action frame. It is transmitted by a requesting STA to request information from another STA. The format of the GAS Initial Request Action field(#2042) is shown in GAS Initial Request Action field.

| - GAS Initial Request Action field(#2042) format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Public Action(Ed) |
| 2 | Dialog Token |
| 3 | Advertisement Protocol element |
| 4 | Query Request length |
| 5 | Query Request |
| 6 | Multi-band (optional) |
| 7 | GAS Extensions (optional) |

The Category field is defined in 9.4.1.11 (Action field).(#3403)

The Public(Ed) Action field is defined in Error: Reference source not found.(#3403)

The Dialog Token field is defined in 9.4.1.12 (Dialog Token field) and set by the requesting STA.

The Advertisement Protocol element is defined in 9.4.2.93 (Advertisement Protocol element). The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Request length field is defined in Query Request length field. The value of the Query Request length field is set to the total number of octets in the Query Request field.

|  | B0 B15 |
| --- | --- |
|  | Query Request length |
| Octets: | 2 |
| - Query Request length field |  |

The Query Request field is defined in Query Request field. The Query Request field is a generic container whose value is a GAS Query that is formatted in accordance with the protocol identified(#1638) in the Advertisement Protocol element.

|  | Query Request |
| --- | --- |
| Octets: | variable |
| - Query Request field |  |

When present in a GAS Initial Request frame, the Multi-band element indicates the frequency band, operating class, and channel number to which the GAS Initial Request frame applies.

When present in a GAS Initial Request frame, the GAS Extensions element indicates the STA support for GAS extensions in GAS frame exchanges.

- GAS Initial Response frame format

The GAS Initial Response frame is a Public Action frame. It is transmitted(#3104) in response responding to a GAS Initial Request frame or a Group Addressed GAS Request frame. The format of the GAS Initial Response Action field(#2042) is shown in GAS Initial Response Action field.

| - GAS Initial Response Action field(#2042) format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Public(Ed) Action |
| 2 | Dialog Token |
| 3 | Status Code |
| 4 | GAS Comeback Delay |
| 5 | Advertisement Protocol element |
| 6 | Query Response Length |
| 7 | Query Response (optional) |
| 8 | Multi-band (optional) |

The Category field is defined in 9.4.1.11 (Action field).(#3403)

The Public(Ed) Action field is defined in Error: Reference source not found.(#3403)

The Dialog Token field is copied from the corresponding GAS Initial Request frame .

The Status Code values are defined in Table 9-46 (Status codes).

The GAS Comeback Delay field specifies the delay time value in TUs. The GAS Comeback Delay field format is provided in GAS Comeback Delay field. The behavior is described in 11.25.3.1 (GAS Protocol). The value 0 will be returned by the STA when a Query Response is provided in this frame.

|  | B0 B15 |
| --- | --- |
|  | GAS Comeback Delay |
| Octets: | 2 |
| - GAS Comeback Delay field |  |

The Advertisement Protocol element is defined in 9.4.2.93 (Advertisement Protocol element). The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Response Length field is defined in Query Response length field. The value of the Query Response Length field is set to the total number of octets in the Query Response field. If the Query Response Length field is set to 0, then there is no Query Response included in this Action frame.

|  | B0 B15 |
| --- | --- |
|  | Query Response Length |
| Octets: | 2 |
| - Query Response length field |  |

The Query Response field is defined in Query Response field. The Query Response field is a generic container whose value is the response to a GAS Query and is formatted in accordance with the protocol specified in the Advertisement Protocol element.

|  | Query Response |
| --- | --- |
| Octets: | variable |
| - Query Response field |  |

When present in a GAS Initial Response frame, the Multi-band element indicates the frequency band, operating class, and channel number to which the GAS Initial Response frame applies.

- GAS Comeback Request frame format

The GAS Comeback Request frame is a Public Action frame. It is transmitted by a requesting STA to a responding STA. The format of the GAS Comeback Request Action field(#2042) is shown in GAS Comeback Request Action field.

| - GAS Comeback Request Action field(#2042) format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Public(Ed) Action |
| 2 | Dialog Token |
| 3 | Multi-band (optional) |
| 4 | GAS Extensions (optional) |

The Category field is defined in 9.4.1.11 (Action field).(#3403)

The Public(Ed) Action field is defined in Error: Reference source not found.(#3403)

The Dialog Token field is copied from the corresponding GAS Initial Request frame.

When present in a GAS Comeback Request frame, the Multi-band element indicates the frequency band, operating class, and channel number to which the GAS Comeback Request frame applies.

When present in a GAS comeback Request frame, the GAS Extensions element indicates a request to retransmit a GAS Query Response fragment.

- GAS Comeback Response frame format

The GAS Comeback Response frame is a Public Action frame. It is transmitted by a responding STA to a requesting STA. The format of the GAS Comeback Response Action field(#2042) is shown in GAS Comeback Response Action field.

| - GAS Comeback Response Action field(#2042) format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Public(Ed) Action |
| 2 | Dialog Token |
| 3 | Status Code |
| 4 | GAS Query Response Fragment ID |
| 5 | GAS Comeback Delay |
| 6 | Advertisement Protocol element |
| 7 | Query Response Length |
| 8 | Query Response (optional) |
| 9 | Multi-band (optional) |
| 10 | GAS Extensions (optional) |

The Category field is defined in 9.4.1.11 (Action field).(#3403)

The Public(Ed) Action field is defined in Error: Reference source not found.(#3403)

The Dialog Token field is copied from the Dialog Token field of the corresponding GAS Comeback Request frame or GA GAS Request frame. The same value of the Dialog Token field(#6545) is(MDR) present in all fragments of a multi-fragment query response.

The Status Code values are defined in Table 9-46 (Status codes). The same status code value will be present in all fragments of a multi-fragment query response.

The GAS Query Response Fragment ID is defined in 9.4.1.34 (GAS Query Response Fragment ID field). If the responding STA has not received a response to the query that it posted on behalf of a requesting STA, then the responding STA sets the GAS Query Response Fragment ID to 0. When there is more than one query response fragment, the responding STA sets the GAS Query Response Fragment ID to 0 for the initial fragment and increments it by 1 for each subsequent fragment in a multi-fragment Query Response. The More GAS Fragments field is set to 0 whenever the final fragment of a query response is being transmitted. A GAS Query Response Fragment ID field having a nonzero Fragment ID and the More GAS Fragments field set to 1 indicates to the requesting STA that another GAS Comeback frame exchange should be performed to continue the retrieval of the query response.

The GAS Comeback Delay field format is provided in GAS Comeback Delay field. A nonzero GAS Comeback Delay value is returned by the responding STA in this frame to indicate that the GAS Query being carried out on behalf of the requesting STA is still in progress.

- A nonzero value indicates to the requesting STA that another GAS Comeback frame exchange should be performed after the expiration(#5402) of the GAS Comeback Delay timer in order to retrieve the query response.

- This field is set to 0 for all GAS Comeback Response frames containing a query response or a fragment of a multi-fragment query response.

The Advertisement Protocol element is defined in 9.4.2.93 (Advertisement Protocol element). The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Response Length field is defined in Query Response length field. The value of the Query Response Length field is the total number of octets in the Query Response field. If the Query Response Length field is set to 0, then there is no Query Response included in this Action frame.

The Query Response field is defined in Query Response field. The value of the Query Response field is a generic container dependent on the advertisement protocol specified in the Advertisement Protocol element and the query itself. In a multi-fragment query response, the response to the query posted on behalf of a requesting STA is fragmented such that each fragment to be transmitted fits within the MMPDU size limitation.

When present in a GAS Comeback Response frame, the Multi-band element indicates the frequency band, operating class, and channel number to which the GAS Comeback Response frame applies.

When present in a GAS Comeback Response frame, the GAS Extensions element indicates a response to a request for retransmission of a GAS Query Response fragment.

Insert the following new clauses at the end of Clause 9.6.8, updating the clause numbering as appropriate:

9.6.8.34 Group Addressed (GA) GAS Request frame format

The Group Addressed (GA) GAS Request frame is a Public Action frame. It is transmitted by a requesting STA to request information from another STA. The format of the GAS Initial Request Action field(#2042) is shown in GAS Initial Request Action field.

| - GA GAS Request Action field(#2042) format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Public Action(Ed) |
| 2 | Dialog Token |
| 3 | Advertisement Protocol element |
| 4 | Query Request length |
| 5 | Query Request |
| 6 | Multi-band (optional) |
| 7 | GAS Extensions |

The Category field is defined in 9.4.1.11 (Action field).(#3403)

The Public(Ed) Action field is defined in Error: Reference source not found.(#3403)

The Dialog Token field is defined in 9.4.1.12 (Dialog Token field) and set by the requesting STA.

The Advertisement Protocol element is defined in 9.4.2.93 (Advertisement Protocol element). The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Request length field is defined in Query Request length field. The value of the Query Request length field is set to the total number of octets in the Query Request field.

The Query Request field is defined in Query Request field. The Query Request field is a generic container whose value is a GAS Query that is formatted in accordance with the protocol identified(#1638) in the Advertisement Protocol element.

When present in a GA GAS Request frame, the Multi-band element indicates the frequency band, operating class, and channel number to which the GA GAS Request frame applies.

The GAS Extensions element indicates parameters that the AP or PCP may use to in transmiting a response back to the requesting STA.

9.6.8.35 Group Addressed (GA) GAS Response frame format

The Group Addressed (GA) GAS Response frame is a Public Action frame. It is transmitted(#3104) in response to a GAS Initial Request frame or GA GAS Request frame. The format of the GA GAS Response Action field(#2042) is shown in GAS Initial Response Action field.

| - GA GAS Response Action field(#2042) formatinput.doc |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Public(Ed) Action |
| 2 | Dialog Token |
| 3 | Status Code |
| 4 | GAS Comeback Delay |
| 5 | Advertisement Protocol element |
| 6 | Query Response Length |
| 7 | Query Response (optional) |
| 8 | Multi-band (optional) |
| 9 | GAS Extensions |

The Category field is defined in 9.4.1.11 (Action field).(#3403)

The Public Action field is defined in Error: Reference source not found.(#3403)

The Dialog Token field is copied from the corresponding GAS Initial Request frame or GA GAS Request.

The Status Code values are defined in Table 9-46 (Status codes).

The GAS Comeback Delay field specifies the delay time value in TUs. The GAS Comeback Delay field format is provided in GAS Comeback Delay field. The behavior is described in 11.25.3.1 (GAS Protocol). The value 0 will be returned by the STA when a Query Response is provided in this frame.

The Advertisement Protocol element is defined in 9.4.2.93 (Advertisement Protocol element). The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Response Length field is defined in Query Response length field. The value of the Query Response Length field is set to the total number of octets in the Query Response field. If the Query Response Length field is set to 0, then there is no Query Response included in this Action frame.

The Query Response field is defined in Query Response field. The Query Response field is a generic container whose value is the response to a GAS Query and is formatted in accordance with the protocol specified in the Advertisement Protocol element.

When present in a Group-addressed GAS Response frame, the Multi-band element indicates the frequency band, operating class, and channel number to which the GAS Initial Response frame applies.

When present in a Group-addressed GAS Response frame, the GAS Extensions element indicates parameters to indicate the response to which STAs Query Request.

Adopt the following marked changes updating the clause numbering as appropriate:

- Interworking procedures: generic advertisement service (GAS)

This subclause describes the actions and procedures that are used to invoke GAS. GAS may be used to enable network selection or service discovery for STAs when dot11InterworkingServiceActivated is true. GAS provides transport mechanisms for advertisement services while STAs are in the unassociated state as well as the associated state. This is accomplished via the use of Public Action (#100)frames, which are Class-1 frames. GAS information(#2376) shall be transmitted using Public Action frames. When management frame protection is negotiated, stations shall use individually addressed Protected Dual of Public Action frames instead of Public Action frames.

A GAS frame(#2376) exchange may take place between two STAs; one STA transmits a GAS Query Request and the other STA transmits the GAS Query Response as described in Error: Reference source not found. The Advertisement Protocol transported by the GAS is one of the query protocols in Table 9-215 (Advertisement protocol ID definitions).

A STA may transmit group addressed GAS Query Request. Multiple STAs that receive a group addressed GAS Query Request may send a unicast or group addressed GAS Query Response.

A STA that receives multiple, similar GAS Query Requests from multiple STAs that require the same GAS Query Response, may aggregate the response and transmit a group addressed GAS Query Response.

GAS shall be supported by a STA when dot11InterworkingServiceActivated is true. ANQP shall be supported by a STA when dot11InterworkingServiceActivated is true. Other advertisement protocols shall be supported when the corresponding dot11GASAdvertisementID is present.

A STA(#1289) shall not transmit a GAS Query for any Advertisement Protocol unless that Advertisement Protocol ID is included in the Advertisement Protocol element in a Beacon or Probe (#99)Response frame. The Advertisement Protocol element specifies the Advertisement Protocols that a STA may use to communicate with Advertisement Servers, which may be collocated with a STA or in an external network. The Advertisement Protocol identifies the query language used by the Advertisement Server. The GAS protocol, which is used to transport Queries and Query Responses, is transparent to the Advertisement Protocol.

- GAS Protocol

- General

The presence of the Interworking element in Beacon or Probe Response frames indicates support for the GAS protocol. The additional presence of the GAS Extensions element indicates support for the use of group addressed GAS frames. The presence of the Advertisement Protocol element in Beacon or Probe Response frames indicates the Advertisement Protocol IDs supported in the BSS or IBSS. A STA transmits a GAS Query Request in either a GAS Initial Request or Group Addressed GAS Request frame and the responding STA provides the GAS Query Response or information on how to receive the GAS Query Response, in a GAS Initial Response or Group Addressed GAS Response frame. The GAS Query Response shall be delivered in a single GAS Initial Response frame, a Group Addressed GAS Response frame, or in one or more GAS Comeback Response frames; the GAS Query Response shall not be split between a GAS Initial Response frame and one or more GAS Comeback Response frames. The GAS message sequence diagrams are shown in Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found.

Error: Reference source not found describes the GAS frame(#2376) exchange sequence when dot11GASPauseForServerResponse is true and the result of the GAS query(#333) fits within one MMPDU.

Error: Reference source not found describes the GAS frame(#2376) exchange sequence when dot11GASPauseForServerResponse is true and the result of the GAS query(#333) is too large to fit in one MMPDU and GAS fragmentation is used for delivery. The number of GAS Comeback Request and GAS Comeback Response frames(#2376) depends on the number of GAS fragments required for delivery of the (#333)result of the GAS query.

Error: Reference source not found describes the GAS frame(#2376) exchange sequence when dot11GASPauseForServerResponse is false. The number of GAS Comeback Request and GAS Comeback Response frames(#2376) depends on the number of GAS fragments required for delivery of the GAS Query Response.

Figure xx1 describes the GAS frame(#2376) exchange sequence when a STA sends a Group Addressed GAS Query Request frame to a group of STAs. The receiving STA may or may not respond to the GAS query request. If the receiving STA responds it may respond with either a unicast GAS Query Response frame or a Group Addressed GAS Query Response frame.

Figure xx1 – Group addressed GAS Query Request exchange sequence

Figure xx2 describes the GAS frame(#2376) exchange sequence when multiple STAs send a GAS Query Request frame. The receiving STA may respond with a GAS Query Response frame or may aggregate responses and respond with a Group Addressed GAS Query Response frame.

Figure xx2 – Group addressed GAS Query Response exchange sequence

Figure xx3 describes the GAS frame(#2376) exchange sequence when a STA issues a GAS Initial Request Frame, and during the course of receiving a fragmented GAS Comeback Response frame, requests a specific fragment for retransmission from the transmitting STA.

Figure xx3 – Group addressed GAS Query Request for a specific fragmentsequence

- STA procedures to transmit a GAS Query

Upon receipt of an MLME-GAS.request primitive, the requesting STA shall engage in the following procedure to transmit a query:

- The requesting STA sends a GAS Query by transmitting a GAS Initial Request frame containing a Dialog Token, an Advertisement Protocol element containing an Advertisement Protocol ID and the GAS Query in the Query Request field. If the GAS Initial Request frame requests information relating(Ed) to a frequency band different from the frequency band in which the frame is transmitted, the STA shall include a Multi-band element in the GAS Initial Request frame with the Band ID, Operating Class, and Channel Number fields set to indicate to which frequency band the GAS Initial Request frame applies, with other fields (#2377)in the Multi-band element being reserved. If the frame requests information relating(Ed) to the frequency band in which the frame is transmitted, a Multi-band element shall not be included in the frame. If the requesting STA transmits a GA GAS, it includes a GAS Extensions element with the GAS flags field indicating support for GA GAS and sets the Maximum Channel Time field to the value of the dot11GASResponseTimer (!! unit conversion necessary !! Probably need to create a new MIB variable, as the value range of this existing MIB variable is not going to work very well. Possibly just divide by 10)divided by 10 and rounded to the nearest integer.

- Upon transmission of the GAS Initial Request frame, the STA shall set a timer, referred to as the dot11GASResponseTimer, equal to dot11GASResponseTimeout or the QueryFailureTimeout parameter provided in the MLME-GAS.request primitive. If both values are present, the timer shall be set to the lesser of the two values.

- If the requesting STA is not in the associated state, it shall remain in active mode until the receipt of a GAS Initial Response frame with the same value of the Dialog Token field as in the GAS Initial Request frame(#6545) or until the expiration(#5402) of the timer, whichever occurs first. If the requesting STA is in the associated state, it may go into power save state until the GAS Initial Response frame is available for receipt or the timer expiration(#5402), whichever occurs first.

- If the dot11GASResponseTimer expires before a GAS Initial Response frame is received, the GAS Query was not successful and the MLME shall issue an MLME-GAS.confirm primitive with ResultCode equal to GAS_QUERY_TIMEOUT(#6062) and shall set the Query Response Length field to 0.

- STA procedures to post a GAS Query to an Advertisement Server

Upon receipt of a GAS Initial Request or a Group Addressed GAS Request frame, an MLME-GAS.indication primitive shall be issued to the STA’s SME. Upon receipt of an MLME-GAS.indication primitive indicating the receipt of a GAS Initial Request frame, the STA shall transmit a GAS Initial Response or Group Addressed GAS Response frame to the requesting STA according to the following procedures. Upon receipt of an MLME-GAS.indication primitive indicating the receipt of a Group Addressed GAS Request frame, the STA may transmit a GAS Initial Response frame to the requesting STA, or transmit a Group Addressed GAS Response frame according to the following procedures. If the requesting STA is in the associated state and in the power save mode, the responding STA shall buffer the MMPDU for transmission according to the procedures in Error: Reference source not found; otherwise the STA shall queue the MMPDU for transmission as follows:

- If the Advertisement Protocol ID in the Advertisement Protocol element does not equal the value contained in any dot11GASAdvertisementID,(MDR) then the STA shall not post the query to an Advertisement Server. The STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to GAS_ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED(#3524) (see Table 9-46 (Status codes)), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame and a Comeback Delay and Query Response Length both set to 0.

- If the query request corresponds to an Advertisement Protocol whose server is currently unreachable, the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to SERVER_UNREACHABLE(#3524), an Advertisement Protocol element containing an Advertisement Protocol ID equal to the Advertisement Protocol ID contained in the GAS Initial Request frame and a Comeback Delay and Query Response Length both set to 0. The method used by the AP to determine the server is unreachable is out of scope of this standard(#6326). A STA receiving a status code indicating SERVER_UNREACHABLE(#3524) should wait at least 1 minute before transmitting any further queries using the same Advertisement Protocol ID to the responding STA.

- If the Advertisement Protocol ID in the Advertisement Protocol element equals the value contained in any dot11GASAdvertisementID,(MDR) and the Query Request was received in a Group Addressed GAS Request frame, the Advertisement Server processes the request. If there is no response to the Query Request, the Advertisement Server may drop the request.

- If the Advertisement Protocol ID in the Advertisement Protocol element equals the value contained in any dot11GASAdvertisementID,(MDR) then the STA shall initialize a timer, referred to as the PostReplyTimer, to the value in dot11GASResponseTimeout and post the query to the Advertisement Server identified by the Advertisement Protocol ID. If the GAS Extensions element is present in the Query Request, the STA includes the GAS Extensions information, the MAC Address and the Dialog token when it posts the query to the Advertisement Server. The methods and protocols the STA uses to post the query are outside the scope of this standard(#3329).

- (needs some work) If the Advertisment Server receives multiple GAS Query Requests in a time less than the Maximum Channel time received in a GAS Query Request,  the Advertisement Server may aggregate responses into a single GAS Query Response. The Advertisement server responds to the STA including the aggregated responses.

- e)	If dot11GASPauseForServerResponse is false, the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS(#3524), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to the value in dot11GASComebackDelay(#5046) for this Advertisement Protocol and a Query Response Length set to 0.

- If dot11GASPauseForServerResponse is true, the GAS Query Response is delivered as defined in STA procedures for transmitting the GAS Query Response.

- STA procedures for transmitting the GAS Query Response

After receiving a query response from the Advertisement Server, the responding STA shall buffer the query response for a minimum of dot11GASResponseBufferingTime after the expiration(#5402) of the GAS Comeback Delay or until the query response is delivered. If the responding STA does not receive a GAS Comeback Request frame whose source MAC address and dialog token(#6545) match the source MAC address and value of the Dialog Token field respectively of the corresponding GAS Initial Response frame(#6545) within this time, it may drop the query response. If the query response received from the Advertisement Server is larger than dot11GASQueryResponseLengthLimit for the matching dot11GASAdvertisementID or is larger than the value of the Query Response Length Limit field(#7022) received from the requesting STA,(#7389) the responding STA shall discard the response and instead return a status code of GAS_QUERY_RESPONSE_TOO_ LARGE(#3524) in the GAS Comeback Response frame. This behavior helps to prevent abuses of the medium that may be caused by overly general queries, which evoke a very large query response.

The GAS protocol supports Query Responses whose length is greater than the IEEE Std(#130) 802.11 maximum MMPDU size by the STA’s use of the GAS Query Response Fragment ID field in the GAS Comeback Response frame; the Query Response Fragment ID shall be set to 0 for the initial fragment and incremented by 1 for each subsequent fragment in a multi-fragment query response. If the Query Response is a multi-fragment response (i.e., contains more than 1 fragment), the STA shall transmit all fragments that belong to the same Query Response until all fragments are exhausted. The STA shall set the More GAS Fragments field of the GAS Query Response Fragment ID to 0 when the transmitted fragment is the final fragment.

If the GAS Initial Request frame that initiated the GAS transaction contains a Multi-band element, but the GAS Initial Response frame transmitted as a response does not contain a copy of the same Multi-band element, the Status Code in the GAS Initial Response frame shall be set to REQUEST_DECLINED(#3524). Otherwise, the requesting and responding STAs shall include a copy of the same Multi-band element in all subsequent GAS Initial Response, GAS Comeback Request, and GAS Comeback Response frames transmitted as part of the GAS transaction. Inclusion of the Multi-band element indicates to which frequency band the GAS transaction applies. If the GAS Initial Request frame that initiated the GAS transaction does not contain a Multi-band element, then none of the subsequent GAS Initial Response, GAS Comeback Request, and GAS Comeback Response frames transmitted as part of the GAS transaction shall include a Multi-band element.

The following procedures shall be used by the responding STA to deliver the query response to the requesting STA:

- If dot11GASPauseForServerResponse is true:

- If the PostReplyTimer expires before the GAS Query Response is received from the Advertisement Server, then the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to GAS_QUERY_TIMEOUT (see Table 9-46 (Status codes)), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0 and a Query Response Length set to 0. If the query response is subsequently received from the Advertisement Server, it shall be dropped by the responding STA.

- If the Query Response received from the Advertisement Server is larger than dot11GASQueryResponseLengthLimit or requires more than 128 fragments for transmission to the requesting STA, it shall be dropped by the responding STA. Then the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to GAS_QUERY_RESPONSE_TOO_ LARGE(#3524) (see Table 9-46 (Status codes)), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0 and a Query Response Length set to 0.

- If the query response’s length is less than or equal to(#3447) the maximum MMPDU size, the STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS,(#3524) an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0, the Query Response and a Query Response Length set to the query response length. This completes the GAS Query and GAS Query Response exchange.

- If the query response’s length is larger than the maximum MMPDU size, the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS(#3524), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 1 TU, and a Query Response Length set to 0; this indicates the query response will be transmitted using GAS Comeback Request and Response frames that support GAS fragmentation as follows.

- If dot11GASPauseForServerResponse is false:

- If the PostReplyTimer expires before the GAS Query Response is received from the Advertisement Server then the responding STA shall buffer for transmission a GAS Comeback Response frame with a status code equal to GAS_QUERY_TIMEOUT (see Table 9-46 (Status codes)). If the query response is subsequently received from the Advertisement Server, it shall be dropped by the STA.

- If the Query Response received from the Advertisement Server is larger than dot11GASQueryResponseLengthLimit, it shall be dropped by the responding STA. Then the STA shall buffer for transmission a GAS Comeback Response frame with status code set to GAS_QUERY_RESPONSE_TOO_ LARGE.(#3524)

- If the Query Response is received before the expiration(#5402) of the PostReplyTimer and its length is less than dot11GASQueryResponseLengthLimit, then the Query Response shall be buffered in one or more GAS Comeback Response frames with status code set to SUCCESS(#3524). The responding STA transmits one GAS Comeback Response frame in response to each GAS Comeback Request frame. If the Query Response received from the Advertisement Server is less than or equal to the maximum MMPDU size(#7630), then the GAS Query Response Fragment ID shall be set to 0 and the More GAS Fragments field in the GAS Query Response Fragment ID shall be set to 0. If the Query Response received from the Advertisement Server is greater than the maximum MMPDU size(#7630), then the GAS Query Response Fragment ID shall be set to 0 if this is the first fragment of the Query Response transmitted; otherwise it shall be incremented by 1; the More GAS Fragments field in the GAS Query Response Fragment ID shall be set to 1 if there are more fragments of the Query Response to be transmitted; otherwise it shall be set to 0 (i.e., this fragment is the last fragment of the Query Response).If a responding STA receives a GAS Comeback Request that includes the GAS Extensions element with the Fragment ID field set to a valid Fragment ID the responding STA shall respond with a GAS Comeback Response frame that includes the fragment corresponding to the received Fragment ID.,.

- If a responding STA receives a GAS Comeback Request frame whose source MAC address and dialog token(#6545) match the destination MAC address and value of the Dialog Token field(#6545) respectively of an outstanding GAS Initial Response frame and the query response has not been received from the Advertisement Server and the PostReplyTimer has not expired, the responding STA shall transmit a GAS Comeback Response frame with status code equal to GAS_RESPONSE_NOT_RECEIVED_FROM _SERVER(#3524) (see Table 9-46 (Status codes)) and GAS Comeback Delay set to the value in dot11GASComebackDelay(#5046) for this Advertisement Protocol to indicate when the requesting STA should come back(#5697) to obtain its Query Response.

- If a responding STA receives a GAS Comeback Request frame whose source MAC address and Dialog Token do not match the destination MAC address and value of the Dialog Token field respectively of an outstanding GAS Initial Response frame(#6545), the STA should transmit a GAS Comeback Response frame with a status code equal to NO_OUTSTANDING_GAS_REQUEST.(#3524)

A requesting STA shall transmit a GAS Comeback Request frame including the Dialog Token (drawn from the corresponding GAS Initial Response frame) immediately after the expiration(#5402) of the GAS Comeback Delay. In response, the responding STA provides the Query Response in one or more GAS Comeback Response frames with the corresponding Dialog Token.

If the transmitting and requesting STA support GAS Extensions, and the requesting STA is unable to receive a GAS Query Response fragment, the requesting STA may transmit a GAS Comeback Request frame, including the GAS Extensions element with the Fragment ID sub field of the GAS Flags field set to one, and the Fragment ID field set to the fragment ID of the Query Response to be retransmitted. Upon successfully receiving the GAS Comeback Request, the responding STA shall transmit a GAS Comeback Response frame that includes the Query Response fragment corresponding to the value receive in the Fragment ID field. If the Query Response fragment is not available, the responding SA shall respond with a GAS Comeback Response frame with a status code set to GAS_FRAGMENT_NOT_AVAILABLE.

If a requesting STA receives a GAS Comeback Response frame with status equal to QUERY_RESPONSE_OUTSTANDING(#3524), the requesting STA shall wait for the GAS Comeback Delay from that frame and upon expiration(#5402) of the GAS Comeback Delay, transmit another GAS Comeback Request frame. If the requesting STA’s dot11GASResponseTimer (set in STA procedures to transmit a GAS Query step b) expires prior to receiving a GAS Comeback Response frame whose source MAC address and dialog token(#6545) match those in the corresponding GAS Initial Response frame, the STA shall issue an MLME-GAS.confirm primitive with ResultCode equal to GAS_QUERY_TIMEOUT(#6062) and shall set the Query Response Length to 0.

If a requesting STA receives a GAS Comeback Response frame with status equal to SUCCESS(#3524) and the More GAS Fragments field in the GAS Query Response Fragment ID equal to 1, it shall transmit another GAS Comeback Request frame in order to retrieve the next GAS fragment of a multi-fragment query response.

If a requesting STA receives a GAS Comeback Response frame with status equal to SUCCESS(#3524) and the More GAS Fragments field in the GAS Query Response Fragment ID equal to 0, the requesting STA’s MLME shall determine that all fragments have been received by confirming that all fragment IDs from 0 to the value in the GAS Query Response Fragment ID when the More GAS Fragments field was equal to 0 have been received. Upon receipt of the first GAS Comeback Response frame and every GAS Comeback Response frame thereafter, the dot11GASResponseTimer shall be reset. If all of the query response fragments were received before the expiration(#5402) of the dot11GASResponseTimer, then the MLME shall issue an MLME-GAS.confirm (MDR)primitive with result code set to SUCCESS(#3524) along with the query response. If not all of the query response fragments were received(#5100) before the expiration(#5402) of the dot11GASResponseTimer, then the MLME shall issue an MLME-GAS.confirm (MDR)primitive with ResultCode equal to GAS_QUERY_TIMEOUT(#6063) and shall set the Query Response Length to 0.

After a requesting STA receives the first GAS fragment of a multi-fragment query response, it shall continue retrieving the query response until all GAS fragments are received or until a transmission failure is detected; the requesting STA shall not commence the retrieval of a another GAS Query Response from the same STA until all GAS fragments are received or until a transmission failure is detected on the first GAS Query Response.

If a requesting STA receives a GAS Comeback Response with status equal to (#6062)GAS_QUERY_TIMEOUT or GAS_QUERY_RESPONSE_TOO_ LARGE(#3524) then the MLME shall issue an MLME-GAS.confirm (MDR)primitive with result code so indicating and shall set the Query Response Length to 0.

If a requesting STA receives a GAS Comeback Response with status equal to NO_OUTSTANDING_GAS_REQUEST(#3524), then the MLME shall issue an MLME-GAS.confirm (MDR)primitive with result code set to NO_OUTSTANDING_GAS_REQUEST(#6064) and shall set the Query Response Length to 0.]

- 11.25.3.1.4a Group addressed GAS procedures

- 11.25.3.1.4a.1 STA procedures for transmitting the Group Addressed GAS Query Request frame

Upon receipt of an MLME-GAS.request primitive with the GAmode parameter set to 1 (true), the requesting STA shall follow procedures defined in 11.25.3.1.2 to transmit a Group Addressed GAS Request frame instead of a  GAS Initial Request frame.

- 11.25.3.1.4a.2 STA procedures for transmitting the GAS Group Addressed Query Response frame

Upon receipt of an aggregated Query Response from the Adverisement Server, th (if the Advertisment Server identifies equivalence in the query element then it informs the non-AP STA to possibly aggregate)e responding STA may transmit the aggregated  Query responses using Group Addressed GAS Response frame to the receiving STAs. The length of the Group Addressed GAS Response frame shall not exceed the IEEE 802.11 maxiumum MMPDU size. The STA shall include a list of MAC Address/Dialog Token pairs in the Response Map duples sub-field of the GAS Extensions element included in the Group Addressed GAS Response frame.

- GAS procedures interaction with Multiple BSSID Set

Non-AP STAs in the unassociated state may use GAS procedures to query Advertisement Servers for information. As described in Error: Reference source not found, APs indicate their support for a particular GAS Advertisement Protocol by including an Advertisement protocol element with that Advertisement protocol ID in Beacon and Probe Response frames as described in 9.3.3.3 (Beacon frame format) and 9.3.3.11 (Probe Response frame format) respectively. Non-AP STAs receiving Beacon or Probe Response frames from different APs may choose to engage in GAS frame exchange sequences with one or more of these APs. In some deployment scenarios, these APs may be operating as a Multiple BSSID set (as defined in Error: Reference source not found) and may relay the GAS queries to the same Advertisement Server. Depending on the configuration of the IEEE Std(#130) 802.11 access network, the external network and the Advertisement Server, a query response from the Advertisement Server might or might(MDR) not be dependent on the BSSID used in the GAS frame exchange sequence and thus the STA from which the query was relayed. If the GAS Query Response is dependent on the BSSID, a requesting STA may choose to post queries using GAS procedures to more than one STA and expect possibly different Query Responses. If the Query Response is not dependent on the BSSID, then a requesting STA may choose to post queries using GAS procedures to only one STA in the Multiple BSSID set (i.e., posting the same query to another member of the Multiple BSSID set would yield the same response).

When a Multiple BSSID (as defined in Error: Reference source not found) set contains two or more members and dot11InterworkingServiceActivated is true and dot11GASAdvertisementID is present and a query to the Advertisement Server corresponding to (#7128)dot11GASAdvertisementID is not dependent on the BSSID value used in the GAS frame exchange sequence to post the query, then the PAME-BI bit in the Advertisement Protocol tuple of the Advertisement Protocol element corresponding to (#7128)dot11GASAdvertisementID shall be set to 1; otherwise this bit shall be set to 0.

- ANQP procedures

11.25.3.2.1 General

A STA may use ANQP to retrieve information as defined in Table 9-271 (ANQP-element definitions) from a peer STA.

A non-AP STA(#1289) shall not transmit an ANQP request(#7162) to an AP for any ANQP-element unless the ANQP Advertisement Protocol ID is included in the Advertisement Protocol element in a Beacon or Probe Response frame from that AP.(#337)

The ANQP request(#7162) consists of(Ed) one or more ANQP elements drawn from Table 9-271 (ANQP-element definitions) having an element type of Q in Error: Reference source not found and shall be ordered by (MDR)nondecreasing Info ID. A Query List ANQP-element(#7162) included in an ANQP request(#7162) is formed as described in STA procedures to transmit a GAS Query(#5165). The ANQP request(#7162) is transported in the Query Request field of GAS Request frames as described in STA procedures for transmitting the GAS Query Response.(#305)

The ANQP response(#7162) should consist of(Ed) ANQP elements having Info IDs present in the Query List ANQP-element response(#7162) (if present) plus zero or more responses to other query elements; these ANQP elements shall be chosen from among those listed in Table 9-271 (ANQP-element definitions) having an element type of S in Error: Reference source not found and shall be ordered by non-decreasing Info ID. The ANQP response(#7162) is transported in the Query Response field of GAS Response frames, as described in STA procedures for transmitting the GAS Query Response.

Insert new characters and a new row just before the Symbols row. Also insert a new line within the Symbols row at the end of Table 11-15 :

Table 11-15—ANQP usage

|  |  |  | BSS |  | IBSS |
| --- | --- | --- | --- | --- | --- |
| ANQP-element name | ANQP-element (subclause) | ANQP- element type | AP | Non-AP STA | STA |
| Service Hash Request | 9.4.5.27 | Q | R,G | T,G | — |
| Service Information Request | 9.4.5.28 | Q | R,G | T,G | — |
| Service Information Response | 9.4.5.29 | S | T,G | R,G | — |
| Service Hash Response | 9.4.5.30 | S | T,G | R,G | — |
| Symbols Q element is an ANQP request(#7162) S element is an ANQP response T ANQP-element may be transmitted by MAC entity R ANQP-element may be received by MAC entity G Group addressed ANQP request/response may be transmitted and received by a MAC entitiy — ANQP-element is neither transmitted nor received by MAC entity |  |  |  |  |  |