IEEE P802.11
Wireless LANs

| Proposed 802.11ba Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei Technologies | F1-17, Huawei Base, Bantian, Longgang District, Shenzhen, China |  | ming.gan@huawei.com |
| David Xun Yang |  |  | +86-15914117462 | David.yangxun@huawei.com |
| Yunsong Yang |  | 10180 Telesis Court, STE 165, San Diego, CA 92130, U.S.A. |  | yangyunsong@huawei.com |
| Edward Au |  | 303 Terry Fox Drive Kanata, ONT, Canada |  | edward.ks.au@huawei.com |
| Osama Aboul-Magd |  |  | +1-613-287-1405 | osama.aboulmagd@huawei.com |
| Peter Loc |  |  |  | peterloc@iwirelesstech.com |
| Xiaofei wang | InterDigital |  |  | Xiaofei.Wang@InterDigital.com |
| Woojin Ahn | WILUS |  |  | woojin.ahn@wilusgroup.com |

### Abstract

This submission proposes the 802.11ba Functional Requirements as derived from the 802.11ba PAR [Ref-1] and CSD [Ref-2].

- Introduction

  - Purpose

This document proposes requirements for solutions addressing functionality to be provided by the 802.11ba amendment, referred to as the TGba Functional Requirements (FRs).

  - Scope

The scope for deriving functional requirements is set by the P802.11ba PAR [Ref-1] and CSD [Ref-2].

The functional requirements as stated in this document cover the following aspects of 802.11ba:

- System performance

- Bands of Operation

- Coexistence

- Compliance to PAR

  - Notation

Requirements are identified by a preceding unique number in the format of “TGba Rn” , where n is an integer number representing the ID of the requirements.

- Functional Requirements

  - System Performance

    - Active receiver power consumption

- The mechanisms defined in the 802.11ba amendment shall enable the wake up radio to consume less than one milliwatt when it is active.

    - Latency of transmission

- The mechanisms defined in the 802.11ba amendment should meet the latency requirement of the traffic of the 802.11ba device’s primary connective radio.

    - Network Performance

- The network performance shall not be degraded by the operation of additional 802.11ba devices compared to that of the additional legacy devices.

    - Range of transmission

- The mechanisms defined in the 802.11ba amendment shall enable wake-up radio to work within the same range as the primary connectivity radio.

  - Bands of Operation

- The 802.11ba amendment shall define operations for 2.4 GHz and 5 GHz bands.

- The 802.11ba amendment may define operations for the license-exempt sub-1GHz frequency bands.

  - Coexistence

- The 802.11ba amendment shall enable coexistence with legacy IEEE 802.11 devices operating in the same band of the wake-up radio.

  - Compliance to PAR

- The 802.11ba amendment shall comply with the PAR [Ref-1] and the CSD [Ref-2].

- References

- 11-16-1045-09-0wur-a-par-proposal-wur-sg

- 11-16-0936-04-0wur-a-csd-proposal-for-wake-up-radio-wur