IEEE P802.11
Wireless LANs

| 802.11 LB 227 comment resolutions (relative to P802.11ak D3.0 and IEEE 802.11-2016) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 | 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

This submission resolves GLK-GCR related comments from LB 227. This submission resolves the following comments: 1403, 1452 and 1455.

History:

- r0 – initial submission

- r1 – restored “set to” in the statements describing setting of state variables. Added resolution to CID 1403.

- r2 - more refinements to the resolution to CID 1455.

- R3 – fix some typos in the resolution to 1455

| CID | Commenter | Page | Line | Clause | Comment | Proposed Change | Resolution |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1403 | Yongho Seok | 41.02 | 2 | 6.3.27 | MLME-DLS.confirm, MLME-DLS.indication and MLME-DLS.response primitivies contain the GLK Capabilities element. But, the GLK Capabilities element is not included in DLS Request frame (9.6.4.2) and DLS Response frame (9.6.4.3). | Insert the GLK Capabilities element to Table 9-299 and Table 9-200. | Revise. Insert new rows to Tables 9-299 and 9-300 to include GLK Capabilities element. Add a reference to the GLK Capabilities element definition in the description of the fields/elements that are included in the DLS Request and Response frames. |  |  |  |  |  |  |  |  |
| 1455 | Joseph Levy | 72.17 | 17 | 11.24.16.3.1 | Align the style of the text so that it matches the base line text it is being inserted into. Change from "set to true" to "true" and "shall be set" to "shall be true" | Chang text to: "A GLK STA with dot11GLKGCRImplemented true shall implement the GLK-GCR procedures defined in 11.24.16.4.2 (GLK-GCR unsolicited 20 retry) and 11.24.16.4.3 (GLK-GCR block ack). When dot11GLKGCRImplemented is true the GLK-GCR Supported bit shall be 1 in the GLK Capabilities Flags field in the GLK Capabilities element, to indicate support for GLK-GCR, otherwise the bit is 0." | Counter. Accept all changes but the ones to remove “set to” |  |  |  |  |  |  |  |  |
| 1452 | Joseph Levy | 65.41 | 41 | 10.24.10.2 | The placement of the NOTE does not seem to make sense. Move the NOTE to an appropriate location | Have the NOTE follow the bulleted list of the reference text in IEEE 802.11-2016, not following the : as it is shown in the amendment. | Counter: Delete the note referred in the comment. GLK-GCR Scoreboarding is described in 10.24.10.2.a |  |  |  |  |  |  |  |  |

#### .9.6.4.2 DLS Request frame format

Insert a new row to the end of Table 9-299 and a description of the field at the end of the list of field descriptions as shown below:

Table 9-299—DLS Request frame Action field format

| Order | Information | Notes |
| --- | --- | --- |
| 12 | GLK Capabilities | The GLK Capabilities element is present if dot11GLKImplemented is true; otherwise not present. |

The GLK Capabilities field contains a GLK Capabilities element as defined in 9.4.2.220 (GLK Capabilities element) and describes the GLK capabilities of the originator of the DLS request.

9.6.4.3 DLS Response frame format

Insert a new row to the end of Table 9-300 and a description of the field at the end of the list of field descriptions as shown below:

Table 9-300—DLS Response frame Action field format

| Order | Information | Notes |
| --- | --- | --- |
| 12 | GLK Capabilities | The GLK Capabilities element is present if dot11GLKImplemented is true; otherwise not present. |

The GLK Capabilities field contains a GLK Capabilities element as defined in 9.4.2.220 (GLK Capabilities element) and describes the GLK capabilities of the target destination. This information is included only if the DLS result code corresponds to SUCCESS (DLS status code 0).

## 11.24 Wireless network management procedures

### 11.24.16 Group addressed transmission service

#### 11.24.16.3 GCR procedures

#### 11.24.16.3.1 Overview

Replace the following in CL. 11.24.16.3.1

Whan a GLK STA implements the GLK-GCR procedures defined in 11.24.16.4.2 (GLK-GCR unsolicited retry) or 11.24.16.4.3 (GLK-GCR block ack), it shall set dot11GLKGCRImplemented to true; otherwise dot11GLKGCRImplemented shall be set to false.  .16.4.2 (GLK-GCR unsolicited retry) and 11.24.16.4.3 (GLK-GCR block ack).  When dot11GLKGCRImplemeorted bit shall be set to 1 iCapabilities element, to indicate support for GLK-GCR, otherwise the bit shall beis set to 0.

Annex B, Protocol Implementation Conformance

Statement (PICS)

(normative)

B.4 PICS proforma—IEEE Std 802.11- <year>

B.4.3 IUT Configuration

Insert a new row after the CFGLK entry as shown below:

| Item | IUT Configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFGLK | GLK Enbaled STA | 11.52 GLK Operation | (NOT CFOCB):O | Yes  No  N/A  |
| CFGLK.1 | GLK-GCR | 11.24.16.4 GLK-GCR | CFGLK:O | Yes  No  N/A  |