IEEE P802.11
Wireless LANs

IEEE 802.11 TGba Simulation Scenarios and Evaluation Methodology Document

Date:	January 2017

Author:	Shahrnaz Azizi
	Intel Corporation
	2200 Mission College Blvd, Santa Clara, CA 95054
	Phone: 408 765 1834
	e-Mail: shahrnaz.azizi@intel.com

Abstract

This document provides the simulation scenarios that should be considered for TGba. It defines evaluation metrics and methodology that proposals to TGba should follow to demonstrate their suitability for WUR development in their corresponding simulation scenarios.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document. Please let Shahrnaz know if any name is missing or is preferred to be removed from this list.)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Shahrnaz Azizi | Intel | 2200 Mission College Blvd, Santa Clara, CA 95054 | 408 765 1834 | shahrnaz.azizi@intel.com |
| Steve Shellhammer | Qualcomm |  |  |  |
| Leif Wilhelmsson | Ericsson |  |  |  |
| Minyoung Park | Intel |  |  |  |
|  |  |  |  | 'tomo.adachi@toshiba.co.jp' |
| Xiaofei Wang |  |  |  | 'Xiaofei.Wang@InterDigital.com' |
| Ross Yujian |  |  |  | 'ross.yujian@huawei.com' |
|  |  |  |  | 'Shimi Shilo (TRC)' <Shimi.Shilo@huawei.com> |
| Po-kai Huang |  |  |  | <po-kai.huang@intel.com> |
|  |  |  |  | 'Joseph.Levy@InterDigital.com' |
|  |  |  |  | Oghenekome.Oteri@InterDigital.com |
| Carol Ansley |  |  |  | Carol.Ansley@arris.com |
| Minho Cheong |  |  |  | minho.cheong@newracom.com |
| John Son |  |  |  | John Son <jhson@wilusgroup.com> |
| Woojin Ahn |  |  |  | Woojin Ahn <woojin.ahn@wilusgroup.com> |
| Alexander Maltsev |  |  |  | <alexander.maltsev@intel.com> |
|  |  |  |  | lei.huang@sg.panasonic.com |
| Jinsoo Choi |  |  |  | <js.choi@lge.com> |
|  |  |  |  | <dxd@meizu.com> |
|  |  |  |  | <shwang@etri.re.kr> |
|  |  |  |  | <Junghoon.Suh@huawei.com> |
| Esung Park |  |  |  | esung.park@lge.com |
|  |  |  |  | dongguk.lim@lge.com |
| Peter Loc |  |  |  | peterloc@iwirelesstech.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| r0 | Empty skeleton submitted to satisfy the automatic document system | 23 January 2017 |
| r1 r2 r3 r4 | Initial contribution Simplified simulation scenarios. Simplified Evaluation by removing MAC related part. Included inputs for evaluation and simulation of TGba features from the March 2017 meeting Corrected revision numbers in the header. Added names to the list of contributors Added Ch B model | 24 January 2017 16 March 2017 16 March 2017 16 March 2017 |

# Introduction

This document defines simulation scenarios and evaluation criteria and methodology to be used for

-	Evaluation of performance of features proposed in TGba

-	Generation of simulation results.

For the physical layer evaluation, the simple scenario of one AP and one WUR STA is considered.

.

## Purpose of document

A proposal submitted for consideration in the 802.11 TGba is required to provide physical layer simulation results based on the receiver architecture given in the Figure 1.Receiver Architecture, and is required to disclose results for the configuration and parameters defined in this document.

## Relationship to Functional Requirements

The main purpose of this document is to define metrics to enable evaluation and adoption of the proposed methods in TGba.

In addition, the functional requirements [2] may define additional requirement for the TGba stations.

## Relationship to Usage Models

The IEEE 802.11 TGba has defined usage models [3] from which extended simulation scenarios can be defined. However this document consider a simple link level simulation scenario.

# Definitions

| Term | Definition |
| --- | --- |
| SNR | The signal to noise ratio is defined as 20MHz SNR for easy reference and comparison to typical 20MHz WLAN operation. Notes: - SNR over 20 MHz, means SNR = E[s^2] / E[n^2], where the expectation is sampling is done at 20 MHz samples. For link budget calculations, using 20 MHz bandwidth gives kTB = -114dBm/MHz + 13 dBMHz = -101dBm - If a multi-user case is defined, then the SNR is defined for the signal received at one receiver while other signlas are considered intereferers. |

# Simulation Scenarios

The simulation scenario is defined as a link level simulation between one AP and one WUR station with the following simulation cases:

- Transmission and Reception of one Wake-up packet in a 20MHz channel

- Transmission and Reception of Wake-up packet in a 20MHz channel placed between two 20MHz adjacent .11 legacy stations operating with +16 dB higher signal level as defined for ACI requirement for MCS0

- If multi-user operation within the 11ax OFDMA structure is proposed, then adjacent RU interference on both sides of the WUR allocation shall be included. In addition alternate adjacent RUs need to be simulated.

# Evaluation

The reeiver architecture shown in Figure 1 should be used for simulation.

The implementation choice of Low Pass Filter is left open to the evaluator, however when reporting simulation results, it is required to include filter characteristics and parameters, e.g., order, bandwidth, type, etc.

Figure 1.Receiver Architecture

| Number | Name | Definition | Simulation Scenario / Impairments | Status | Notes | Mandatory / optional | Disclosure |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ## PHY Related |  |  |  |  |  |  |  |
| In this section, the performance of the physical layer will be shown given realistic implementation and radio impairments under a subset of channel conditions. The results will be given in terms of PER for a given data rate and thus should be independent of MAC aspects of the proposals. |  |  |  |  |  |  |  |
| ### PHY Rates and Preambles |  |  |  |  |  |  |  |
|  | Data rates |  |  |  |  |  |  |
|  | Preambles |  |  |  |  |  |  |
| ### Channelization |  |  |  |  |  |  |  |
|  | Channelization |  |  |  |  |  |  |
|  | Spectral Mask |  |  |  |  |  |  |
| ### PHY Performance |  |  |  |  |  |  |  |
|  | AWGN PER performance |  |  |  | Three packet sizes should be considered: 32-bit/64-bit/128-bit |  |  |
|  | PER performance in non AWGN channels | Use - the IEEE Channel Model B/D - UMi-NLOS channel model for outdoor use cases [7] |  |  | Three packet sizes should be considered: 32-bit/64-bit/128-bit |  |  |
|  | Offset Compensation |  |  |  |  |  |  |
|  | ACI impact to WUR | The simulation scenario includes placement of adjacent 20MHz legacy OFDM signals on both left and right side of the WUR |  |  | Sampling rate of 160MHz should be used | AWGN is Mandatory |  |
|  | False Alarm | The simulation scenario includes in-channel 20MHz legacy OFDM signal to demonstrate robustness of WUR packet detection |  |  |  |  |  |

# Physical layer impairments

| Number | Name | Definition | Notes |
| --- | --- | --- | --- |
|  | PA non-linearity | The IEEE 802.11 PA non-linearity as defined in [5] is used at the transmitter of wake-up signal |  |
|  | Carrier frequency offset | Tx Oscilator inaccuracy of 20ppm Rx Oscilator inaccuracy of 180ppm | The total of +/- (20+180) ppm in 5GHz band is will provide a total inaccuracy of approximately 1MHz, that is considerably large and reasonable for the low power consuming WUR station. |
|  | Phase noise |  |  |
|  | Noise figure | + 8 dB worse than 11ax NF defnined in [5] | . |

# References

- 802.11-16/1045r9 WUR PAR

- TGba Functional Requirment

- TGba Usage Models

- 11/14-0980r14 TGax, “Simulation scenarios”

- 11/14-0571r12 TGax, “Evaluation methodology”

- 11/17-0326r0 TGba, “WUR phase noise model follow up”

- IMT-advanced Channel Model