IEEE P802.11
Wireless LANs

IEEE 802.11 TGba Simulation Scenarios and Evaluation Methodology Document

Date:	January 2017

Author:	Shahrnaz Azizi
	Intel Corporation
	2200 Mission College Blvd, Santa Clara, CA 95054
	Phone: 408 765 1834
	e-Mail: shahrnaz.azizi@intel.com

Abstract

This document provides the simulation scenarios that should be considered for TGba. It defines evaluation metrics and methodology that proposals to TGba should follow to demonstrate their suitability for WUR development in their corresponding simulation scenarios.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document. Please let Shahrnaz know if any name is missing or is preferred to be removed from this list.)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Shahrnaz Azizi | Intel | 2200 Mission College Blvd, Santa Clara, CA 95054 | 408 765 1834 | shahrnaz.azizi@intel.com |
| Steve Shellhammer | Qualcomm |  |  |  |
| Leif Wilhelmsson | Ericsson |  |  |  |
| Minyoung Park | Samsung |  |  |  |
| Jianhan Liu |  |  |  | Jianhan.Liu@mediatek.com |
| Tomo Adachi |  |  |  | 'tomo.adachi@toshiba.co.jp' |
| Xiaofei Wang |  |  |  | 'Xiaofei.Wang@InterDigital.com' |
| Ross Yujian |  |  |  | 'ross.yujian@huawei.com' |
| Shimi Shilo |  |  |  | 'Shimi Shilo (TRC)' <Shimi.Shilo@huawei.com> |
| Po-kai Huang |  |  |  | <po-kai.huang@intel.com> |
| Joseph Levy |  |  |  | 'Joseph.Levy@InterDigital.com' |
| Oghenekome Oteri |  |  |  | Oghenekome.Oteri@InterDigital.com |
| Carol Ansley |  |  |  | Carol.Ansley@arris.com |
| Minho Cheong |  |  |  | minho.cheong@newracom.com |
| John Son |  |  |  | John Son <jhson@wilusgroup.com> |
| Woojin Ahn |  |  |  | Woojin Ahn <woojin.ahn@wilusgroup.com> |
| Alexander Maltsev |  |  |  | <alexander.maltsev@intel.com> |
| Lei Huang |  |  |  | lei.huang@sg.panasonic.com |
| Jinsoo Choi |  |  |  | <js.choi@lge.com> |
| Xiandong Dong |  |  |  | <dxd@meizu.com> |
| Sunghyun Hwang |  |  |  | <shwang@etri.re.kr> |
| Junghoon Suh |  |  |  | <Junghoon.Suh@huawei.com> |
| Eunsung Park |  |  |  | esung.park@lge.com |
| Dongguk Lim |  |  |  | dongguk.lim@lge.com |
| Peter Loc |  |  |  | peterloc@iwirelesstech.com |
| Alan Zeleznikar |  |  |  | Alan.Zeleznikar@arris.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| r0 | Empty skeleton submitted to satisfy the automatic document system | 23 January 2017 |
| r1 r2 r3 r4 r5 r6 r7 | Initial contribution Simplified simulation scenarios. Simplified Evaluation by removing MAC related part. Included inputs for evaluation and simulation of TGba features from the March 2017 meeting Corrected revision numbers in the header. Added names to the list of contributors Added Ch B model Grammatical edits & inserted/corrected names in the contributors list Added phase noise and filter models Modified the definition for low pass filter model | 24 January 2017 16 March 2017 16 March 2017 16 March 2017 08 May 2017 10 July 2017 11 July 2017 |

# Introduction

This document defines simulation scenarios, evaluation criteria and methodology to be used for

-	Evaluation of performance of features proposed in TGba

-	Generation of simulation results.

For the physical layer evaluation, the simple scenario of one AP and one WUR STA is considered.

.

## Purpose of Document

A proposal submitted for consideration in the 802.11 TGba requires providing physical layer simulation results based on the receiver architecture given in Figure 1.Receiver Architecture, and requires the  disclosure ofresults for the configuration and parameters defined in this document.

## Relationship to Functional Requirements

The main purpose of this document is to define metrics to enable evaluation and adoption of the proposed methods in TGba.

In addition, the functional requirements [2] may define additional requirement for the TGba stations.

## Relationship to Usage Models

The IEEE 802.11 TGba group has defined usage models [3] from which extended simulation scenarios can be defined. However this document consider a simple link level simulation scenario.

# Definitions

| Term | Definition |
| --- | --- |
| SNR | The signal to noise ratio is defined as 20MHz SNR for easy reference and comparison to typical 20MHz WLAN operation. Notes: - SNR over 20 MHz, means SNR = E[s^2] / E[n^2], where the expectation is sampling is done at 20 MHz samples. For link budget calculations, using 20 MHz bandwidth gives kTB = -114dBm/MHz + 13 dB MHz = -101dBm - If a multi-user case is defined, then the SNR is defined for the signal received at one receiver while other signals are considered interferers. |

# Simulation Scenarios

The simulation scenario is defined as a link level simulation between one AP and one WUR station with the following simulation cases:

- Transmission and reception of one Wake-up packet in a 20MHz channel

- Transmission and reception of one Wake-up packet in a 20MHz channel placed between two 20MHz adjacent .11 legacy stations operating with +16 dB higher signal level as defined for ACI requirement for MCS0

- If multi-user operation within the 11ax OFDMA structure is proposed, then adjacent RU interference on both sides of the WUR allocation shall be included. In addition alternate adjacent RUs need to be simulated.

# Evaluation

The receiver architecture shown in Figure 1 should be used for simulation.

The implementation choice of low pass filter is left open to the evaluator, however when reporting simulation results, the inclusion of  filter characteristics and parameters (e.g., order, bandwidth, type, etc) is required as given in the table below.

Figure 1.Receiver Architecture

| Number | Name | Definition | Simulation Scenario / Impairments | Notes | Mandatory / optional |  | Disclosure |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## PHY Related |  |  |  |  |  |  |  |  |
| In this section, the performance of the physical layer will be shown given realistic implementation and radio impairments under a subset of channel conditions. The results will be given in terms of PER for a given data rate and thus should be independent of MAC aspects of the proposals. |  |  |  |  |  |  |  |  |
| ### PHY Rates and Preambles |  |  |  |  |  |  |  |  |
|  | Data rates |  |  |  |  |  |  |  |
|  | Preambles |  |  |  |  |  |  |  |
| ### Channelization |  |  |  |  |  |  |  |  |
|  | Channelization |  |  |  |  |  |  |  |
|  | Spectral Mask |  |  |  |  |  |  |  |
| ### PHY Performance |  |  |  |  |  |  |  |  |
|  | AWGN PER performance |  |  | Three packet sizes should be considered: 32-bit/64-bit/128-bit |  |  |  |  |
|  | PER performance in non AWGN channels | Use - the IEEE Channel Model B/D - UMi-NLOS channel model for outdoor use cases [7] |  | Three packet sizes should be considered: 32-bit/64-bit/128-bit |  |  |  |  |
|  | Offset Compensation |  |  |  |  |  |  |  |
|  | ACI impact to WUR | The simulation scenario includes placement of adjacent 20MHz legacy OFDM signals on both the left and the right sides of the WUR |  | Sampling rate of 160MHz should be used | AWGN is Mandatory |  |  |  |
|  | Low-pass Filter | For single user transmissions using the central 4MHz channel: use a 2nd order Butterworth low-pass filter with cutoff frequency at approximately 5 MHz. | The following matlab code can optionally be used for the filter design: clear all; clc; order_LPF=5; Fcut = 2.5e6; %3dB cut-off frequency at 2.5MHz Fs = 50e6; [num_LPF,den_LPF] = butter(order_LPF, Fcut/(0.5*Fs), 'low'); [h,f]=freqz(num_LPF,den_LPF,512,Fs); figure(12) plot(f, 10*log10(abs(h).^2),'r'); grid on; | This would meet the power budget of ~40uw for the filter, accompanied by ~40uw of a ring oscillator with the phase noise model given in [6]. To report performance results for 100-200 uw WUR. |  |  |  |  |
|  |  | For transmissions using the non-central 4MHz channel and/or multi-Tx: use a 5th order Butterworth low-pass filter with cutoff frequency at 2.5MHz. |  | Uses the phase noise model given in [6], which obtains the noise profile from a higher power consuming LO. To evaluate advanced multi-Tx features in the expense of higher power consumption. |  |  |  |  |
|  | False Alarm | The simulation scenario includes an in-channel 20MHz legacy OFDM signal to demonstrate robustness of WUR packet detection |  |  |  |  |  |  |

# Physical layer impairments

| Number | Name | Definition | Notes |
| --- | --- | --- | --- |
|  | PA non-linearity | The IEEE 802.11 PA non-linearity as defined in [5] is used at the wake-up signal transmitter |  |
|  | Carrier frequency offset | Tx oscillator inaccuracy of 20ppm Rx oscillator inaccuracy of 180ppm | The total of +/- (20+180) ppm frequency drift in the 5GHz band will provide a total inaccuracy of approximately 1MHz, which is considerably large and reasonable for the low power consuming WUR station. |
|  | Phase noise | The phase noise model as defined in 11/17-0326r0 [6] is used. | The simple phase noise model 802.11-04/224r1 used for primary connectivity radio does not take power consumption into consideration and hence is not suitable for the WUR. |
|  | Noise figure | + 8 dB worse than 11ax NF defined in [5] | . |

# References

- 802.11-16/1045r9 WUR PAR

- TGba Functional Requirements

- TGba Usage Models

- 11/14-0980r14 TGax, “Simulation scenarios”

- 11/14-0571r12 TGax, “Evaluation methodology”

- 11/17-0326r0 TGba, “WUR phase noise model follow up”

- IMT-advanced Channel Model