IEEE P802.11 Wireless LANs

| Proposed Resolution to CID 852-855, 865, and 869 on TGaj D5.0 from the initial sponsor ballot |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Shiwen HE | SEU |  |  | shiwenhe@seu.edu.cn |

Abstract

This document proposes resolutions CID 852-855, 865, and 869 on TGaj D5.0 from the initial sponsor ballot.

Editorial Comments

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 852 | 25.2.2 | 192 | 39 | E | MIMO MCS are defined, however, there is no discussion (in clause 10 how beamforming training is done in the case of several MIMO antenna arrays | Explain how beamforming training works in this case. |  |

Proposed resolution: Revised

MIMO MCS is only used for a transmission that uses an omnidirectional antenna pattern or quasiomni antenna pattern. It also means that the beamforming training is not a necessary step for MIMO MCS mechanism. Therefore, The beamforming training that works in the case of several MIMO antenna arrays is not defined in IEEE 802.11aj draft.

- Support MIMO transmission (SC mode and OFDM mode with omnidirectional antenna pattern or quasiomni antenna pattern transmit and receivetransceiver)

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 853 | 25.3 | 196 | 52 | E | Channelization, (what channels CMMG operates in is not clearly defined in the PHY clause) | Add definition\clarify |  |

Proposed resolution: Rejected

The channelization is clearly described in 25.10 and 25.11.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 854 | 25.3．5.2 | 203 | 62 | E | Sequences of length 256 are too long at the sample rate of 540MHz with maximum frequency offset of 20ppm. 40ppm (between two statios) at 45GHz translates to 1.8MHz. A full cycle at 1.8MHz as 0.55usec, just slightly longer than the 0.47usec of the sequence. This means that integration of a full sequence will use inverse phase for half the sequence, lowering the correlator output. | shorten the sequence, or justify the length |  |

Proposed resolution: Revised

The length of the sequence is adjusted to be 32.

25.3.5.2.1 STF of control mode

The Short Training field is composed of 1450 repetitions of the sequence Z(n) of length 25632. The sequences Z(n) is defined in Error: Reference source not found.

25.3.5.2.2 STF of SC and OFDM mode

The Short Training fields of SC and OFDM modes share the same definition. The common Short Training field is composed of 17 repetitions of the sequence Z(n) of length 32. The sequences Z(n) is defined in Error: Reference source not found.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 855 | 25.3．5.2 | 203 | 62 | E | The length of the control PHY preamble is equal to that of a the SC and OFDM PHY. This does not makes sense as the operate in signficantly different SNRs. | Lengthen the control PHY preamble and shorten the OFDM and SC. |  |

Proposed resolution: Revised

The control PHY preamble is adjusted to include 50 repetitions of the sequence Z(n) of length 32, however, the length of the sc and ofdm preabmle is adjusted to include 17 repetitions of the sequence Z(n) of length 32.

25.3.5.2.1 STF of control mode

The Short Training field is composed of 1450 repetitions of the sequence Z(n) of length 25632. The sequences Z(n) is defined in Error: Reference source not found.

25.3.5.2.2 STF of SC and OFDM mode

The Short Training fields of SC and OFDM modes share the same definition. The common Short Training field is composed of 17 repetitions of the sequence Z(n) of length 32. The sequences Z(n) is defined in Error: Reference source not found.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 865 | 10.32.5 | 103 | 22 | E | Include in the 'Definitions' ther terms "beamformer" and "beamformee" respectively. Terminoligies are used in several lines in this Sub-clause and elsewhere in the document. | As noted in the comment column. |  |

Proposed resolution: Revised

The beamformer and beamformee refer to the CMMG beamformer and CMMG beamformee, respectively. The definitions of CMMG beamformer and CMMG beamformee are given out in Clause 3, respectively. The details of the corresponding revision please refer to CID 860.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 869 | 25.3.5.2 | 203 | 65 | E | The phrase "...and mod is the modulus operation" is repeated several times throughout the document and is not required. The MOD operation is well known and defined within IEEE 802.11-2016. | Remove all occurances of this phrase on P203L65, P204L26, P205L12, P206L12, P215L7 |  |

Proposed resolution: Accepted

P. 203, L65:

where TC is SC mode chip time of CBW 540 MHz and mod is the modulus operation.

P. 204, L26:

where TC is SC mode chip time of CBW 540 MHz and mod is the modulus operation.

P. 205, L12:

where  is SC mode chip time of CBW 540 MHz and mod is the modulus operation.

P. 206, L12:

where TC is SC mode chip time of CBW 540 MHz and mod is the modulus operation.

P. 215, L17:

, is modulus operation