IEEE P802.11
Wireless LANs

| 802.11 Max Idle Period field description (relative to IEEE 802.11-2016) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-05-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 | 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

The BSS Max Idle Period element is described in Cl. 9.4.2.79 and its use is described in Cl. 11.24.13. There is no description of the units in which the value in the Max Idle Period field (except in Annex-C). Cl. 11.24.13 states “The Max Idle Period field of the BSS Max Idle Period element indicates the time period during which a STA
can refrain from transmitting frames to its associated AP without being disassociated.” It is not clear what the referred “time period” is; and how its value is determined from the value in the Max Idle Period field.

This submission proposes to add a statement to Cl. 9.4.2.79 descrining the units in which the value in the Max Idle Period field is specified.

History:

R0: Initial Version

Editor: Edit the third paragraph of 9.4.2.79 as shown below.

#### 9.4.2.79 BSS Max Idle Period element

#### The Max Idle Period field indicates the idle timeout limit, as described in 11.24.13 and is expressed in units of 1000 TUs.