IEEE P802.11
Wireless LANs

| Figure 9-589ck update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes text changes to update the P802.11aq D8.1 for the following SBO CIDs: 7137, 7140

      - GAS Extension element

The GAS Extension element is defined in Figure 9-589cj. When present in the GAS Initial Request frame, the GAS Extension element indicates the STA capability to receive a group-addressed GAS Initial response frame.

| Ele- ment ID | Length | Element ID Exten- sion | GAS Flags | Maximum Channel Time | Frag- ment ID | Number of Response Map Duples | Response Map Duples |
| --- | --- | --- | --- | --- | --- | --- | --- |

Octets:   1               1                1                1               1             0 or 1              0 or 1           0 or 1

#### Figure 9-589cj—GAS Extension element format

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1.

The GAS Flags field is defined in Figure 9-589ck.

B0	B1	B2	B3			B4 - B7

| Group addressed GAS | Maximum Channel Time Flag | Fragment ID Flag | Response Map Flag | Reserved |
| --- | --- | --- | --- | --- |

Bits:          1                               1                         1                         1                           4

#### Figure 9-589ck—GAS Flags field

Group-addressed GAS sub-field is set to 1 to indicate that the STA support group-addressed GAS and is set to 0 otherwise.

The Maximum Channel Time Flag sub-field is set to 1 to indicate that the Maximum Channel Time field is present in the element and is set to 0 otherwise.

The Fragment ID Flag sub-field is set to 1 to indicate that Fragment ID field is present in the element and is set to 0 otherwise.

The Response Map Flag sub-field is set to 1 to indicate that Number of Response Map Duples and the Response Map Duples fields are present in the element and is set to 0 other-wise.

The Maximum Channel Time field indicates the maximum duration the STA will or needs to remain on the channel to receive a GAS Initial Response, a GAS Comeback Response, or Group-addressed GAS Response, expressed as a multiple of 10 TUs beginning from the end of the PPDU carrying this element. The field has a valid range of 5-255.

The Fragment ID field in the GAS Comeback Request indicates which fragment the STA is requesting. It is only present when the Fragment ID Flag sub-field is set to 1 in the GAS Flags field.

The Number of Response Map Duples field is indicates the number of Response Map duple subfields. It is only present when the Response Map Flag subfield is set to 1 in the GAS Flags field.