IEEE P802.11
Wireless LANs

| 802.11 Liaison statement from 3GPP TSG CT WG1 on formatting of Emergency Call Number Unit |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-05-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@ieee.org |

Abstract

This document contains a liaison from 3GPP TSG CT WG1.

The liaison is embedded below and reproduced on the following pages.

- 3GPP TSG CT WG 1 Meeting 104 TDoc C1-172613 Zhangjiajie, P.R of China, 15-19 May 2017 Title: LS on formatting of Emergency Call Number Unit Release: 14 Source: 3GPP TSG CT WG 1 Meeting 104 To: IEEE 802.11 Cc: GSMA TSG, GSMA RILTE (NG) Contact person: John-Luc Bakker JBakker@blackberry.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: C1-172620 # 1 Overall description IEEE Std 802.11-2012 defines an Emergency Call Number ANQP-element in section 8.4.4.5. Emergency Call Number ANQP-element can have a list of the Emergency Call Number Unit fields and the Emergency Call Number field is defined as a variable-length UTF-8 formatted field containing information, used to reach emergency services, from the network (e.g., dialed digits, emergency service URN label). 3GPP CT1 would like to inform IEEE 802.11 that 3GPP CT1 has further defined a formatting for the Emergency Call Number Unit field in 3GPP TS 24.302, Annex I for 3GPP purposes (see attached CR). # 2 Actions To IEEE 802.11 ACTION: 3GPP TSG CT WG1 asks the IEEE 802.11 working group to take the above into account. The IEEE 802.11 working group may wish to add a reference to 3GPP TS 24.302 in future updates of IEEE Std 802.11. # 3 Dates of next TSG CT WG1 meetings TSG CT WG1 Meeting 105 21 - 25 August 2017 Krakow (Poland) TSG CT WG1 Meeting 106 23 - 27 October 2017 TBC, India

The attachment contains:

3GPP TSG-CT WG1 Meeting #104 C1-1702620 - Zhangjiajie, P.R of China, 15-19 May 2017 rev of C1-172399, C1-171972, …, C1-170626 CR-Form-v11.1. CHANGE REQUEST. 24.302; CR; 0608; rev; 8; Current version:; 14.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network; X. Title:; Managing local emergency number over the non-3GPP access. Source to WG:; Intel, BlackBerry UK Ltd., TeliaSonera AB, Nokia, Alcatel-Lucent Shanghai Bell. Source to TSG:; C1. Work item code:; SEW2-CT; Date:; 2017-04-07. Category:; B; Release:; Rel-14. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In 23.167, Annex J.2, SA2 has introduced (through S2-166858) the following requirement on UE to store up to ten local emergency numbers. - The list of downloaded emergency numbers shall be deleted at country change. The mobile equipment shall be able to store up to ten local emergency numbers received from the network. However, in SA1#77, SA1 agreed S1-171445, which update the requirements on Local Emergency Numbers List as follows:- - Emergency call numbers downloaded to the UE over WLAN from untrusted sources shall not be used by the UE. Subject to local 3GPP operator policy, the UE may use additional emergency call number(s) received via WLAN access for emergency calls via 3GPP access or WLAN access, while in the country where the numbers were received. These SA1 changes in S1-171445 means:- - Only consider any emergency number you obtain if the source is trusted. - There will be a configuration setting, set by the local 3GPP operator, which allows or not allow the avaialble emergency number list to be used when determining if a call is an emergency call. - It is an option for the UE to support the requirements on local emergency number list when in WLAN.. Summary of change:; A new subclause is introduced to specify the provisioning and handling of the list of local emergency numbers. Introduce the means that provide these local emergency numbers. Consequences if not approved:; The Local Emergency Numbers List will not be available and its usability and handling will not be defined.. Clauses affected:; 2, 4.X (new), 6.4.1, Annex I (new). Y; N. Other specs; x; Other core specifications; TS/TR ... CR .... affected:; x; Test specifications; TS/TR ... CR .... (show related CRs); x; O&M Specifications; TS/TR ... CR .... Other comments:.