IEEE P802.11
Wireless LANs

| Comment Collection ANQP tab resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-06-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes text changes to REVmd D0.1 for the following CIDs:

105, 343, 344, 345, 349, 352, 353, 354

Note: This submission does not address CID 351 in the ANQP tab of document 11-17-0927r1

| 105 | 1226.00 | 53 | 9.4.5.19 | Neighbor Report ANQP-element is defined in a manner that makes it practically impossible to parse in the case multiple Neighbor Report elements are included. This is due to the problematic "The Element ID and the Length fields of the Neighbor Report element, as shown in Figure 9-295 (Neighbor Report element format), are not included." sentence. Those fields are needed here to make it possible to parse the Neighbor Report element subfield with more than one element. It should be noted that known implementation efforts in this area (e.g., based on WFA programs) are already doing this and no known deployed implementation follows this sentence in IEEE 802.11. The standard should be updated to fix the issue and match what vendors will end up implementing and deploying here. | Delete the sentence: "The Element ID and the Length fields of the Neighbor Report element, as shown in Figure 9-295 (Neighbor Report element format), are not included." |
| --- | --- | --- | --- | --- | --- |
| 343 | 1226.00 | 52 | 9.4.5.19 | Each "Neighbor Report element field" within this ANQP-element may have different variable lengths and therefore the sentence "The Element ID and the Length fields of the Neighbor Report element, as shown in Figure 9-295, are not included" prevents each "Neighbor Report element field" from being correctly parsed. This sentence should be removed. | Delete the sentence "The Element ID and the Length fields of the Neighbor Report element, as shown in Figure 9-295, are not included." |

      - Neighbor Report ANQP-element

The format of the Neighbor Report element is shown in Figure 9-295 (Neighbor Report element format) defined in 9.4.2.37 (Neighbor Report element). The Co-Located BSSID List subelement is present when there is at least one other BSS which is co-located with the reporting BSS.

| 352 | 1230.00 | 46 | 9.4.5.24 | The sentence "The Query AP List ANQP-element declares that the STA performing the ANQP query is requesting the ANQP-element corresponding to that Info ID be returned in the ANQP query response" is not referring to the correct response | Change the sentence to "The Query AP List ANQP-element declares that the STA performing the ANQP query is requesting the ANQP-element corresponding to that Info ID be returned in the AP List Response. " |
| --- | --- | --- | --- | --- | --- |
| 345 | 1230.00 | 50 | 9.4.5.24 | The sentence "Each ANQP-element can be returned in response to Query AP List ANQP-element using the procedures in 11.25.3.2 (GAS Protocol)" does not read well and the reference is incorrect. | Change the sentence to "See 11.25.3.3 (ANQP procedures) for information on ANQP procedures" |

      - Query AP List ANQP-element

The Query AP List ANQP-element provides a list of APs and a list of identifiers of ANQP-elements that the requesting STA is querying. The Query AP List ANQP-element declares that the STA performing the ANQP query is requesting the ANQP-element corresponding to that Info ID be returned in the AP List response. This element allows an optimization of the ANQP query procedure by having multiple queries in a single ANQP query list thus reducing the time necessary for network discovery and selection. See 11.25.3.3 (ANQP procedures) for information on ANQP procedures.

| 344 | 1231.00 | 52 | 9.4.5.25 | There is no definition of what an "ANQP attribute" is within this paragraph. The reference is also incorrect in this paragraph and the sentences appear to repat themselves. | Change the paragraph to "Each AP N Query Response field contains an ANQP response corresponding to each ANQP Query ID in the received Query AP List ANQP-element as specified in 9.4.5.24 (Query AP List ANQP-element(11ai)). Each ANQP response comprises one or multiple ANQP-elements (Table 9-281 (ANQP-element definitions)." |
| --- | --- | --- | --- | --- | --- |

      - AP List Response ANQP-element

Each AP N Query Response field contains an ANQP response corresponding to each ANQP Query ID in the received Query AP List ANQP-element as specified in 9.4.5.24 (Query AP List ANQP-element(11ai)). Each ANQP response comprises one or multiple ANQP-elements (Table 9-281 (ANQP-element definitions)).

| 349 | 1956.00 | 57 | 11.25.3.3.1 | There is no definition of what an "ANQP attribute" is. | Change the sentence from: "The STA stores the ANQP CAG Version in the received CAG ANQP-element, the ANQP attributes, and information corresponding to that CAG Version with the values of BSSID, or HESSID and the corresponding SSID associated with the responding AP within the cache" to "The STA stores within the cache, the ANQP CAG Version and ANQP responses corresponding to each Info ID from the received CAG ANQP-element, together with the values of BSSID, or HESSID and the corresponding SSID of the responding AP. The STA obtains the required ANQP responses either as an ANQP response to a previous ANQP request, or by transmiting an ANQP request for the ANQP response." |
| --- | --- | --- | --- | --- | --- |

        - General

Page 1956,  5th Paragraph

The STA stores within the cache, the ANQP CAG Version and ANQP responses corresponding to each Info ID from the received CAG ANQP-element, together with the values of BSSID, or HESSID and the corresponding SSID of the responding AP.  The STA obtains the required ANQP responses either as an ANQP response to a previous ANQP request, or by transmiting an ANQP request for the ANQP response.

| 353 | 1959.00 | 48 | 11.25.3.3.1 | There is no definition of what an "ANQP attribute" is. | Change the sentence "If the CAG Versions do match, the STA should use the stored ANQP attributes and information of the CAG for network discovery(11ai)" to "If the CAG Versions do match, the STA should use the stored ANQP responses corresponding to that CAG Version for network discovery(11ai)" |
| --- | --- | --- | --- | --- | --- |
| 354 | 1959.00 | 51 | 11.25.3.3.1 | The sentence "The ANQP server assignment of the ANQP CAG Version associated with the to ANQP attributes and information is out of the scope of this document(11ai)." needs to be clarified, as it's not clear what an ANQP attribute is. | Change the sentence to "The access network server assignment of the ANQP CAG Version associated with the ANQP responses is out of the scope of this document(11ai)." |

        - General

Page 1959,  3rd Paragraph

A FILS STA that makes use of the CAG Version is subject to the following:

— The STA may obtain the CAG Version from the CAG element received in Beacon or Probe Response frames.

— The STA searches its cached information using either the BSSID, HESSID, or SSID of the AP as an index to obtain the correct cached ANQP CAG Version entry.

— If the cached ANQP CAG Version does not match the CAG version received in Beacon or Probe Response frames, the STA should transmit an ANQP query request for any of the ANQP-elements contained within the CAG. If the CAG Versions do match, the STA should use the stored ANQP responses corresponding to that CAG Version for network discovery(11ai).

The access network server assignment of the ANQP CAG Version associated with the ANQP responses is out of the scope of this document(11ai)..