IEEE P802.11 Wireless LANs

| Proposed Resolution to CID 904, 907, 908, 909, 913, 914, and 919 on TGaj D6.0 from the sponsor ballot |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-06-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Shiwen HE | SEU |  |  | shiwenhe@seu.edu.cn |
| Haiming Wang | SEU |  |  | hmwang@seu.edu.cn |

Abstract

This document proposes resolutions CID 904, 907, 908, 909, 913, 914, and 919 on TGaj D6.0 from the initial sponsor ballot.

Editorial Comments

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 904 | 25.3.10 | 208 | 5 | E | P. 208 L. 5, the description of the SC chip rate Tc is not correct. P. 208 L. 10, the specific value of Tc need to be defined. | Propose to rewrite the SC chip rate Fc equal to 440MHz and define the Tc as Tc is the SC chip time duration for 540MHz channel bandwidth. |  |

Proposed resolution: Revised

Change the first sentence in the first paragraph in 25.3.10 as follows:

The STF fields, the CEF fields, and the SIG fields that are transmitted using SC modulation are specified at the SC chip rate  equal to 440 MHz.

Change the second paragraph in 25.3.10 from

“Let x(nTc) = xSTF(nTc) + xCEF(nTC-tCEF) + xSIG(nTC-tSIG), where rCEF is described in 25.3.5(CMMG PHY preamble).”

to

“Let , where  is the SC chip time duration for 540 MHz channel bandwidth and  is described in Error: Reference source not found.”

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 907 | 10.32.5 | 98 | 64 | T | "A CMMG beamformer shall set the response type format indicated in the CSI/Steering subfield of the CMMG Control field of any sounding frame excluding the NDP and of any PPDU with the NDP Sounding Announcement field equal to 1 to Noncompressed Beamforming) that is supported by the CMMG beamformee." The noncompressed beamforming feedback method has been removed from 11aj. | Reword this paragraph. |  |

Proposed resolution: Revised.

Change the last paragraph in 10.32.5, P. 98, L.64, from

“A  CMMG  beamformer  shall  set  the  response  type  format  indicated  in  the  CSI/Steering  subfield  of  the CMMG Control field of any sounding frame excluding the NDP and of any PPDU with the NDP Sounding Announcement field equal to 1 to Noncompressed Beamforming) that is supported by the CMMG beamformee.”

to

“A CMMG beamformer shall set the response type format indicated in the CSI/Steering subfield of the CMMG Control field of any sounding frame excluding the NDP and of any PPDU with the NDP Sounding Announcement field equal to 1 to the value of the type Compressed Beamforming.”

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 908 | 10.32.5 | 99 | 48 | T | "The value of within an explicit beamforming feedback frame transmitted by a CMMG beamformee shall not exceed the value indicated in the Beamformee STS Capability subfield of the CMMG Capabilities element." There is no Beamformee STS Capability subfield defined in 11aj. | Reword this paragraph. |  |

Proposed resolution: Revised

Delete the paragraph in P.99, L. 47-50:

“The value of within an explicit beamforming feedback frame transmitted by a CMMG beamformee shall not exceed the value indicated in the Beamformee STS Capability subfield of the CMMG Capabilities element.”

Insert a new paragraph as the last paragraph of 10.32.5：

“A CMMG beamformee that transmits a feedback frame of type Compressed Beamforming in response to a sounding PPDU sent by a CMMG beamformer shall set the value of  in the MIMO Control field of the feedback frame to be the same value as the number of streams in the sounding PPDU.”

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 909 | 10.32.5 | 99 | 64 | T | "... a frame that is appropriate for the current frame exchange sequence following Table 10-18b (Rules for HT beamformee immediate feedback transmission responding to NDP sounding)." There is no Table 10-15b in 11aj D6.0. | Reword this paragraph. |  |

Proposed resolution: Revised.

Change the title of Table 10-15b in P. 99 L.64-65 from “Table 10-18b (Rules for HT beamformee immediate feedback transmission responding to NDP sounding).” to “Table 10-18b (Rules for CMMG beamformee immediate feedback transmission responding to NDP sounding). “

Insert Table 10-18b after the paragraph in P.99L.60-65 as follows:

Table 10-18b-Rules for CMMG beamformee immediate feedback transmission responding to NDP sounding

| Type of response Rule | Rule |
| --- | --- |
| Control response | If the transmission of a control response frame is required in response to the NDP request for feedback, the control response frame is transmitted an SIFS after reception of the PPDU that elicited the control response, and the feedback response frame may be transmitted an SIFS after the reception of the NDP. - If the feedback response frame is not transmitted an SIFS after the reception of the NDP and the CMMG beamformee is subsequently required to transmit an Ack or BlockAck frame in the same TXOP, then the feedback response may be aggregated with the Ack or BlockAck frame. - If the feedback response frame is not transmitted an SIFS after the reception of the NDP and is not transmitted with an aggregated Ack or BlockAck frame in the same TXOP, then the feedback response frame is delayed until the CMMG beamformee’s next transmission within the TXOP. |
| No control response | If the transmission of a control response frame is not required in response to the NDP request for feedback, the feedback response frame may be sent an SIFS after the reception of the NDP. - If the feedback response frame is not transmitted an SIFS after the reception of the NDP and the CMMG beamformee is subsequently required to transmit an Ack or BlockAck frame in the same TXOP, then the feedback response may be aggregated with the Ack or BlockAck frame. - If the feedback response frame is not transmitted an SIFS after the reception of the NDP and is not transmitted with an aggregated Ack or BlockAck frame in the same TXOP, then the feedback response frame is delayed until the CMMG beamformee’s next transmission within the TXOP. |

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 913 | 9.4.1.61 | 32 | 12 | T | "...CMMG CSI (see 9.6.31.2 (CMMG CSI frame format)), CMMG Noncompressed Beamforming (see 9.6.31.3 (CMMG Noncompressed Beamforming frame format))..." 9.6.31.2 (CMMG CSI frame format) and 9.6.31.3 (CMMG Noncompressed Beamforming frame format) have been removed from 11aj. | Reword this paragraph.. |  |

Proposed resolution: Revised

Rewrite the first paragraph in 9.4.1.61 as follows:

The CMMG MIMO Control field is used to manage the exchange of CMMG MIMO channel state or transmit beamforming feedback information. It is used in the CMMG CSI (see 9.6.31.2 (CMMG CSI frame format)), CMMG Noncompressed Beamforming (see 9.6.31.3 (CMMG Noncompressed Beamforming frame format)),  and CMMG Compressed Beamforming (see Error: Reference source not found) frames.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 914 | 9.4.1.61 | 33 | 13 | T | "For CMMG Noncompressed Beamforming feedback:" The Noncompressed Beamforming feedback has been removed from 11aj. | Reword the description for the Coefficient Size subfield. |  |

Proposed resolution: Revised

Remove the “Coefficient Size subfield” from Figure 9-121a as follows:

|  | Nc Index | Nr Index | Channel Width | Grouping | Coefficient Size | Codebook Information | Remaining Matrix Segment | Reserved | Sounding Timestamps |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 9-121a - CMMG MIMO Control field

Remove the row of the coefficient Size subfield in Table 9-76a as follows:

| Table 9-76a—Subfields of the CMMG MIMO Control field |  |
| --- | --- |
| Subfield | Description |
| … | … |
| Coefficient Size | Indicates the number of bits in the representation of the real and imaginary parts of each element in the matrix. For CMMG CSI feedback: Set to 0 for Nb =4 Set to 1 for Nb =5 Set to 2 for Nb =6 Set to 3 for Nb =8 For CMMG Noncompressed Beamforming feedback: Set to 0 for Nb =4 Set to 1 for Nb =5 Set to 2 for Nb =6 Set to 3 for Nb =8 |
| … | … |

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 919 | 10.36.6.6.1 | 106 | 25 | T | "In the CMMG protected period that have 540 MHz or 1080 MHz bandwidth, the STA can use dynamic bandwidth operation to negotiate a bandwidth can be use in this SP." Grammatical mistake. | Reword this paragraph. |  |

Proposed resolution: Revised

Rewrite the first paragraph of 10.36.6.1 as follows:

“In the CMMG protected period that havehas 540 MHz or 1080 MHz bandwidth, the STA can use dynamic bandwidth operation to negotiate a bandwidth that can be used in this SP.”