IEEE P802.11
Wireless LANs

| 802.11 CC25 - Proposed Resolutions for CID 8 and Others |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corporation |  |  | Emily.h.qi@intel.com |

Abstract

This document contains proposed resolutions to CC25 comments: CID 8, 9, 10, 11, 12, 16, 347, and 348,

| 7101 | 1706.09 | 11.11.10.3 | "the reporting AP has dot11LciCivicInNeighborReport and the neighbor AP has LCI MeasurementCapability (RM Enabled Capabilities element with the LCI Measurement Capability Enabled fieldset to 1) dot11RMLCIMeasurementActivated equal to true"-- this has at least two errors "has dot11LciCivicInNeighborReport" and"Measurement Capability (...) dot11RMLCIMeasurementActivated equal to true"Note that " (RM Enabled Capabilities element with the LCI Measurement Capability Enabled field set to 1)" is also a informal way of anonymously referencing a transmission by the AP)." this can also be improved. This informality occurs in a number of places in this subclause. The proposed changes addresses two of these. | Change cited text to:"the reporting AP has dot11LciCivicInNeighborReport equal to true and the neighbor AP indicates support for LCI measurement(the neighbor AP has transmitted an RM Enabled Capabilities element with the LCI Measurement Capability Enabled field equal to true)"Make matching changes at 1706.32. |
| --- | --- | --- | --- | --- |

TGmd Editor: Change 9.4.2.177, 9.4.5.24, 9.4.5.25, 9.4.5.26, and 9.4.5.27 as follows (set to “All Markup” in the REVIEW):

- CAG Number element(11ai)

The Common Advertisement Group (CAG) is a group of elements that are defined by the same advertisement protocol and that do not change on a rapid basis within an AP. The CAG Number element provides one or more current version numbers of the CAG (CAG Version) associated with the AP, where each version number is associated with a specific advertisement protocol. The CAG Number element is optionally present in the Beacon or Probe Response frame to reduce GAS frame exchanges when dot11InterworkingServiceActivated is true.

The format of the CAG Number element is shown in Error: Reference source not found.

|  | Element ID | Length | CAG Tuples |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | n x 2 |

The Element ID and Length fields are defined in 9.4.2.1 (General).

The CAG Tuples field contains one or more 2-octet CAG Tuple fields. The format of a CAG Tuple field is shown in CAG Tuple field.

|  | B0 B7 | B8 B15 |
| --- | --- | --- |
|  | CAG Version | Advertisement Protocol ID |
| Bits: | 8 | 8 |
| - CAG Tuple field(11ai) |  |  |

The CAG Version subfield is an unsigned integer indicating the current version of the CAG of the associated advertisement protocol. The use of CAG Version is explained in 11.25.3.3 (ANQP procedures).

The Advertisement Protocol ID subfield is an 8-bit subfield and carries a value equal to the Advertisement Protocol ID of the advertisement protocol associated with the CAG Version in the same CAG Tuple field. The Advertisement Protocol ID is defined in Error: Reference source not found in Error: Reference source not found.

- Query AP List ANQP-element(11ai)

The Query AP List ANQP-element provides a list of APs and a list of identifiers of ANQP-elements that the requesting STA is querying. The Query AP List ANQP-element declares that the STA performing the ANQP query is requesting the ANQP-element corresponding to that Info ID be returned in the ANQP query response. This element allows an optimization of the ANQP query procedure by having multiple queries in a single ANQP query list thus reducing the time necessary for network discovery and selection. Each ANQP-element can be returned in response to Query AP List ANQP-element using the procedures in 11.25.3.2 (GAS Protocol).

The format of the Query AP List ANQP-element is provided in Query AP List ANQP-element format.

|  | Info ID | Length | AP List | ANQP Query IDs |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | n x 2 |  |  |
| - Query AP List ANQP-element format(11ai) |  |  |  |  |  |  |

The Info ID and Length fields are defined in Error: Reference source not found.

The AP List field is a variable length field defined in AP List field format that contains the list of BSSIDs for requested information.

|  | AP List Length | BSSIDs |  |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | n x 6 |  |  |
| - AP List field format(11ai) |  |  |  |  |

The AP List Length field is a 1-octet field whose value indicates the length of the BSSIDs field..

The BSSIDs field contains one or more BSSID fields.

The BSSID field is 6 octets in length and indicates the BSSID of an AP that the requesting STA wants to query.

The ANQP Query IDs field contains one or more ANQP Query ID fields that are ordered by increasing info ID value(Ed).  Each ANQP Query ID field is an unsigned integer of length two octets.

The ANQP Query ID field value is drawn from Table 9-271 (DNS Info Control subfield settings(11ai)).

- AP List Response ANQP-element(11ai)

The AP List Response ANQP-element provides the response to the Query AP List ANQP-element request. The ANQP query response is defined in 11.25.3.3 (ANQP procedures). The format of the AP List Response ANQP-element is defined in AP List Response ANQP-element format.

|  | Info ID | Length | AP Response Tuples |  |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |  |
| - AP List Response ANQP-element format(11ai) |  |  |  |  |

The Info ID and Length fields are defined in Error: Reference source not found.

The AP Response Tuples field contains zero or more AP Response Tuple fields.

The format of the AP Response Tuple field is defined in Figure 9-659a.

|  | AP Identifier | AP Response Length | AP Query Response |
| --- | --- | --- | --- |
| Octets: | 6 | 2 | variable |

Figure 9-659a -- AP Response Tuple field format

The AP Identifier field  indicates the BSSID of the AP that the requesting STA queries.

The AP Response Length field indicates the number of octets in the following AP Query Response field.

The AP Query Response field contains zero or more ANQP-elements (Table 9-588 (ESP Information field format)) that correspond to the ANQP response to the received Query AP List ANQP-element as specified in Query AP List ANQP-element. This field is also formatted in accordance with ANQP. This field can contain one or more values of the ANQP attributes that are specific for a particular AP and were requested by a STA via the Query AP List ANQP-element.

- FILS Realm Information ANQP-element(11ai)

The FILS Realm Information ANQP-element provides a list of information about the domains. The format of the FILS Realm Information ANQP-element is defined in Figure 9-660.

|  | Info ID | Length | Realm Identifiers |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | n x 2 |

- FILS Realm Information ANQP-element format(11ai)

The Info ID field and Length fields are defined in Error: Reference source not found.

The Realm Identifiers field contains zero or more Realm Identifier fields as defined in Figure 9-602 (Realm Identifier field(11ai)).

- CAG ANQP-element(11ai)

The CAG ANQP-element provides the info IDs for the ANQP-elements contained within a CAG associated with ANQP and the current value of the ANQP CAG version, indicating the version of information within the CAG associated with ANQP. The format of the CAG ANQP-element is defined in Figure 9-661.  The selection of the specific number of info IDs and the specific values of info IDs in a CAG associated with ANQP is left to the implementation and is beyond the scope of this standard(Ed).

|  | Info ID | Length | ANQP CAG Version | Info IDs |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | n x 2 |  |
| - CAG ANQP-element format(11ai) |  |  |  |  |  |

The Info ID and Length fields are defined in Error: Reference source not found.

The ANQP CAG Version field indicates the current version of the CAG associated with ANQP.

The Info IDs field contains zero or more Info ID fields. The Info ID field is an unsigned integer of length two octets andcontains the info ID of an ANQP-element Info ID specified in Table 9-271 (DNS Info Control subfield settings(11ai)).