IEEE P802.11
Wireless LANs

| 802.11 [Resolutions to CID #148 and 339 (relative to IEEE 802.11-2016) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-07-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 | 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

This submission proposes resolutions to CIDs 148 and 339.

History:

R0: Initial Version

| 148 | Mark RISON | 9.4.2.22.18 | 937 | 47 | T | N | "The Measurement Start Time field contains the least significant 4 octets of the TSF (synchronized with the associated AP) at the time (+/- 32 us) at which the initial Fine Timing Measurement frame was transmitted where the timestamps of both the frame and response frame were successfully measured." -- no measurement is done with the iFTM, except when both the initiator and the responder set the ASAP bit to 1 | Change "initial" to "first" in the cited text | MAC: 2017-06-21 16:52:46Z: If the "initial Fine Timing Measurement frame" is special ("initial" doesn't mean "first"), perhaps we should capitalize the 'I' in 'Initial', and treat this as a unique frame? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The intent is to identify the “first” or “initial” successful Fine Timing Measurement Request frame from the initiator to the responder; and the subsequent “first” or “initial” successful Fine Timing Measuremetn frame from the responder to the initiator with an FTM session as special frames – these are special because they carry additional information.

Clause 11.24.6.1 defines FTM session and introduces the concept of negotiation, measurement and termination. FTM session is established when a FTM Request, FTM frame exchange is successfully completed. The FTM request and the FTM frame used to establish the FTM session are special and respectively called

- session establishment FTM Request and session establishment FTM frames

- FTM session negotiation Request and FTM session negotiation frames

- Others

Depending on the term that we agree on, we need to replace occurrences of initial FTM Request and initial FTM in the specification with appropriate “agreed upon” replacement.

Resolution:

Accept In Principle: Define “initial” FTM Request and “initial” FTM frames; and replace

| 339 | Qi Wang | 11.24.6.4 | 1920 | 50 | T | N | Equation (11-6) is incorrect. | Modify equation (11-6) to: clock offset = [(t2-t1)-(t4-t3)]/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Equation 11-6 is correct but the subtlety of the currently captured t2 and t3 being used with the t1 and t4 from the payload of the next FTM frame received is not clear in the description. In addition, the corresponding equation for Timing Measurement equation 11-4 may also require some changes both for consistency with the corresponding descriptions for FTM and for correctness.

Proposed Resolution: Accept in Principle – modify figures 11-33, to use t1_n, t2_n, t3_n and t4_n notation used in Figures 11-35. 11-36 and 11-37; and modify equations 11-4 and 11-6 to be

Clock_offset = [(t2_n – t1_n) – (t4_n – t3_n)]/2