IEEE P802.11
Wireless LANs

| Issues with 11aq text related to GAS protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-08-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 400, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |
| Michael Montemurro | BlackBerry | 4701 Tahoe Blvd, Mississauga, ON. Canada L4W0B4 | +1 289-261-4183 | mmontemurro@blackberry.com |

Abstract

This document discusses potential issues with 11aq D10.0 text related to 11.25.3.2 GAS Protocol and provides a text proposal to fix them.

- The issues

In P802.11aq D10.0, clause 11.25.3.2.3 (STA procedures to post a GAS Query to an Advertisement Server), there are a number of bullets, summaired as follows:

| Bullet | Situation | The STA behaviors |
| --- | --- | --- |
| a) | Advertisement Protocol ID not supported. | The responding STA returns a failure Status Code without posting the query to an advertisement server. |
| b) | Server not reachable | The responding STA returns a failure Status Code without posting the query to an advertisement server. |
| c) | Describing behaviors of the advertisement server, more specifically, relating to processing the query and dropping the query when there is no response. Since the advertisement server receives the query, this is a scenario under d). | None. |
| d) | The Advertisement Protocol ID supported (and implicitly, the server is reachable) | The responding STA posts the query to the advertisement server. |
| e) | Describing behaviors of the advertisement server, more specifically, relating to aggregating responses into one. Since the advertisement server receives the query, this is a scenario under d). | None. |
| f) | This is a scenario under d), where dot11GASPauseForServerResponse is false. | The responding STA informs the requesting STA to come back after the indicated Comeback Delay expires. |
| g) | This is a scenario under d), where dot11GASPauseForServerResponse is true. | No further specification beyond what is defined in clause 11.25.3.2.4. |
| h) | When two CAG Versions match (and implicitly, the Advertisement Protocol ID is supported) | The responding STA informs the requesting STA that the cached CAG is still valid without posting the query to the advertisement server. |

First, the added bullets c) and e) happens only when bullet d) occurs. The current ordering doesn’t reflect that. Recommending to list them as sub-bullets of bullet d).

Second, like the bullets a) and b), the added bullet h) doesn’t cause the responding STA to post the query to the advertisement server, while bullet d) does and bullets f) and g) are different scenarios after bullet d) happens. Therefore, bullet h) is mutually exclusive with bullets d), f) and g), but the current ordering of these bullets didn’t reflect that and may cause problem. For example, whenever the condition in bullet h) is met, the condition in bullet d) is met first, thus according to bullet d), the responding STA posts the query to the advertisement server, contradicting to bullet h). Recommending to list bullet h) before bullet d).

Third, “If there is no response to the Query Request, the Advertisement Server may drop the request.” in bullet c) is insuffient to stop the responding STA (the AP) from responding to the GAS query, because it didn’t specify any changes to the behavior of the responding STA. Thus, according to the following highlighted text in 802.11-2016, even if the advertisement server drops the request (implying that no response will be sent to the responding STA), the responding STA will still transmit a GAS frame with a status code equal to GAS_QUERY_TIMEOUT and a null response to the requesting STA:

“11.25.3.2.4 STA procedures for transmitting the GAS Query Response

…

[802.11-2016, Page 1830]

The following procedures shall be used by the responding STA to deliver the query response to the requesting STA:

a) If dot11GASPauseForServerResponse is true:

1) If the PostReplyTimer expires before the GAS Query Response is received from the Advertisement Server, then the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to GAS_QUERY_TIMEOUT (see Table 9-46), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0 and a Query Response Length set to 0. If the query response is subsequently received from the Advertisement Server, it shall be dropped by the responding STA.

…

[802.11-2016, Page 1831]

b) If dot11GASPauseForServerResponse is false:

1) If the PostReplyTimer expires before the GAS Query Response is received from the Advertisement Server then the responding STA shall buffer for transmission a GAS Comeback Response frame with a status code equal to GAS_QUERY_TIMEOUT (see Table 9-46). If the query response is subsequently received from the Advertisement Server, it shall be dropped by the STA.

”

- Proposed text changes

<First Change to P802.11aq D10.0>

11.25.3.2.3 STA procedures to post a GAS Query to an Advertisement Server

…

c) If the GAS Initial Request frame includes a CAG Number element and all the CAG Versions in the CAG Number element match a valid CAG Version that the STA receives from the associated Advertisement Server, which is identified by the CAG Information Type subfield within the same CAG Tuple field as the CAG Version subfield, the STA may transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to SUCCESS_CAG_VERSIONS_MATCH, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, and a Comeback Delay and Query Response Length both set to 0, without posting the query to an Advertisement Server.

d) If the Advertisement Protocol ID in the Advertisement Protocol element equals the value contained in any dot11GASAdvertisementID, then the STA shall initialize a timer, referred to as the PostReplyTimer, to the value of the Maximum Channel Time received in the GAS Initial Request or Group Addressed GAS Request frame, otherwise to the value in dot11GASResponseTimeout. The STA posts the query to the Advertisement Server identified by the Advertisement Protocol ID. If the GAMode associated with the Query Request is true, the STA includes the GAMode parameter, the GAS extension information, the MAC Address and the Dialog token when it posts the query to the Advertisement Server for processing. The methods and protocols the STA uses to post the query are outside the scope of this standard.

- If the GAMode associated with the Query Request is true and the Advertisement Server has no response to the Query Request, the Advertisement Server may drop the request.

- If the Advertisement Server receives multiple GAS Query Requests resulting in the same response, the Advertisement Server may aggregate these responses into a single GAS Query Response. The Advertisement server responds to the STA including the aggregated responses as described in 11.25.3.2.4a.

e) If dot11GASPauseForServerResponse is false, the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to the value in dot11GASComebackDelay for this Advertisement Protocol and a Query Response Length set to 0.

f) If dot11GASPauseForServerResponse is true, the GAS Query Response is delivered as defined in 11.25.3.2.4.

<Second Change to P802.11aq D10.0>

11.25.3.2.4 STA procedures for transmitting the GAS Query Response

…

The following procedures shall be used by the responding STA to deliver the query response to the requesting STA:

a) If dot11GASPauseForServerResponse is true:

1) If the PostReplyTimer expires before the GAS Query Response is received from the Advertisement Server for responding to a GAS Initial Request frame, then the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to GAS_QUERY_TIMEOUT (see Table 9-46), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0 and a Query Response Length set to 0. If the query response is subsequently received from the Advertisement Server, it shall be dropped by the responding STA.

2) If the PostReplyTimer expires before the GAS Query Response is received from the Advertisement Server for responding to a Group Addressed GAS Request frame, then the responding STA shall not transmit an individually addressed GAS Initial Response frame or a Group Addressed GAS Response frame to the requesting STA. Note: If there is no response to the Query Request, the Advertisement Server may drop the request.

3) If the Query Response received from the Advertisement Server is larger than dot11GASQueryResponseLengthLimit or requires more than 128 fragments for transmission to the requesting STA, it shall be dropped by the responding STA. Then the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to GAS_QUERY_RESPONSE_TOO_ LARGE (see Table 9-46), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0 and a Query Response Length set to 0.

4) If the query response’s length is less than or equal to the maximum MMPDU size and the query response is not an aggregated response, the STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame or the Group Addressed Request frame, a GAS Comeback Delay set to 0, the Query Response and a Query Response Length set to the query response length. This completes the GAS Query and GAS Query Response exchange.

5) If the query response’s length is less than or equal to the maximum MMPDU size and the query response is an aggregated response, the STA shall transmit a Group Addressed GAS Response frame containing a dialog token set to 0, a Status Code set to SUCCESS, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame or the Group Addressed Request frame, a GAS Comeback Delay set to 0, the Query Response and a Query Response Length set to the query response length, and a GAS Extension element containing a list of MAC Address/Dialog Token pairs in the Response Map Duples subfield of the GAS Extension element, identifying the requesting STAs and their GAS Query Requests to which the Group Addressed GAS Response frame responds. This completes the GAS Query and GAS Query Response exchange.

6) If the query response’s length is larger than the maximum MMPDU size, the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 1 TU, and a Query Response Length set to 0; this indicates the query response will be transmitted using GAS Comeback Request and Response frames that support GAS fragmentation as follows. If the responding STA is capable of retransmitting a GAS Query Response fragment upon request, the responding STA shall include, in the GAS Initial Response frame, a GAS Extension element with the Fragment Retransmission subfield of the GAS Flags field set to 1.