IEEE P802.11
Wireless LANs

| Clause 11.25.3.2.4 update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |
| Michael Montemurro | BlackBerry Ltd | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-289-261-4183 | mmontemurro@blackberry.com |

Abstract

This document proposes text changes to update clause 11.25.3.2.4 of P802.11aq D11.0

Update the following sections of the following clause (and ignore the comments):

11.25.3.2.4 STA procedures for transmitting the GAS Query Response

The following procedures shall be used by the responding STA to deliver the query response to the requesting STA:

- If dot11GASPauseForServerResponse is true:

  - If the Qquery Rresponse is received from the Advertisement Server before the PostReplyTimer expires, and if the query response’s length is less than or equal to the maximum MMPDU size and the Query Response is not an aggregated response, the STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame or the Group addressed Request frame, a GAS Comeback Delay set to 0, the Query Response and a Query Response Length set to the query response length. This completes the GAS request/response exchange.

  - If the Qquery Rresponse is received from the Advertisement Server before the PostReplyTimer expires, and if the query response's length is less than or equal to the maximum MMPDU size and the Query Response is an aggregated response, the STA shall transmit a Group Addressed GAS Response frame containing a dialog token set to 0, a Status Code set to SUCCESS, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame or the Group Addressed Request frame, a GAS Comeback Delay set to 0, the Query Response and a Query Response Length set to the query response length, and a GAS Extension element containing a list of MAC Address/Dialog Token pairs in the Response Map Duples subfield of the GAS Extension element, identifying the requesting STAs and their Query Requests to which the Group Addressed GAS Response frame responds. This completes the GAS request/response exchange.

  - If the PostReplyTimer expires before the Query Response is received from the Advertisement Server for responding to a GAS Initial Request frame, then the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to GAS_QUERY_TIMEOUT (see Table 9-46), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0 and a Query Response Length set to 0. If the query response is subsequently received from the Advertisement Server, it shall be dropped by the responding STA.

  - If the PostReplyTimer expires before the Query Response is received from the Advertisement Server for responding to a Group Addressed GAS Request frame, then the responding STA shall not transmit an individually addressed GAS Initial Response frame or a Group Addressed GAS Response frame to the requesting STA. Note: If there is no response to the Query Request, the Advertisement Server may drop the request.

  - If the Query Response received from the Advertisement Server is larger than dot11GASQueryResponseLengthLimit or requires more than 128 fragments for transmission to the requesting STA, it shall be dropped by the responding STA. Then the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to GAS_QUERY_RESPONSE_TOO_ LARGE (see Table 9-46), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0 and a Query Response Length set to 0.

  - If the Query Response’s length is larger than the maximum MMPDU size, the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to SUCCESS, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 1 TU, and a Query Response Length set to 0; this indicates the Query Response will be transmitted using GAS Comeback Request and Response frames that support GAS fragmentation as follows. If the responding STA is capable of retransmitting a Query Response fragment upon request, the responding STA shall include, in the GAS Initial Response frame, a GAS Extension element with the Fragment Retransmission subfield of the GAS Flags field set to 1.