IEEE P802.11
Wireless LANs

| Misc CR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-01-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F1-17-A011S, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan | Huawei Technologies |  |  | ming.gan@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on D2.0.

The 14 comments resolved in this document are:

11405, 11406, 11407, 11634, 11637, 11638, 11654, 11655, 12673, , 12890, 13365, 14071, 14084, 14321

The changes marked in this document are based on TGax Draft D2.0.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 11405

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 10360 | 406.26 | 28.3.10.7.2 | "Set to the value of the SPATIAL_REUSE parameter of the TXVECTOR, which contains a value from Table 28-21 (Spatial Reuse subfield encoding for an HE SU PPDU, HE ER SU PPDU, and HE MU PPDU) subfield encoding for an HE MU PPDU, HE SU PPDU or HE ER SU PPDU, see 27.11.6 (SPATIAL_REUSE)". Sentence is not clear and has grammar error | as in the comment | Instruction to Editor: REVISE Agreed in principal, the detail changes are shown Please make the changes according to the changes in doc 18-0006r0 |

Instruction to Editor: Please make the following changes to P406, L26

Set to the value of the SPATIAL_REUSE parameter of the TXVECTOR, which contains a value from Table 28-21 (Spatial Reuse subfield encoding for an HE SU PPDU, HE ER SU PPDU, and HE MU PPDU) subfield encoding for an HE MU PPDU, HE SU PPDU or HE ER SU PPDU, see 27.11.6 (SPATIAL_REUSE).

## CID 11406, CID 13365

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 11406 | 411.57 | 28.3.10.7.2 | "STBC is not applied in MU-MIMO RUs." and "STBC is not applied in RUs that are used for MUMIMO allocation" are duplicated. Remove the 2nd statement | as in the comment | Instruction to Editor: REVISE Agreed with the duplicated description. The first statement is deleted since the 2nd statement is more clear Please make the changes according to the changes in doc 18-0006r0 |
| 13365 | 411.00 | 28.3.10.7.2 | Text " STBC is not applied in MU-MIMO RUs" and "STBC is not applied in RUs that are used for MU-MIMO allocations" | seems redundant, can remove one of the them | Instruction to Editor: REVISE Agreed with the duplicated description. Please make the changes according to the changes in doc 18-0006r0 |

Instruction to Editor: Please make the following changes to P411, L57

STBC is not applied in MU-MIMO RUs.

## CID 11407

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 11407 | 418.33 | 28.3.10.7.2 | Add the defintion of m sequence | as in the comment | Instruction to Editor: REVISED Agreed the definition of m is needed. Please make the changes according to the changes in doc 18-0006r0 |

Instruction to Editor: Please add the following paragraph to P418, 49

mL is the serial input as shown in figure 28-23—CRC calculation

## CID 11634

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 11634 | 404.59 | 28.3.10.7.2 | Lets define or provide references to definitions of all the variables. | as in comment | Instruction to Editor: REVISED Agreed that the variables should be either defined or referred. Please make the changes according to the changes in doc 18-0006r0 |

Discussion:

In equation (28-15) as shown above, most of the varialbes are defined in the descriptions of previous equations. The locations where the descriptions first show up are shown below. Only the descriptions of iBW  and NSTS are missing. Although it is better to add all the descrioptions in every equation, however, it is also big overhead. Hence, it is suggetsed to refer the equations where the descriptions can be found.

[Above eq (28-1]] iseg: frequency segment index

[Above eq (28-1]] iTX: transmit chain index

[Missing in Chapter 28, first shown in equation (28-8)]NSTS is defined in Table 28-15—Frequently used parameters

[Description of (28-3/4)]  is defined in Table 28-16—Number of modulated subcarriers and guard interval duration values for HE PPDU fields

[Description of (28-3/4)] is given in 17.3.2.5 (Mathemat-ical conventions in the signal descriptions).

[Description of (28-6)] N20MHz is defined in 21.3.7.3 (Channel frequencies)

[Missing, first shown in eq (28-6)] iBW

[Description of (28-3/4)] , where  is defined in the description of equation (28-3) and (28-4)

[Description of (28-8)] , where  is defined in Equation (28-50)

[Description of (28-3/4)]

[Description of (28-12)]

[Defined (28-15)] p1

[Description of (28-12)] Pk

[Description of (28-6)] KShift(iBW)

[Description of (28-3/4)] is given in Table 28-12 (Timing-related constants)

[Description of (28-3/4)]

[Description of (28-3/4)]

Instruction to Editor: Please add the following paragraph to P401, 43

NSTS is defined in Table 28-15—Frequently used parameters.

Instruction to Editor: Please add the following paragraph to P401, 15

iBW is the index of 20MHz channels,

Instruction to Editor: Please add the following paragraph to P405, 3

Please refer to the description of equation (28-1, 3, 4, 6, 8, 12) for the definitions of the other variables.

## CID 11654&11655

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 11654 | 451.39 | 28.3.10.10 | Provide definitions or references for all variables in Eq 28-53. | as in comment | Instruction to Editor: REVISED Agreed that the variables should be either defined or referred to. Please make the changes according to the changes in doc 18-0006r0 |
| 11655 | 451.39 | 28.3.10.10 | what is the difference between K_r^HE-LTF and K_r. | as in comment | Instruction to Editor: REVISED The descriptions of K_r^HE-LTF and K_r is in the description of (28-5), suggest to refer to the previous description Please make the changes according to the changes in doc 18-0006r0 |

Discussion

Equation (28-53) and Equation (28-54) are waveforms of HE-LTF for SU/MU/ER SU and TB PPDU respectively. The descriptions should be given together. Moreover, most of the definitions of varialbes are given in the descriptions of previuos equations. It is better to refer to them, rather than repeat again.

Some varialbes are listed here

[Descriptions are Missing, first shown up in eq 28-3]NRU

[Description of (28-3/4)]

[Description of (28-1)]

[In (28-34)]

[Description of (28-4)]

Instruction to Editor: Please make the following changes to P451, 63:

Where in equations (28-53) and (28-54), the following notations are used:

Instruction to Editor: Please add the following paragraph to P451, 65:

is the number of HE MU PPDU recipients (see Table 28-15 (Frequently used parameters)) in RU r

Please refer to the description of equation (28-5) for the definitions of  and .

Please refer to the description of equation (28-1, 3, 4, 6, 8, 12, 34) for the definitions of the other variables.

Instruction to Editor: Please add the following paragraph to P397, 50:

NRU is defined in Table 28-15—Frequently used parameters

## CID 11637

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 11637 | 407.8 | 28.3.10.7.2 | Consider the case of HE ER SU PPDU, Doppler=0, DCM=1 and STBC=1. What do I set "Nsts And Midamble Periodicity" to if I have one space-time stream? "Set to 0" does not apply since it says "when the STBC subfield is set to 0". "Set to 1" does not apply because it says "for two space-time streams". | as in comment | Instruction to Editor: REJECT Reason: When STBC=1, 1SS makes 2 space-time streams, hence it applies to the case of “Set to 1” |

## CID 11638

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 11638 | 407.20 | 28.3.10.7.2 | Consider the case of HE ER SU PPDU, Doppler=1, DCM=1 and STBC=1. What do I set "Nsts And Midamble Periodicity" to if I have one space-time stream? "Set to 0" does not apply since it says "when the STBC subfield is set to 0". "Set to 1" does not apply because it says "for two space-time streams". | as in comment | Instruction to Editor: REJECT Reason: When STBC=1, 1SS makes 2 space-time streams, hence it applies to the case of “Set to 1” |

## CID 12673

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 12673 | 142.35 | 9.4.2.237.3 | The MU Beamformer subfield has no associated behaviour | Delete from 9.4.2.237.3 (figure and table) and from 27.6.2 (last sentence first bullet, and second bullet) | Instruction to Editor: REJECT On one hand, the Tx capability is up to the AP, to decide if to act as a MU Beamformer or not. Whilst on the other hand, the non-AP STA can use this information to decide if or not to associate to one AP or not. Moreover, in Page 1097 of P802.11-2016, MU Beamformer Capable also exists in Table 9-249—Subfields of the VHT Capabilities Information field Considering multiple aspects, it is better to follow VHT and keep it there. |

## CID 12890

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 12890 | 239.6 | 27.4.4.1 | "A STA that sends a PPDU to an intended recipient " -- the STA is not going to send the PPDU to an unintended recipient | Delete "to an intended recipient" in the cited text | Instruction to Editor: ACCEPTED |

## CID 14071

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 14071 | 412.28 | 28.3.10.7.2 | range "of" | Change "range 4 to 7" to "range of 4 to 7". Also on line 30. | Instruction to Editor: ACCEPTED |

## CID 14084

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 14084 | 463.6 | 28.3.11.9 | Is "b0 b1 b2 b3 b4 b5 b6 b7 b8 b9" in Figures 28-35 ~ 38 defined? | Define "b0 b1 b2 b3 b4 b5 b6 b7 b8 b9". | Instruction to Editor: REVISED Here we can follow 256-QAM, and just mentions that: The bit string convention in Figure 28-35, Figure 28-36, Figure 28-37, and Figure 28-38 follows the bit string convention outlined in 17.3.5.8. Please make the changes according to the changes in doc 18-0006r0 |

Instruction to Editor: Please add the following sentence to the end of P463, L2:

The bit string convention in Figure 28-35, Figure 28-36, Figure 28-37, and Figure 28-38 follows the bit string convention outlined in 17.3.5.8.

## CID 14321

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 14321 | 432.32 | 28.3.10.8.6 | There is no clear definition of load balancing. Please define it before using it in the text of behavior description. Also there is a seperated section talking about network load balancing. It may confuse people using the same term for different technical purpose. | as in the comment | Instruction to Editor: REVISED It does exist the definition in P432, L32: In case of load balancing for RUs of size greater than 242 subcarriers where User fields corresponding to the same MU-MIMO allocations are split into two HE-SIG-B content channels. However, it is not in the first place where “load balancing for RUs of size greater than 242 subcarriers” shows. Suggest to also have the definition where the term first shows. Please make the changes according to the changes in doc 18-0006r0 |

Instruction to Editor: Please make the following changes to the P429, L8:

In the case of load balancing for RUs of size greater than 242-tone RU where User fields corresponding to the same MU-MIMO allocations are split into two HE-SIG-B content channels, y2y1y0 = 000-111 indicates number of User fields in the HE-SIG-B content channel that contains the corresponding 8-bit RU Allocation subfield.