IEEE P802.11
Wireless LANs

| HE BSS Load Information Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018 – 01 - 15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Frank Hsu | MediaTek Inc. |  |  | frank.hsu@mediatek.com |
| Gabor Bajko | MediaTek Inc. |  |  | gabor.bajko@mediatek.com |
| James Yee | MediaTek Inc. |  |  | james.yee@mediatek.com |

Abstract

This submission proposes resolutions for comments of TGax Draft 2.0 with the following CIDs:

12016, 13045

Revisions:

-	Rev 0: Initial version of the document.

| CID | Commenter | P.L | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 12016 | James Yee | 118.01 | 9.4.2 | With the introduction of new features such as OFDMA and UL MU MIMO, the existing BSS load elements (9.4.2.160 & 9.4.2.28), which address STA numbers, primary/secondary channel busy condition and DL MU-MIMO underutilization (11ac) are not sufficient for addressing the BSS load status in a 11ax BSS. A further enhanced BSS Load element needs to be defined. | Add a new information element or element extension to define 11ax BSS Load. The new IE shall address utilization status of OFDMA as well as UL/DL MU MIMO, as well as provisions to allow future extensions. | Revised. Agree with the comment. Proposed resolution adds HE BSS load element accordingly. TGax editor to make the changes shown in 11-18/0083r0 under all headings that include CID 12016. |
| 13045 | Ming Gan | 118.01 | 9.4.2 | BSS load element provides the channel utilization such that the unassociated STA can choose the proper AP, and extended BSS load element further provides the the spatial stream underutilization given the busy channel such that unassociated STA with MU-MIMO capability can choose the proper AP. Now 11ax introduce OFDMA, there is the probability of frequence underutilization given the busy channel. | Define a HE BSS load element considering frequency utilization such to help unassociated STA to choose a best AP | Revised. Agree with the comment. Proposed resolution adds HE BSS load element accordingly. TGax editor to make the changes shown in 11-18/0083r0 under all headings that include CID 13045. |

Discussion:

None.

- Instruction to Editor: Add the following section in the next version of 11ax specification as a new information element

The HE BSS Load element contains information reported from AP on channel bandwidth underutilization, MU DL and UL PPDU utilization, SU DL PPDU utilization, and load reports by mean available RU sizes in the BSS. The element format is defined in Figure x-xx0 (HE BSS Load element). This element may be used by a STA to select an appropriate AP to associate with based on implementation specific AP selection algorithm.

|  | Subelement ID | Length | Element ID Extension | HE STA count | MU UL PPDU Utilization | MU DL PPDU Utilization | SU DL PPDU Utilization |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 1 | 1 | 1 |

|  | Frequency UL Underutilization | Frequency DL Underutilization | Spatial UL Underutilization | Spatial DL Underutilization | Optional Subelements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | Variant |

Figure x.xx0 HE BSS load element format

The Element ID and Length fields are defined in 9.4.2.1. (General).

The Element ID Extension field is defined in 9.4.2.1.

The HE STA count indicates the total number of STAs currently associated with this BSS which have transmitted the HE Capabilities element.

The W1 DL PPDU Utilization defines the time percentage, linearly scaled with 255 representing 100%, that the medium is acquired by AP to transmit W1 DL PPDUs over a measurement interval. This percentage is computed using the formula,

where

TW1DL is computed as the sum of the periods which are used for W1 DL PPDUs over the measurement interval,

dot11ChannelUtilizationBeaconIntervals represents the number of consecutive beacon intervals during which the W1 DL PPDU utilization is measured. The default value of dot11ChannelUtilizationBeaconIntervals is defined in Annex C.

The formula above is applied to either DL PPDU Utilization of SU or MU by setting W1 to be SU or MU, respectively.

The MU UL PPDU Utilization defines the time percentage, linearly scaled with 255 representing 100%, that the medium is used for MU UL PPDU transmission from STAs over a measurement interval. This percentage is computed using the formula,

where

TMUUL is computed as the sum of the periods which are used for MU UL PPDU transmission over the measurement interval, and dot11ChannelUtilizationBeaconIntervals is the same as defined in MU DL PPDU utilization.

The Frequency W2 Underutilization is defined as the percentage, linearly scaled with 255 representing 100%, which the AP has underutilized the frequency domain resources for given periods that the medium is used for MU W2 PPDU transmission. This percentage is computed as using the formula,

where

TMUW2,j is jth MU W2 PPDU period,

N is the number of MU W2 PPDU transmission periods during the measurement interval,

Tbusy,W2 is , the sum of periods used for MU W2 PPDU transmission,

RUi,j is the RU weight of the ith RU transmitted or interfered over the jth MU W2 PPDU period and RU weight is the weighting of RU’s bandwidth over the channel bandwidth. M is the number of RUs which is interfered or used for transmission.

The formula above is applied to either Channel BW Frequency UL or DL Underutilizations by setting W2 to UL or DL, respectively.

The Spatial W3 Underutilization is defined as the percentage, linearly scaled with 255 representing 100%, which the AP has underutilized the spatial domain resources for given periods that the medium is used for MU W3 PPDU transmission. This percentage is computed as using the formula,

where

Nmax_SS is the maximum number of spatial streams supported by the AP,

TMUW3,j is jth MU W3 PPDU period,

RUMi,j is the RU weight of the ith transmitted RU whose size is at least 106-tone over the jth MU W3 PPDU period and Mj is the number of such RUs allocated for transmission,

N is the number of MU W3PPDU transmission periods during the measurement interval,

Tbusy,W3 is defined as , the weighted sum of periods used for MU W3 PPDU transmission,

NSS,i,j is the number of spatial streams used for the ith transmitted RU whose size is at least 106-tone over the jth MU W3 PPDU period.

The formula above is applied to either Spatial UL or DL Underutilizations by setting W3 to UL or DL, respectively.

The element is allowed to carry optional subelements defined in Table x.xx2.

- Table x-xx2 — Optional subelement IDs for HE BSS Load subelement

| Subelement ID | Subelment name | Extensible |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Active STA count | Yes |
| 2 | Active HE STA count | Yes |
| 3 | BSS Load |  |
| 4 | Extended BSS Load |  |
| 5-255 | Reserved |  |

The Active STA count subelement format is defined in Figure x.xx2.

The Active MU capable STA count subelement format is defined in Figure x.xx3.

|  | Subelement ID | Length | Active STA count |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure x.xx2 Active STA count subelement format

|  | Subelement ID | Length | Active HE STA count |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure x.xx3 Active MU capable STA count subelement format

The Subelement ID field is 1 to uniquely identify this subelement as the Active STA counts subelement.

The Subelement ID field is 2 to uniquely identify this subelement as the Active HE STA counts subelement.

An active STA is defined as during the measurement interval, the AP has successfully received at least one MPDU from that STA to the AP. The measurement interval is the same as defined in MU DL utilization measurement.

The Active STA count field indicates the number of active STAs among all STAs associated to the AP.

The Active HE STA count field indicates the number of active STAs among all HE STAs associated to the AP.

The Length fields of the subelements are as defined in 9.4.3 (Subelements).

The BSS Load subelement is the same as the BSS Load element as defined in 9.4.2.28 (BSS Load element)..

The Extended BSS Load subelement is the same as the Extended BSS Load element as defined in 9.4.2.160 (Extended BSS Load element).