802.11ba Draft Specification

| Spec Text for WUR Beacon Frame |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F1-17, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |

Abstract

This submission contains spec text to be incorporated in P802.11ba D0.1 related to these motions:

4.4	WUR Beacon

R.4.4.A: Define a WUR Beacon frame which can be transmitted periodically:

- The WUR Beacon is transmitted to WURs.

[Motion 4, March 2017, see [1] [24]]

4.9.3	Wake-up frame

- AP indicates a BSS parameter update by incrementing a counter in the wake-up frame.

[Motion, Sep 2017, see [4] [49]]

Revision History:

- Rev 0: Initial version of the document

Editing instructions formatted like this are intended to be copied into the TGba Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGba Editor: Editing instructions preceded by “TGba Editor” are instructions to the TGba editor to modify or insert material in the TGba draft.  As a result of adopting the changes, the TGba editor will execute the instructions rather than copy them to the TGba Draft.

- Abbreviations and acronyms

TGba Editor: Instruction: Insert a new acronym as follows:

TWBTT	Target WUR beacon transmission time

TGba Editor: Instruction: Insert a new subclause as follows:

- Maintaining synchronization

31.3.1 General

31.3.2 WUR Beacon generation

The AP shall define the timing for WUR by transmitting WUR Beacon frames according to dot11WBeaconPeriod. This defines a series of TWBTTs exactly dot11WBeaconPeriod TUs apart. At each TWBTT, the AP shall schedule a WUR Beacon frame as the next frame for transmission according to the medium access rules specified in Clause 10 except that one of the following conditions is met

— There is zero WUR STA associated with this AP

— AP does not provide WUR service to any associated WUR STA

— All the PCRs of the associated WUR STAs are in awake state

NOTE—To achieve this requirement, the AP suspends any pending transmissions until the WUR beacon has been transmitted.

The WUR beacon period is included in WUR mode element sent by AP, and a STA with WURx shall adopt that WUR beacon period when joining the BSS, i.e., the STA sets dot11WBeaconPeriod to that WUR beacon period.

NOTE—Though the transmission of a WUR Beacon frame might be delayed because of CSMA deferrals, subsequent WUR Beacon frames are scheduled at the undelayed nominal WUR beacon interval.

TGba Editor: Instruction: Add the following paragraph to the subcaluse of 9.10.3.2 WUR wake up frame:

- MAC frame format for Wake Up Radio (WUR) frames

9.10.3.2 WUR Wake Up frame format

TD control field of a WUR Wake Up frame which contains CL WUR Wake Up frame and VL WUR Wake Up frame contains Counter subfield and reserved subfield as illustrated in Figure 9-AA3 (TD control field of WUR Wake up frame).

| B0 BX | BX+1 B11 |
| --- | --- |
| Counter | Reserved |
| TBD | TBD |
| Figure 9-AA3 – TD control field of WUR Wake up frame |  |

A counter subfield of CL WUR Wake Up frame and VL WUR Wake Up frame is defined as an unsigned integer initialized to 0, that increments when a critical update to the PCR’s BSS parameters has occurred. The size of the counter subfield is TBD.