IEEE P802.11
Wireless LANs

| 802.11 [802.11az Trigger Frame Format] (relative to REVmd D0.5) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-01-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Jonathan Segev | Intel Corporation |  |  | jonathan.segev@intel.com |
| Chittabrata Ghosh | Intel Corporation |  |  |  |

Abstract

This submission proposes P802.11az draft amendment text for the P802.11az Negotiation Protocol. The baseline documents that this proposal depends on are:

- D0.05 of REVmd

- D8.0 of PIEEE802.11aj

- D5.0 of PIEEE802.11ak

- D13.0 of PIEEE802.11aq

History:

R0: Initial Version

[Modify Section 9.3.1.23 as follows:]

The Trigger Type subfield indicates the type of the Trigger frame and the encoding is defined in Table 9-25b

(Trigger Type subfield encoding).

Table 9-25b—Trigger Type subfield encoding

| Trigger Type field value | Description |
| --- | --- |
| 0 | Basic Trigger |
| 1 | Beamforming Report Poll (BRP) |
| 2 | MU-BAR |
| 3 | MU-RTS |
| 4 | Buffer Status Report Poll (BSRP) |
| 5 | GCR MU-BAR |
| 6 | Bandwidth Query Report Poll (BQRP) |
| 7 | NDP Feedback Report Poll |
| 8 | Location |
| 89-15 | Reserved |

|  | AID12/ | Allocation | Type |  |  |  |  |  | User Info |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 9-52g—User Info field

ID12 subfe the  STA for which the User Info field is intended. An AID12/RID12 sat is 0 or2ontiguous RUs for random access (see 27.5.5 (UL OFDMA-based raccess ubfield set to 4095 is reserved to indicate start of Padding field (see 27.5.3.2.2 (Padding for Trigger frame or frame containing UMRS Control field)).[Questions for the group for HEz UL sounding subtype:]

Probably redundant as we use the BW field value from the TF Common Info field

AFAIK entire BW allocated to each STA.?>>>> specify which values.

Coding Type – value 0 is BCC value 1 is LDPC, neither should be relevant. i.e. mark as ignored.

DCM (Dual Carrier Modulation) – DCM value 0 indicates DCM is not used.

s used in the response to the TF.The start number of the SS and the number of SS. For HEz is used as is.

nt

e format is defined in Figure 9-52??c (Trigger frame). The RAfield is set to bro

The Trigger Dependent Common Info subfield is always present in the Location Trigger frame,  and is descrbed in Figure YYYY9-52??.

|  | Location trigger subtype | SAC | Reserved |
| --- | --- | --- | --- |
| Bits: | 4 | TBD2403 | 4 |

Figure 9-52?? - Trigger Dependent Common Info subfield for the Location Trigger Frame

Value of the Trigger Subtype for the Location Trigger Frame variant is defined in Table XXXX.

Table xxxx — Location Trigger subtype of the Location Trigger Dependent Common Info Field

| Trigger Subtype field value | Description |
| --- | --- |
| 0 | HEz Poll |
| 1 | HEz Uplink Soonding |
| 2 | HEz LMR |
| 3 | Location Negotiation |
| 4-15 | Reserved |

The SAC (Security Authentication Code) subfield is valid if the Trigger Subtype field value is 1, i.e., HEz UL Sounding and reserved, otherwise. The SAC field provides the SAC value of the UL or DL sounding associated with the measurement instance (see Section 11.22.6.4.2.3 HEz Range Measurement Sounding)i.

The Location TF sub-variants shall be transmitted with a Trigger Frame MAC Padding Duration of 816usec.

The format of the Trigger Dependent User Info field for the Location Trigger Frame is dependent on the Trigger Subtype field value in the Location Trigger Dependent Common Info field (refer to Table XXXX).

9.3.1.23.9.1 HEz Poll Trigger Sub-variant

The Per User Info field in the HE Poll Trigger sub-variant is identical to the format of the Per User Info field in the Basic Trigger Frame (see Section 9.3.1.23.1), except that the AID12 subfield carries an AID12 for an associated locating STA or an RID12 for an unassociated locating STA.

a TD User In

9.3.1.23.9.2 HEz Uplink Sounding Sub-variant

The Trigger Dependent User Info subfield is present when the Trigger Subtype is HEz Uplink Sounding. The User Info field for the HEz Uplink Sounding sub-variant is defined in Figure 9-52??.

|  | B0 B11 | B12 B20 | B21 B24 | B25 | B26 B31 | B32 B38 | B3920 B632 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AID12/RID12 | RU Allocation Reserved | MCS Reserved | DCM Reserved | SS Allocation | Target RSSI | Reserved | Trigger Dependent User Info |
| Bits: | 12 | 9 | 4 | 1 | 6 | 7 | 14 | variable |

Figure 9-52??—User Info field for HEz Uplink Sounding sub-variant

The Trigger Dependent User Info subfield carries the SAC field. The length of this subfield is TBD.

9.3.1.23.9.3 HEz LMR Pub-variant

The Per User Info field in the HE Poll Trigger sub-variant is identical to the format of the Per User Info field in the Basic Trigger Frame (see Section 9.3.1.23.1), except that the AID12 subfield carries an AID12 for an associated locating STA or an RID12 for an unassociated locating STA.