IEEE P802.11
Wireless LANs

| 802.11 Proposed Resolution for CID1055, 1056 and 1057 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-05-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel |  |  | Emily.h.i@intel.com |
| Daniel Leiderman | Intel |  |  | daniel.leiderman@intel.com |
| Yaron Alpert | Intel |  |  | yaron.alpert@intel.com |
| Nehru Bhandaru | Broadcom |  |  | nehru.bhandaru@broadcom.com |

Abstract

This document provides normative text for Dynamic Generation of Password ID and provides resolutions for LB232 CID 1055, CID 1056, and CID 1057.

| 1055 | 2320.02 | 12.4.3 | What is "dot11RSNConfigPasswordValueTable? I couldn't find the definition in the draft. Should it have a definition in Annex C? | Please define it. |
| --- | --- | --- | --- | --- |
| 1056 | 819.39 | 9.3.3.12 | The Password Identifier element is included in the unprotected authentication frame. It may violate the privacy of users (household). For example, it exposes a group of devices and number of devices that are sharing the same password. Particularly, when these devices belongs to the same household (apartment) in an apartment building, it violates the privacy of users/residents. | Please provide a method to avoid privacy exposure, or commenter will bring a contribution, |
| 1057 | 2320.01 | 12.4.3 | "Password identifier" is introduced in D1.0. However, how to generate password identifier is not specified. Is the password identifier input (by user)? or automatically generated by some algorithm? More clarification on password identifier is needed. | Commenter will bring a contribution. |

Instruct the editor to insert a new row before the row “Last” in Table 9-31 as follows:

Table 9-31— Beacon frame body body

| Order | Information | Notes |
| --- | --- | --- |
| < TBD> | Password Salt | The Password Salt element is present if dot11SAEDynamicPasswordIdentifier is true; otherwise not present |

Instruct the editor to insert a new row before the row “Last -1” in Table 9-38 as follows:

Table 9-38— Probe Response frame body body

| Order | Information | Notes |
| --- | --- | --- |
| < TBD> | Password Salt | The Password Salt element is present if dot11SAEDynamicPasswordIdentifier is true; otherwise not present |

Instruct the editor to add a new row in table 9-88 as follows:

Table 9-87—Element IDs

| Element | Element ID | Element ID Extension | Extensible | Fragmentable |
| --- | --- | --- | --- | --- |
| Password Salt (see 9.4.2.a (Password Salt element)) | 255 | <ANA> | Yes | No |

Instruct the editor to add a new subclause at the end of 9.4.2:

9.4.2.a Password Salt element

The Password Salt element is used to derive dynamic password ID. The format of Password Salt element is defined in Figure 9-a (Password Salt element format).

| Element ID | Length | Element ID Extension | Password Salt |
| --- | --- | --- | --- |

Octets		1		1		        1			   16

Figure 9-a—Password Salt element format

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1 (General).

The Password Salt field is a random number to derive dynamic password ID, as specified in 12.4 (Authentication using a password).

Instruct the editor to modify section 12.4.3 as follows:

12.4.3 Representation of a password

Passwords are used in SAE to deterministically compute a secret element in the negotiated group, called a password element. The input to this process needs to be in the form of a binary string. For the protocol to successfully terminate, it is necessary for each side to produce identical binary strings for a given password, even if that password is in character format. There is no canonical binary representation of a character and ambiguity exists when the password is a character string. To eliminate this ambiguity, a STA shall represent a character-based password as an ASCII string. Representation of a character-based password in another character set or use of a password preprocessing technique (to map a character string to a binary string) may be agreed upon, in an out-of-band fashion, prior to beginning SAE. If the password is already in binary form (e.g., it is a binary preshared key) no character set representation is assumed. The binary representation of the password, after being transformed from a character representation or directly if it is already in binary form, is stored in the dot11RSNAConfigPasswordValueTable. When a “password” is called for in the description of SAE that follows the credential from the dot11RSNAConfigPasswordValueTable is used.

When a "password identifier" is called for in the description of SAE that follows and dot11SAEDynamicPasswordIdentifier is false, the identifier from the dot11RSNAConfigPasswordValueTable is used.

When a "password identifier" is called for in the description of SAE that follows and dot11SAEDynamicPasswordIdentifier is true, the identifier is defined as:

identifier = Truncate-32 ( HMAC-SHA-256 (password identifier, password salt))

Where

password salt 	a random number received in the Beacon or Probe Response frame .

Instruct the editor to modify C.3 as follows:

C.3 MIB detail

Dot11StationConfigEntry::=

SEQUENCE {

......

dot11SAEDynamicPasswordIdentifier			TruthValue }

dot11SAEDynamicPasswordIdentifier OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“.This is a capability variable.

Its value is determined by device capabilities.
This attribute, when true, indicates that the generaic generation of password identifier option is implemented and used.”

::= { dot11StationConfigEntry <ANA>}