802.11ba Draft Specification

| Proposed Text for BSS Parameters Update Notification Follow Up |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-05-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F1-17, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |

Abstract

This submission contains spec text to be incorporated in P802.11ba D0.2 related to the following passed straw polls in 11-18-0437-04-00ba-bss-parameters-update-notification-follow-up:

SP 1

- Do you agree the following text

SP 2

- Do you agree the following text

SP 3

- Do you agree the following text

Revision History:

- Rev 0: Initial version of the document

Editing instructions formatted like this are intended to be copied into the TGba Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGba Editor: Editing instructions preceded by “TGba Editor” are instructions to the TGba editor to modify or insert material in the TGba draft.  As a result of adopting the changes, the TGba editor will execute the instructions rather than copy them to the TGba Draft.

TGba Editor: Instruction: Insert a new subclause as follows:

  - PCR’s BSS Parameters Update Notification

The AP shall increase the value of the Counter field in the next transmitted broadcast WUR Wake Up frame when a critical update occurs to any of the elements inside the Beacon frame. The following events shall classify as a critical update:

  - Inclusion of  a Channel Switch announce element

  - Inclusion of an Extended Channel Switch announce element

  - Inclusion of a Wide Bandwidth Channel Switch element

  - Inclusion of an Operating Mode Notification element

  - Modification of the EDCA parameters

  - Modification of the HT Operation element

  - Modification of the VHT Operation element

  - Modification of the DSSS Parameter Set

  - Modification of the CF Parameter Set element

  - Inclusion of a Channel Switch Wrapper element

An AP may classify other changes in the Beacon frame as critical updates and among these updates can be included those that are described in 11.2.2.17 (TIM Broadcast).

The WUR STA that receives a Counter field that contains a value that is different from the previously received Counter field shall follow the procedure defined in 11.2.3.17 subject to its PCR delay constraints.

TGba Editor: Instruction: Change the following paragraph to the subcaluse of 9.10.3.2 WUR wake up frame:

9.10.3.2 WUR Wake Up frame format

The frame format of the WUR wake-up frame is as defined in Figure 9-747a (WUR frame format).

The Frame Control field is defined in 9.10.2.1.1 (Frame Control field).

The Frame Body may be present in the WUR wake-up frame, which signaling is TBD.

The Frame Body, if present, contains indication of multiple STAs. The signaling is TBD.

The Address field of the WUR wake-up frame is set to

—the Wake Up ID of the intended WUR STA when the frame is individually addressed

—the Group ID when the frame is group addressed

—the Transmit ID when the frame is broadcast addressed.

—0 when multiple WIDs are included in the frame body of the frame.

TD control field of a broadcast WUR Wake Up frame contains Counter subfield and reserved subfield as illustrated in Figure 9-xx (TD control field of broadcast WUR Wake Up frame).

|  | B0 B3 | B4 B11 |
| --- | --- | --- |
|  | Counter | Reserved |
| Bits | 4 | 8 |
| Figure 9-xx –TD control field of broadcast WUR Wake Up frame |  |  |

A counter subfield of a WUR Wake Up frame is defined as an unsigned integer initialized to 0, that increments when a critical update to the PCR’s BSS parameters has occurred.