IEEE P802.11
Wireless LANs

| Proposed Full Duplex Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-06-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F1-17, Huawei Base, Bantian, Longgang District, Shenzhen, China |  | ming.gan@huawei.com |
| Yan Xin | Huawei |  |  | yan.xin@huawei.com |
| Edward Au | Huawei |  |  | edward.ks.au@huawei.com |
| David Xun Yang | Huawei |  |  | david.yangxun@huawei.com |

### Abstract

This submission proposes the Full Duplex (FD) Functional Requirements to support the Full Duplex TIG Report [Ref-1].

R0: Initial version

R1: Change “shall” to “should”

- Introduction

  - Purpose

This document proposes requirements for solutions addressing functionality to be provided by the FD amendment, referred to as the FD Functional Requirements (FRs).

  - Scope

The scope for deriving functional requirements is outlined in the Full Duplex TIG Report [Ref-1] and Full Duplex Usage Model [Ref-2]. In this document, the functional requirements cover the following aspects:

- Bands and bandwidths of FD operations

- Maximum throughput over an allocated channel bandwidth

- Latency enhancement

- FD capability of AP STA and Non-AP STA

- Backward compatibility and co-existence with legacy 802.11 devices

- Functional Requirements

  - Bands and bandwidths of FD operations

- FD amendment should define operations in frequency bands between 1 GHz and 7.125 GHz.

- FD amendment should support 20 MHz, 40MHz and 80MHz bandwidths, and may support 160MHz and 320 MHz bandwidths.

- FD amendment may support different bandwidths for transmission and reception at the same time in full duplex transmission.

  - Maximum throughput over an allocated channel bandwidth

- The mechanisms defined in the FD amendment should provide at least one mode of operation capable of achieving up to two-time improvement in the throughput per station in an allocated channel bandwidth.

- The mechanisms defined in the FD amendment should improve average system throughput.

  - Latency enhancement

- The mechanisms defined in the FD amendment should provide at least one operation to improve the average transmission latency compared with legacy non-DMG devices.

  - FD capability of AP STA and non-AP STA

- FD amendment should enable AP STA to cancel certain amount of self-interference to provide sufficient SINR at receiver side in order to achieve throughput gain per station in an allocated channel bandwidth compared with half duplex transmission

- FD amendment may enable non-AP STA to cancel certain amount of self-interference to provide sufficient SINR at receiver side in order to achieve throughput gain per station in an allocated channel bandwidth compared with half duplex transmission.

- FD amendment should ensure no degradation on throughput for FD capable STA when it performs half duplex transmission.

  - Backward compatibility and co-existence with legacy 802.11 devices

- FD amendment should enable coexistence with legacy IEEE 802.11 devices operating in the same band.

- FD amendment should enable backward compatibility with legacy IEEE 802.11 devices operating in the same band.

- References

- 11-18-0498-00-00fd-framework-fd-tig-report

- 11-18-0758-00-00fd-full-duplex-usage-model