IEEE P802.11
Wireless LANs

| TGbb Simulation Scenarios |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-08-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Oliver Pengfei Luo | Hisilicon | Q20 Huawei Building, No. 156 Beiqing Rd. Beijing, 100095, China | +861060612361 | oliver.luo@hisilicon.com |
| Jiamin Chen |  |  |  | jiamin.chen@mail01.huawei.com |
| John Qiang Li |  |  |  | john.liqiang@hisilicon.com |
| Nikola Serafimovski | pureLiFi |  |  | nikola.serafimovski@purelifi.com |
| Chong Han | pureLiFi |  |  | Chong.han@purelifi.com |
| Tuncer Baykas | Istanbul Medipol University | Kavacik Mah. Ekinciler Cad. No.19 Kavacik Kavsagi-Beykoz, Istanbul 34810, Turkey |  | tbaykas@medipol.edu.tr |

Abstract

This document describes the simulation scenarios for IEEE802.11bb.

# Glossary of acronyms

- Glossary of acronyms

| Acronym | Defination |
| --- | --- |
| A-MPDU | Aggregated Mac Protocol Data Unit |
| A-MSDU | Aggregated MAC Service Data Units |
| ACK | Acknowledge |
| AP | Access Point |
| BSS | Basic Service Sets |
| BW | Bandwidth |
| CTS | Clear To Send |
| DL | Down Link |
| FOV | Filed Of View |
| FTP | File Transfer Protocol |
| FWHM | Full Width At Half Maximum |
| GI | Guard Interval |
| HCF | Hybrid Coordination Function |
| IP | Internet Protocol |
| IR | Infrared |
| LED | Lighting Emitting Diode |
| MAC | Media Access Control |
| P2P | Peer-To-Peer |
| PD | Photodiode |
| PHY | Physical Layer |
| PIN | P-Type, Intrinsic And N-Type |
| RTS | Request To Send |
| RX | Receiver |
| STA | Station |
| TCP | Transmission Control Protocol |
| TDLS | Tunneled Direct Link Setup |
| TX | Transmiter |
| UL | Up Link |

# Introduction

This document defines simulation scenarios to be used for

- Evaluation of performance of features proposed in TGbb.

- Generation of results for simulators calibration purpose.

Each scenario is defined by specifying

- Topology: AP/STAs positions, P2P STAs pair positions, obstructions , layout, propagation model

- Traffic model

  - UL: STA - AP traffic

  - DL: AP – STA traffic

  - P2P traffic (tethering, Soft-APs, TDLS)

  - ‘Idle’ management (generating management traffic such as probes/beacons)

- List of PHY, MAC, Management parameters

  - We may want to fix the value of some parameters to limit the degrees of freedom, and for calibration

Per each of above items, the scenario description defines a detailed list of parameters and corresponding values.

The Monte Carlo simulations necessary to show the statistical performance of the system should use the analytical channel model provided in doc. 11-17/0479r0 for stations at various locations and movement in the environment. Random blockages should be considered in the simulations as a break in the LoS between the AP and the STA.

The implementation of the blockages should be modelled by introducing an object in the space that models the spatial consistency of the blocking. A pseudo-code for the proposed blockage model will be provided in doc. XXXXXX.

The channel model for STA per time instance within a given environment should be simulated as follows:

- Distribute the Aps

- Distribute the STAs

- Determine LoS channel model (using the CIR provided in Doc. 11-18-1582r3 or the analytical model provided in doc. 11-17/0479r0)

- Determine random blockages of the LoS channel model using doc. XXXX)

The movement of an STA should constitute multiple time instances where the location and orientation of the STA is spatially correlated.

Values not specified can be set to any value.

Values included in square brackets [] are default values to be used for calibration.

All other parameters values not included in [], are to be considered mandatory for performance evaluation.

Simulation results should be presented together with the specification of the value used per each of the parameters in the tables.

# Scenarios summary

This document reports the initial agreement according to document 11-18/1422r0.

- Scenarios summary

|  | Scenario Name | Topology | Management | Channel Model | Traffic profile [tentative] |
| --- | --- | --- | --- | --- | --- |
| 1 | Industrial wireless | A - Industrial Robotic work cell e.g. ~8m x 10m x 7m size 2 STAs/AP, P2P pairs | Managed | Indoor- Manufacturing Cell | Industrial |
| 2 | Hospital ward | B - Dense small BSSs e.g. ~8 m × 8 m × 3 m size, ~1-3m inter AP distance, 4 of STAs/light, P2P pairs | Managed | Indoor- Office | Enterprise |
| 3 | Enterprise | C - Dense small BSSs e.g. ~ 6.8 m × 4.7 m × 3 m size ~1-3m inter AP distance 5 of STAs/light, P2P pairs |  |  |  |
|  |  |  |  |  | Enterprise |
| 4 | Residential | D - Apartment bldg. e.g. ~6m x 6m x 3m size, ~0.5-2 m inter AP distance 5 of STAs/light, P2P pairs | Managed | Indoor-Home | Home |

# 1-Industrial wireless

- Topology for industrial wireless scenario

| Topology (A) |  |
| --- | --- |
| (a) Transmitters Receivers (b) (c) - Topology for industrial wireless scenario |  |
| Parameter | Value |
| Environment description | 1 Industrial Robotic work cell - Floors hight: 7 m - Work cell size:8m x 10m x 7m |
| APs location | 1 per work cell, located on the top of the work cell boundary looking in the direction of the robots |
| STAs location | 2 per AP, one installed at the head of each robot, with multiple transceivers facing different directions |
| Channel Model | industrial |
| - PHY parameters for industrial wireless scenario |  |
| PHY parameters |  |
| BW: | [up to 250 MHz] |
| Wavelength: | IR (780 - 5000nm) for Downlink, IR (780 - 5000nm) for Uplink |
| GI: | to be specified by proposer |
| Data Preamble: | to be specified by proposer |
| STA TX power | 1 W optical / LED |
| AP TX Power | 1 W optical / LED |
| Modualtion depth | to be specified by proposer |
| Photodiode model | Hamamatsu S6968 |
| LED model | SFH 4716AS OSRAM Opto Semiconductors |
| Luminaire specifications | See channel model document (11-18-1582r3) |
| Receiver specifications | See channel model document (11-18-1582r3) |
| Noise floor | -70dBm [-174 dBm/Hz] |
| - MAC parameters for industrial wireless scenario |  |
| MAC parameters |  |
| Access protocol parameters: | [HCF?] |
| Aggregation: | [A-MPDU / A-MSDU?] |
| Max # of retries | [5] |
| RTS/CTS | [on/off?] |
| Rate adaptation method | [TBD in Evaluation Methodology] |
| Association | Each STA associated with the AP in same cell |

- Traffic model for industrial wireless scenario

| Traffic model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | Source/Sink | Name | Transport Protocol | Average rate [Mbps] | MSDU size [B] | Max. Delay [ms] | Max. PLR | PKT arrival distribution | AC |
| Dowlink |  |  |  |  |  |  |  |  |  |
| D0 | AP/STA1 | Local file transfer |  | max rate |  |  |  |  |  |
| D1 | AP/STA1 | 4k Video | … |  |  |  |  |  |  |
| D2 | AP/STA2 | Browsing | … |  |  |  |  |  |  |
| D3 | AP/STA3 | … |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  |  |
| DN | AP/STAN |  |  |  |  |  |  |  |  |
| Uplink |  |  |  |  |  |  |  |  |  |
| U1 | STA1/AP |  |  |  |  |  |  |  |  |
| U2 | STA2/AP |  |  |  |  |  |  |  |  |
| U3 | STA3/AP |  |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  |  |
| UN | STAN/AP |  |  |  |  |  |  |  |  |
| P2P |  |  |  |  |  |  |  |  |  |
| P1 | STA1/STA2 |  |  |  |  |  |  |  |  |
| P2 | STA3/STA4 |  |  |  |  |  |  |  |  |
| P3 | STA5/STA6 |  |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  |  |
| PN/2 | STAN-1/ STAN |  |  |  |  |  |  |  |  |
| Idle Management |  |  |  |  |  |  |  |  |  |
| M1 | AP1 | Beacon |  |  | X Bytes |  |  | 1/Xms |  |
| M2 | STA2 | Probe Req. |  |  | X Bytes |  |  | 1/Xs |  |
| M3 | STA3 |  |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  |  |
| MN | STAN |  |  |  |  |  |  |  |  |

# 2 - Hospital ward

- Topology for hospital ward scenario

| Topology (A) |  |
| --- | --- |
| (a) (b) (c) - Topology for hospital ward scenario |  |
| Parameter | Value |
| Environment description | 1 hospital ward - Floors height: 3 m - Ward size:8m x 8m x 3m |
| AP location | 16 lights per ward, 4 lights per AP (4 AP in total), installed on the ceiling See channel model document (11-18-1582r3) |
| STAs location | See channel model document (11-18-1582r3) |
| Channel Model | hospital ward |
| - PHY parameters for hospital ward scenario |  |
| PHY parameters |  |
| BW: | [up to 250 MHz] |
| Frequency: | Visible Light for Downlink, IR for Uplink |
| GI: | to be specified by proposer |
| Data Preamble: | to be specified by proposer |
| Modualtion depth | to be specified by proposer |
| STA TX power | 200 mW optical / LED |
| AP TX Power | 10 W optical / LED (380 -790 nm) |
| TX beam angle of AP | 90 degrees |
| Photosensitive area | 1 cm2 PIN PD |
| Photodiode model | Hamamatsu S6968 |
| LED model |  |
| Luminaire specifications | See channel model document (11-18-1582r3) |
| Receiver specifications | See channel model document (11-18-1582r3) |
| Noise floor | -70 dBm |
| - MAC parameters for hospital ward scenario |  |
| MAC parameters |  |
| Access protocol parameters: | [HCF?] |
| Aggregation: | [A-MPDU / A-MSDU?] |
| Max # of retries | [5] |
| RTS/CTS | [on/off?] |
| Rate adaptation method | [TBD in Evaluation Methodology] |
| Association | Each STA associated with the AP in same cell |

# 3 - Enterprise

- Topology for enterprise scenario

| Topology (A) |  |
| --- | --- |
| (a) (b) (c) (d) - Topology for enterprise scenario |  |
| Parameter | Value |
| Environment description | 1 office with/without cubicles - Floors hight: 3 m - Office size: 6.8 m × 4.7 m × 3 m |
| APs location | 10 lights per office, 3 or 4 lights per AP (3 AP in total), installed on the ceiling |
| STAs location | See channel model document (11-18-1582r3) |
| Channel Model | office |
| - PHY parameters for enterprise scenario |  |
| PHY parameters |  |
| BW: | [up to 300 MHz] |
| Frequency: | Visible light (DL) and IR (UL) |
| Wavelength: | Visible Light (380 -780 nm) for Downlink, IR (780 - 5000nm) for Uplink |
| GI: | to be specified by proposer |
| Data Preamble: | to be specified by proposer |
| STA TX power | 200 mW optical |
| Modualtion depth | to be specified by proposer |
| Luminaire specifications | See channel model document (11-18-1582r3) |
| Receiver specifications | See channel model document (11-18-1582r3) |
| Noise floor | -70 dBm |
| - MAC parameters for enterprise scenario |  |
| MAC parameters |  |
| Access protocol parameters: | [HCF?] |
| Aggregation: | [A-MPDU / A-MSDU?] |
| Max # of retries | [5] |
| RTS/CTS | [on/off?] |
| Rate adaptation method | [TBD in Evaluation Methodology] |
| Association | Each STA associated with the AP in same cell |

# 4 – Residential

# Topology for residential scenario

| Topology (A) |  |
| --- | --- |
| (a) living room Arrangement of luminaries (b) (c) - Topology for enterprise scenario |  |
| Parameter | Value |
| Environment description | A living room with table, chairs, couch, coffee table and human bodies - Floors hight: 3 m - Room size:6m x 6m x 3m |
| APs location | 9 lights per office, 3 lights per AP (3 AP in total), installed on the ceiling |
| STAs location | See channel model document (11-18-1582r3) |
| Channel Model | home |
| - PHY parameters for residential scenario |  |
| PHY parameters |  |
| BW: | [up to 300 MHz] |
| Frequency: | Visible light (DL) and IR (UL) |
| Wavelength: | Visible Light (380 -780 nm) for Downlink, IR (780 - 5000nm) for Uplink |
| GI: | to be specified by proposer |
| Data Preamble: | to be specified by proposer |
| STA TX power | 200 mW optical |
| Modualtion depth | to be specified by proposer |
| Luminaire specifications | See channel model document (11-18-1582r3) |
| Receiver specifications | See channel model document (11-18-1582r3) |
| Noise floor | -70 dBm |
| - MAC parameters for residential scenario |  |
| MAC parameters |  |
| Access protocol parameters: | [EDCA with default EDCA Parameters set] |
| Aggregation: | [A-MPDU] |
| Max # of retries | [5] |
| RTS/CTS | [on] |
| Rate adaptation method | [TBD in Evaluation Methodology] |
| Association | Each STA associated with the AP in same cell |

# Annex 1 - Reference traffic profiles per scenario

Reference traffic profile for Scenario 1

- Reference traffic profile for Scenario 1

| Traffic Model # | Traffic model name | Description | Application traffic (Forward / Backward) | Application Load (Mbps) (Forward / Backward) | A-MPDU Size (B) (Forward / Backward) |
| --- | --- | --- | --- | --- | --- |
| T1 | Local file transfer | FTP/TCP transfer of large file within local network | FTP file transfer / FTP TCP ACK | Full buffer / 0.1 | Max A-MPDU / 64 |
| T2 | Lightly compressed video |  |  |  |  |
| T3 | Internet streaming video/audio |  |  |  |  |
| T4 | 4k video streaming |  |  |  |  |
| T5 | Online game server |  |  |  |  |
| T6 | Management: Beacon |  |  |  |  |
| T7 | Management: Probe requests |  |  |  |  |

Reference traffic profile for Scenario 2

- Reference traffic profile for Scenario 2

| Traffic Model # | Traffic model name | Description | Application traffic (Forward / Backward) | Application Load (Mbps) (Forward / Backward) | A-MPDU Size (B) (Forward / Backward) |
| --- | --- | --- | --- | --- | --- |
| T1 | Local file transfer | FTP/TCP transfer of large file within local network | FTP file transfer / FTP TCP ACK | Full buffer / 0.1 | Max A-MPDU / 64 |
| T2 | Lightly compressed video |  |  |  |  |
| T3 | Internet streaming video/audio |  |  |  |  |
| T4 | 4k video streaming |  |  |  |  |
| T5 | Online game server |  |  |  |  |
| T6 | Management: Beacon |  |  |  |  |
| T7 | Management: Probe requests |  |  |  |  |
| T8 | Multicast Video Streaming | UDP/IP transfer of compressed video streaming | UDP packet transfer/Nothing | 3-6Mbps/Nothing |  |

Reference traffic profile for Scenario 3

- Reference traffic profile for Scenario 3

| Traffic Model # | Traffic model name | Description | Application traffic (Forward / Backward) | Application Load (Mbps) (Forward / Backward) | A-MPDU Size (B) (Forward / Backward) |
| --- | --- | --- | --- | --- | --- |
| T1 | Local file transfer | FTP/TCP transfer of large file within local network | FTP file transfer / FTP TCP ACK | Full buffer / 0.1 | Max A-MPDU / 64 |
| T2 | Lightly compressed video |  |  |  |  |
| T3 | Internet streaming video/audio |  |  |  |  |
| T4 | 4k video streaming |  |  |  |  |
| T5 | Online game server |  |  |  |  |
| T6 | Management: Beacon |  |  |  |  |
| T7 | Management: Probe requests |  |  |  |  |

Reference traffic profile for Scenario 4

- Reference traffic profile for Scenario 4

| Traffic Model # | Traffic model name | Description | Application traffic (Forward / Backward) | Application Load (Mbps) (Forward / Backward) | A-MPDU Size (B) (Forward / Backward) |
| --- | --- | --- | --- | --- | --- |
| T1 | Local file transfer | FTP/TCP transfer of large file within local network | FTP file transfer / FTP TCP ACK | Full buffer / 0.1 | Max A-MPDU / 64 |
| T2 | Lightly compressed video |  |  |  |  |
| T3 | Internet streaming video/audio |  |  |  |  |
| T4 | 4k video streaming |  |  |  |  |
| T5 | Online game server |  |  |  |  |
| T6 | Management: Beacon |  |  |  |  |
| T7 | Management: Probe requests |  |  |  |  |

# Annex 2 – Traffic model descriptions

Management traffic profiles

Note: To be defined, probing profile.

References for traffic models

- 11-13/486, “HEW video traffic modeling” Guoqing Li et al, (Intel) [1] 11-13-1162-01-hew-vide-categories-and-characteristics

- 11-13-1059-01-hew-video-performance-requirements-and-simulation-parameters

- 11-09-0296-16-00ad-evaluation-methodology.doc

- Rongduo Liu et al., “An Emperical Traffic Model of M2M Mobile Streaming Services ”, International conference C on Multimedia information networking and security, 2012

- JO. Rose, “ Statistical properties of MPEG video traffic and their impact on traffic modeling in ATM systems ”, Tech report, Institute of CS in University of Wurzburg

- Savery Tanwir., “A survey of VBR traffic models”, IEEE communication surveys and tutorials, Jan 2013

- Aggelos Lazaris et al., “A new model for video traffic originating from multiplexed MPEG-4 videoconferencing streams”, International journal on performance evaluation, 2007

- A. Golaup et al., “Modeling of MPEG4 traffic at GOP level using autoregressive process”, IEEE VTC, 2002

- K. Park et al., “Self-Similar network traffic and performance evaluation”, John Wiley&Son, 2000

- M Dai et al., “A unified traffic model for MPEG-4 and H.264 video traces”, IEEE Trans. on multimedia, issue 5 2009.

- L Rezo-Domninggues et al., “Jitter in IP network: A cauchy approach”, IEEE Comm. Letter, Feb 2010

- Hongli Zhang et al., “Modeling Internet link delay based on measurement”, International conference on electronic computer technology, 2009.

# Annex 3 - Traffic models

- lightly compressed video (assuming H.264 I-frame only)

  - Requirements

    - Application PLR: 1e-8

    - Delay: 10 ms

  - Parameters

    - Slice inter-arrival time (IAT) = 1/4080 seconds (1/8100 and 1/16200 seconds for 4K and 8K respectively)

    - µ = 15.798 Kbytes

    - σ = 1.350 Kbytes

  - b = 515, 1023, 2047 Mbps (for 1080p, 2160p and 4320p respectively Algorithm for each video source – Input: target bit rate in Mbps (p); Output: slice size in Kbytes (L)

    - At each IAT, generate a slice size L with the following distribution: Normal(µ*(p/b), σ*(p/b))

      - If L > 92.160 Kbytes, set L = 92.160 Kbytes (1080p)

      - If L > 180 Kbytes, set L  = 180 Kbytes (2160p aka 4K)

      - if L > 360 Kbytes, set L = 360 Kbytes (4320p aka 8K)

- Video Conferencing (e.g., Lync) Traffic Model

Unlike video streaming where video traffic is unidirectional, video conferencing is two-way video traffic. The video traffic is generated at each station, send to AP, transverse the internet and reach another AP and then send to the destination.

Station layer model

- Station layer model for video conferencing

STA layering model is shown in Figure A2.1. Because the traffic from AP to station has experienced network jitter, it can be modelled the same way as the traffic model of video streaming.

For the traffic sent from Station to AP, since the traffic has not experienced network jitter, it is a periodic traffic generation as the first two steps described in video streaming.

Video traffic generation

Traffic model from AP to station: use the same model as video streaming.

Traffic model from station to AP: use the first two steps in video streaming traffic model

Evaluation metrics

- MAC throughput,  latency

Application event models

Application event model is used to specify the patterns of the application events, i.e., when to start the applications and how long for each application in the simulation. Different use scenarios may choose different application event models in the simulation.

- Poisson model

Poisson model can be used for random application event pattern where there are many users, each generating a little bit of traffic and requesting network access randomly.

Parameters: TBD

- Hyper-exponential model

Hyper-exponential model can be used for peak event pattern where users requesting network access in big spikes from the mean.

Parameters: TBD

- Local file transfer

  - protocol: TCP (Reno)

  - offered load: infinite

  - MSDU sizes: 64 bytes for TCP connection establishment (3-way handshake) and 1500 bytes for payload data.

  - Algorithm: at the start of simulation, generate a TCP connection establishment with the following TCP parameter configuration (as appropriate for the simulation platform):

| TCP Model Parameters |  |
| --- | --- |
| MSS | Ethernet (1500) |
| Receive Buffer (bytes) | 65535 |
| Receive Buffer Adjustment | None |
| Delayed ACK Mechanism | Segment/Clock based |
| Maximum ACK Delay (sec) | 0.05 |
| Slow-Start Initial Count (MSS) | 1 |
| Fast Retransmit | Enabled |
| Duplicate ACK Threshold | 3 |
| Fast Recovery | Reno |
| Window Scaling | Enabled |
| Selective ACK (SACK) | Disabled |
| ECN Capability | Disabled |
| Segment Send Threshold | Byte Boundary |
| Active Connection Threshold | Unlimited |
| Karn's Algorithm | Enabled |
| Nagle Algorithm | Disabled |
| Initial Sequence Number | Auto Complete |
| Initial RTO (sec) | 3.0 |
| Min RTO (sec) | 1.0 |
| Max RTO (sec) | 64.0 |
| RTT Gain | 0.125 |
| Deviation gain | 0.25 |
| RTT Deviation Coefficient | 4.0 |
| Timer Granularity | 0.5 |

- Web browsing

  - Protocol: HTTP (version 1.0 or above)

  - MSDU sizes: 350 bytes for HTTP requests and 1500 bytes for payload data

  - Algorithm: After each reading time the new requests for pages are generated by the user (mean of 31 seconds), generate a HTTP request with the following parameters enlisted below. The parsing time is the time taken by the HTTP page to fill in all subpage requests which appear from the master page. After going through few of the subpages the user quits the session which is indicated by the last packet of the session. This is shown inFigure A2.2.

Reading Time

First Packet of Session

Last Packet of Session

Reading Time

Parsing Time

- HTTP traffic pattern

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| Main object size (SM) | Truncated Lognormal | Mean = 10710 bytes SD = 25032 bytes Min = 100 bytes Max = 2 Mbytes (before truncation) | if x>max or x<min, discard and generate a new value for x |
| Embedded object size (SE) | Truncated Lognormal | Mean = 7758 bytes SD = 126168 bytes Min = 50 bytes Max = 2 Mbytes (before truncation) | f x>max or x<min, discard and generate a new value for x |
| Number of embedded objects per page (Nd) | Truncated Pareto | Mean = 5.64 Max. = 53 (before truncation) | Subtract k from the generated random value to obtain Nd if x>max, discard and regenerate a new value for x |
| Reading time (Dpc) | Exponential | Mean = 30 sec | = 0.033 |
| Parsing time (Tp) | Exponential | Mean = 0.13 sec |  |

- Full buffer traffic model

Unlike wireless display, video streaming is generated from a video server, and traverses multiple hops in the internet before arriving at AP for transmission to STA. It is a unidirectional traffic from the video server to the station.

Typically, Video streaming application runs over TCP/IP protocol, and video frames will be fragmented at TCP layer before leaving the video server. Since these TCP/IP packets experiences different processing and queuing delay at routers, the inter-arrival time between these TCP/IP packets are not a constant despite the fact that video frames are generated at constant interval at the video application layer.

STA Layering Model

STA layering model is shown in Figure A2.2. Both AP and STA generate video frames at application layer. The video traffic goes through TCP/IP layer and then to MAC layer. The TCP protocol used for video streaming simulation is the same as other traffic model.

- Station layer model for buffered video streaming

Video traffic generation

The video traffic from source and receiver is generated as follows.

Step 1: At application layer, generate video frame size (bytes) according to Weibull distribution with the following PDF.

(A2-1)

Depending on the video bit rate, the parameters to use are specified in Table A2.1.

- Lambda and k parameters with different video bit rates for buffered video streaming

| Traffic Model Class Identifier | Video bit rate | λ | k |
| --- | --- | --- | --- |
| BVS1 | 10Mbps | 34750 | 0.8099 |
| BVS2 | 8Mbps | 27800 | 0.8099 |
| BVS3 | 6Mbps | 20850 | 0.8099 |
| BVS4 | 4Mbps | 13900 | 0.8099 |
| BVS6 | 2Mbps | 6950 | 0.8099 |

Step 2: AT TCP layer, set TCP segment as 1500 bytes and fragment video packet into TCP segments.

Step 3: Add network latency to TCP/IP packets when these segments arrive at AP for transmission. The network latency is generated according to Gamma distribution whose PDF is shown below

(A2-2)

Where

  - k=0.2463

  - θ=60.227

The mean of the latency with the above parameters is 14.834ms. To simulate longer or shorter network latency, scaleθlinearly since mean of Gamma distribution is k*θ

If network latency value is such that the packet arrives at MAC layer after the end of the simulation time, then re-generate another network latency value until the packet arrives at MAC within the simulation window.

Evaluation metrics

- MAC throughput, latency

- TCP throughput, latency

- Gaming traffic model

- Algorithm ：Gaming traffic can be modelled by the Largest Extreme Value distribution. The starting time of a network gaming mobile is uniformly distributed between 0 and 20 ms to simulate the random timing relationship between client traffic packet arrival and reverse link frame boundary[1][2][3]

- Parameter ：initial packet arrival time, the packet inter arrival time, and the packet sizes are illustrated

| Component | Distribution |  | Parameters |  | PDF |
| --- | --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |  |
| Initial packet arrival (ms) | Uniform | Uniform | a=0, b=20 | a=0, b=20 |  |
| Packet arrival time (ms) | Largest Extreme Value | Largest Extreme Value | a=15, b=7 | a=23.5, b=10.5 |  |
| Packet size (Byte) | Largest Extreme Value | Largest Extreme Value | a=390, b=89 | a=158, b=26.2 |  |

- Multicast Video Streaming Traffic Model

Multicast Video Streaming is one-way video traffic from AP to multiple STAs

The video traffic is generated from a video server, and traverses multiple hops in the internet before arriving at AP for transmission to STAs.

Station layer model

- Station layer model for multicast video streaming

STA layering model is shown inFigure A2.3. AP generates video frames at application layer.

Because the traffic from AP to stations has experienced network jitter,

it can be modelled the same way as the traffic model of video streaming.

The video traffic goes through UDP/IP layer and then to MAC layer.

Video traffic generation

Traffic model from AP to station: use the same steps in video streaming traffic model.

We assume bit rate for video streaming 6 Mbps (1080/30p AVC) and 3 Mbps (1080/30p HEVC).

Depending on the video bit rate, the parameters to use are specified in Table A2.2.

- Lambda and k parameters with different video bit rates for multicast video streaming

| Traffic Model Class Identifier | Video bit rate | λ | k |
| --- | --- | --- | --- |
| MC1 | 6Mbps | 20850 | 0.8099 |
| MC2 | 3Mbps | 10425 | 0.8099 |

Evaluation metrics

MAC throughput, latency

References for traffic models

- 11-13/486, “HEW video traffic modeling” Guoqing Li et al, (Intel) [1] 11-13-1162-01-hew-vide-categories-and-characteristics

- 11-13-1059-01-hew-video-performance-requirements-and-simulation-parameters

- 11-09-0296-16-00ad-evaluation-methodology.doc

- Rongduo Liu et al., “An Emperical Traffic Model of M2M Mobile Streaming Services ”, International conference C on Multimedia information networking and security, 2012

- JO. Rose, “ Statistical properties of MPEG video traffic and their impact on traffic modeling in ATM systems ”, Tech report, Institute of CS in University of Wurzburg

- Savery Tanwir., “A survey of VBR traffic models”, IEEE communication surveys and tutorials, Jan 2013

- Aggelos Lazaris et al., “A new model for video traffic originating from multiplexed MPEG-4 videoconferencing streams”, International journal on performance evaluation, 2007

- A. Golaup et al., “Modeling of MPEG4 traffic at GOP level using autoregressive process”, IEEE VTC, 2002

- K. Park et al., “Self-Similar network traffic and performance evaluation”, John Wiley&Son, 2000

- M Dai et al., “A unified traffic model for MPEG-4 and H.264 video traces”, IEEE Trans. on multimedia, issue 5 2009.

- L Rezo-Domninggues et al., “Jitter in IP network: A cauchy approach”, IEEE Comm. Letter, Feb 2010

- Hongli Zhang et al., “Modeling Internet link delay based on measurement”, International conference on electronic computer technology, 2009.

- https://mentor.ieee.org/802.11/dcn/13/15-0866-04-00ay-11ay-evaluation-methodology.doc

# References

- https://mentor.ieee.org/802.11/dcn/18/11-18-1236-01-00bb-ieee-802-11bb-reference-channel-models-for-indoor-environments.pdf

- https://mentor.ieee.org/802.11/dcn/18/11-18-1109-05-00bb-lc-usage-model-document.pptx

- https://mentor.ieee.org/802.11/dcn/13/11-13-1000-02-0hew-simulation-scenarios.ppt

- https://mentor.ieee.org/802.11/dcn/13/11-13-1001-09-0hew-simulation-scenarios-document-template.docx