IEEE P802.11
Wireless LANs

| 802.11 REVmd Proposed Resolutions for Editor’s Notes in REVmd D1.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-11-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corporation |  |  | Emily.h.qi@intel.com |
| Edward Au | Huawei |  |  | edward.ks.au@huawei.com |
| Jiamin Chen | Hisilicon |  |  | Jiamin.chen@mail01.huawei.com |
| Shiwen He | SEU |  |  | shiwenhe@seu.edu.cn |

Abstract

This document contains proposed resolutions to Editor’s Notes in REVmd D1.6

EN#1: Page 10.6 Editor’s Note: Figure i will be updated

Discussion: Resolved in CID 1306

Resolution: Remove the note.

EN#2: at 876.26, Editor’s Note: Table 9-45 and Table 9-46 are identical after implementing CID 1440. Could Table 9-45 be removed and update the reference?

Discussion: Are they identical? If yes, why do we need to two tables?

Resolution:

- At 875.31: Change the title of Table 9-45 from “Action Frame Body” to “Action Frame Body and Action No Ack frame body”.

- At 875.59, change “shown in Table 9-46” to “shown in Table 9-45”.

- Remove Table 9-46.

EN#3: at 1414.60 Editor’s Note: Need to review cross reference 9.11

Discussion: at 1418.39, it states:

“NOTE—The value for the Maximum MPDU Length subfield in the CMMG Capabilities Info field imposes aconstraint on the allowed value of the Maximum MPDU Length in the CMMG Capabilities Info field of the CMMG Capabilities element carried in the same frame (see 9.11). “

There is no subclass called 9.11. Cross reference error, “…see 9.11” shall be “…see 10.12 (A-MSDU operation)”.

Resolution:

Change“…(see 9.11).” to “…(see 10.12 (A-MSDU operation).”

EN#4: at 3417.30, Editor’s Note: Clause 10.40 (Dynamic tone pairing) has been deleted since IEEE 802.11-2016 standards.

Discussion: As CDMG (11aj) STA does not support OFDM mode and Clause 10.40  (Dynamic  tone  pairing)  has  been deleted since IEEE 802.11-2016 standards, no need to define parameters DTP_TYPE and DTP_INDICATOR in Table 24-1.

Resolution:

- At 3416.23: Remove the entry DTP_TYPE in Table 24-1.

- At 3416.26: Remove the entry DTP_INDICATOR in Table 24-1.

EN#5: at 3421.35, Editor’s Note: The windowing function in clause 20 has been removed. Do we need 24.3.5.2?

Discussion: As the windowing function in clause 20 has been removed, remove 24.3.5.2.

Resolution:

At 3431.35: Remove 24.3.5.2 (Windowing function）.

EN#6: at 3446.35, Editor’s Note: There is no longer section 20.3.11.11.2.

Discussion: Here 20.3.11.11.2. shall be 19.3.11.11.2 (Spatial mapping).

Resolution:

Change “20.3.11.11.2” to “19.3.11.11.2”.

EN#7: at 3450.8, Editor’s Note: How is Table 22-8 related to this parameter?

Discussion: Here table 22-8 shall be Table 25-3 (Time-related parameters).

Resolution:

Change “Table 22-8” to “Table 25-3”.

EN#8: at 3495.59 and 3497.30, Editor’s Note: There are two definitions of TGI above.

Discussion: One of them need to be removed.

Resolution:

Remove the second definition of TGI.

EN#9: at 3497.60, Editor’s Note: There is no subclause 20.3.12.3.6 as discussed above.

Discussion: Here 20.3.12.3.6 shall be 19.3.12.3.6 (Compressed beamforming feedback matrix).

Resolution:

Change “20.3.12.3.6” to “19.3.12.3.6”.

EN#10: at 3513.58, Editor’s Note: There is no description about how the RX_START_OF_FRAME_OFFSET is used in 6.3.57.

Discussion: Here 6.3.57 (BSS transition management) shall be 6.3.55 (Timing measurement).

Resolution: change “6.3.57” to “6.3.55”

EN#11: at 4335.28 - 41, Editor’s Note: There are empty cells for the column “Global operating class” for operating classes 11, 12, 13, 14, and 15.

Discussion: The cells are empty because no operating classes defined for 11aj channelization in Table E-4 (Global operating classes). Propose to insert operating classes for 11aj channelization in Table E-4 and change Table E-6 accordingly. (e.g., the operating class 10 in table E-1 (Operating classes in the United States) is allocated as operating class 94 in table E-4 (Globe operating classes)).

Resolution:

Change Table E-4 as follows:

| Table E-4—Global Operating classes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating class | Nonglobal operating class(es) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Channel center frequency index | Behavior limits set |
| … | … | … | … | … | … | … |
| 181 | E-6-10 | 56.16 | 2160 | 2,3 | — | — |
| 182 | E-6-11 | 56.70 | 1080 | 35, 36, 37, 38 | — | — |
| 183 | E-6-12 | 42.66 | 540 | 1,2,3,4,5,6,7,8 | — | LicenseExemptBehavior |
| 184 | E-6-13 | 47.52 | 540 | 9,10 | — | LicenseExemptBehavior |
| 185 | E-6-14 | 42.93 | 1080 | 11,12,13,14 | — | LicenseExemptBehavior |
| 186 | E-6-15 | 47.79 | 1080 | 15 | — | LicenseExemptBehavior |
| 181187~191 | — | Reserved | Reserved | Reserved | — | Reserved |
| ... | ... | ... | ... | ... | ... | ... |

Change Table E-6 as follows:

- Insert corresponding “operating classes” numbers in the column “Global operating classes”.

- Fix a number of copy-paste errors in column “channel set” and “Channel center frequency index” when 11aj was rolled in REVmd.)

| Table E-6—Operating classes in China |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating class | Global operating class (See Table E-4 Global operating classes) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Channel center frequency index | Behavior limits set |
| 10 | 181 | 56.16 | 2160 | 21602,3 | 2,3— | — |
| 11 | 182 | 56.70 | 1080 | 108035, 36, 37, 38 | 35, 36, 37, 38— | — |
| 12 | 183 | 42.66 | 540 | 5401,2,3,4,5,6,7,8 | 1,2,3,4, 5,6,7,8— | LicenseExemptBehavior |
| 13 | 184 | 47.52 | 540 | 5409,10 | 9,10— | LicenseExemptBehavior |
| 14 | 185 | 42.93 | 1080 | 54011,12,13,14 | 11,12,13,14— | LicenseExemptBehavior |
| 15 | 186 | 47.79 | 1080 | 108015 | 15— | LicenseExemptBehavior |
| 16~127 | Reserved | Reserved | Reserved | Reserved | — | Reserved |
| ... | ... | ... | ... | ... | ... | ... |