IEEE P802.11
Wireless LANs

| 802.11 REVmd Proposed Resolutions for Editor’s Notes in REVmd D1.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-11-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corporation |  |  | Emily.h.qi@intel.com |
| Edward Au | Huawei |  |  | edward.ks.au@huawei.com |
| Jiamin Chen | Hisilicon |  |  | Jiamin.chen@mail01.huawei.com |
| Shiwen He | SEU |  |  | shiwenhe@seu.edu.cn |

Abstract

This document contains proposed resolutions to Editor’s Notes in REVmd D1.6

R0: initial draft

R1: incorporated the feedback from TGmd teleconference on Nov 1st. changed the resolution for EN#7, EN#8 and EN#11 to “TBD. Need more discussion. Emily will submit a comment.”

EN#1: Page 10.6 Editor’s Note: Figure i will be updated

Discussion: Resolved in CID 1306

Resolution: Remove the note.

EN#2: at 876.26, Editor’s Note: Table 9-45 and Table 9-46 are identical after implementing CID 1440. Could Table 9-45 be removed and update the reference?

Discussion: Are they identical? If yes, why do we need to two tables?

Resolution:

- At 875.31: Change the title of Table 9-45 from “Action Frame Body” to “Action Frame Body and Action No Ack frame body”.

- At 875.59, change “shown in Table 9-46” to “shown in Table 9-45”.

- Remove Table 9-46.

EN#3: at 1414.60 Editor’s Note: Need to review cross reference 9.11

Discussion: at 1418.39, it states:

“NOTE—The value for the Maximum MPDU Length subfield in the CMMG Capabilities Info field imposes a constraint on the allowed value of the Maximum MPDU Length in the CMMG Capabilities Info field of the CMMG Capabilities element carried in the same frame (see 9.11). “

There is no subclass called 9.11. Cross reference error, “…see 9.11” shall be “…see 10.12 (A-MSDU operation)”.

Resolution:

Change“…(see 9.11).” to “…(see 10.12 (A-MSDU operation).”

EN#4: at 3417.30, Editor’s Note: Clause 10.40 (Dynamic tone pairing) has been deleted since IEEE 802.11-2016 standards.

Discussion: As CDMG (11aj) STA does not support OFDM mode and Clause 10.40  (Dynamic  tone  pairing)  has  been deleted since IEEE 802.11-2016 standards, no need to define parameters DTP_TYPE and DTP_INDICATOR in Table 24-1.

Resolution:

- At 3416.23: Remove the entry DTP_TYPE in Table 24-1.

- At 3416.26: Remove the entry DTP_INDICATOR in Table 24-1.

EN#5: at 3421.35, Editor’s Note: The windowing function in clause 20 has been removed. Do we need 24.3.5.2?

Discussion: As the windowing function in clause 20 has been removed, remove 24.3.5.2.

Resolution:

At 3431.35: Remove 24.3.5.2 (Windowing function）.

EN#6: at 3446.35, Editor’s Note: There is no longer section 20.3.11.11.2.

Discussion: Here 20.3.11.11.2. shall be 19.3.11.11.2 (Spatial mapping).

Resolution:

Change “20.3.11.11.2” to “19.3.11.11.2”.

EN#7: at 3450.8, Editor’s Note: How is Table 22-8 related to this parameter?

Discussion: Here table 22-8 shall be Table 25-3 (Time-related parameters).

Resolution: TBD

Change “Table 22-8” to “Table 25-3”.

Need more discussion. – Emily will submit a comment.

EN#8: at 3495.59 and 3497.30, Editor’s Note: There are two definitions of TGI above.

Discussion: One of them need to be removed.

Resolution: TBD

Remove the second definition of TGI.

Need more discussion. – Emily will submit a comment.

EN#9: at 3497.60, Editor’s Note: There is no subclause 20.3.12.3.6 as discussed above.

Discussion: Here 20.3.12.3.6 shall be 19.3.12.3.6 (Compressed beamforming feedback matrix).

Resolution:

Change “20.3.12.3.6” to “19.3.12.3.6”.

EN#10: at 3513.58, Editor’s Note: There is no description about how the RX_START_OF_FRAME_OFFSET is used in 6.3.57.

Discussion: Here 6.3.57 (BSS transition management) shall be 6.3.55 (Timing measurement).

Resolution: change “6.3.57” to “6.3.55”

EN#11: at 4335.28 - 41, Editor’s Note: There are empty cells for the column “Global operating class” for operating classes 11, 12, 13, 14, and 15.

Discussion: The cells are empty because no operating classes defined for 11aj channelization in Table E-4 (Global operating classes). Propose to insert operating classes for 11aj channelization in Table E-4 and change Table E-6 accordingly. (e.g., the operating class 10 in table E-1 (Operating classes in the United States) is allocated as operating class 94 in table E-4 (Globe operating classes)).

Resolution: TBD

Need more discussion. – Emily will submit a comment.

Change Table E-4 as follows:

| Table E-4—Global Operating classes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating class | Nonglobal operating class(es) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Channel center frequency index | Behavior limits set |
| … | … | … | … | … | … | … |
| 181 | E-6-10 | 56.16 | 2160 | 2,3 | — | — |
| 182 | E-6-11 | 56.70 | 1080 | 35, 36, 37, 38 | — | — |
| 183 | E-6-12 | 42.66 | 540 | 1,2,3,4,5,6,7,8 | — | LicenseExemptBehavior |
| 184 | E-6-13 | 47.52 | 540 | 9,10 | — | LicenseExemptBehavior |
| 185 | E-6-14 | 42.93 | 1080 | 11,12,13,14 | — | LicenseExemptBehavior |
| 186 | E-6-15 | 47.79 | 1080 | 15 | — | LicenseExemptBehavior |
| 181187~191 | — | Reserved | Reserved | Reserved | — | Reserved |
| ... | ... | ... | ... | ... | ... | ... |

Change Table E-6 as follows:

- Insert corresponding “operating classes” numbers in the column “Global operating classes”.

- Fix a number of copy-paste errors in column “channel set” and “Channel center frequency index” when 11aj was rolled in REVmd.)

| Table E-6—Operating classes in China |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating class | Global operating class (See Table E-4 Global operating classes) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Channel center frequency index | Behavior limits set |
| 10 | 181 | 56.16 | 2160 | 21602,3 | 2,3— | — |
| 11 | 182 | 56.70 | 1080 | 108035, 36, 37, 38 | 35, 36, 37, 38— | — |
| 12 | 183 | 42.66 | 540 | 5401,2,3,4,5,6,7,8 | 1,2,3,4, 5,6,7,8— | LicenseExemptBehavior |
| 13 | 184 | 47.52 | 540 | 5409,10 | 9,10— | LicenseExemptBehavior |
| 14 | 185 | 42.93 | 1080 | 54011,12,13,14 | 11,12,13,14— | LicenseExemptBehavior |
| 15 | 186 | 47.79 | 1080 | 108015 | 15— | LicenseExemptBehavior |
| 16~127 | Reserved | Reserved | Reserved | Reserved | — | Reserved |
| ... | ... | ... | ... | ... | ... | ... |