IEEE P802.11
Wireless LANs

| CR on HE-SIG-A and HE-SIG-B – Part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-09-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jia Jia | Huawei Technologies | F1-17-A011S, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | justin.jia@huawei.com |
| Ross Jian Yu | Huawei Technologies |  |  | ross.yujian@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on D3.0.

The 8 comments resolved in this document are:

CID 15661, 15918, 15919, 16056, 16277, 16381, 16804, 16806

The changes marked in this document are based on TGax Draft D3.0 (Page number, Line number etc.). Whilst the proposed resolution has reflected the changes in TGax Draft D3.1.

# Revision Notes

| R0.1 | Initial revision |
| --- | --- |

## CID 15661

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 15661 | 466.61 | 28.3.10.7.2 | "STBC is not applied in RUs that are used for MUMIMO allocation." STBC is not applied for the whole PPDU if one RU is with MUMIMO allocation? There is no case where a single user RU has STBC while another MUMIMO RU has non-STBC. | Clarify | Instruction to Editor: REVISED Agree in principle, STBC can not be used whenever one or more RUs are used for MU-MIMO allocation. Please make the changes according to the changes in doc 18-1435r0 |

Instruction to Editor: Please make the following changes in L61, P466, clause 28.3.10.7.2

| Two Parts of HE-SIG-A | Bit | Field | Number of bits | Description |
| --- | --- | --- | --- | --- |
| HE-SIGA2 | B12 | STBC | 1 | In an HE MU PPDU where each RU includes no more than 1 user, set to 1 to indicate all RUs are STBC encoded in the payload, set to 0 to indicate all RUs are not STBC encoded in the payload. STBC does not apply to HE-SIG-B. STBC is not applied in RUs that are used for MUMIMO allocation if one or more RUs are used for MU-MIMO allocation. |

## CID 15918

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 15918 | 461.01 | 28.3.10.7.2 | 'TxBF' is 'Tx BF' everywhere in spec. except in HE-SIG-A table. | Change 'TxBF' to 'Tx BF' in Table 28-18 | Instruction to Editor: REVISED Agree in principle. Please refer to CID 16038 in 18/1123r4 (https://mentor.ieee.org/802.11/dcn/18/11-18-1123-04-00ax-comments-on-tgax-d3-0.xlsx) for the changes. Note to Editor: These changes have been made by CID 16038 (c.f., 18/1123r4) and have been reflected in D3.1 by changing “Tx Beamforming” to Beamformed”. |

## CID 15919

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 15919 | 489.06 | 28.3.10.8.5 | 'Tx Beamforming' field is not used in spec but 'Tx BF'. Replace 'Tx Beamforming' with 'Tx BF' in Table 28-25 (User field format for a non-MU-MIMO Allocation) | As in comment | Instruction to Editor: REVISED Agree in principle. Please refer to CID 16038 in 18/1123r4 (https://mentor.ieee.org/802.11/dcn/18/11-18-1123-04-00ax-comments-on-tgax-d3-0.xlsx) for the changes. Note to Editor: These changes have been made by CID 16038 (c.f., 18/1123r4) and have been reflected in D3.1 by changing “Tx Beamforming” to Beamformed”. |

## CID 16056

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 16056 | 466.38 | 28.3.10.7.2 | ", up to 4," adds nothing of value | Delete the cited text at the referenced location | Instruction to Editor: ACCEPTED |

## CID 16277

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 16277 | 462.16 | 28.3.10.7.2 | "where B1 is the LSB" -- this is the standard order | Delete the cited text in T28-18, T28-19, T28-20 (2x each). Delete the NOTE in T28-18, T28-19, T28-20, T28-23, T28-25, T28-26 | Instruction to Editor: ACCEPTED |

## CID 16381

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 16062 | 464.55 | 28.3.10.7.2 | "Set to 7 for preamble puncturing in 160 MHz or 80+80 MHz, where in the primary 80 MHz of the preamble the primary 40 MHz is present." -- huh? This is not clear. The secondary 40 MHz is entirely missed out, but the secondary 80 MHz is still there? Or the presence/absence of the secondary 40 MHz is undefined (might be present, might be 50% present, might be absent?) Why not the mode where one of the 20 MHz of the secondary 40 MHz is punctured, like in 80 MHz bw? | Change the cited text to "The value 7 is reserved." | Instruction to Editor: REJECTED For 80MHz, if both channels in secondary channel are punctured out, then it goes back to 40MHz. This is different for 160MHz/80+80MHz channel. |

## CID 16804

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 16063 | 461.8 | 28.3.10.7.2 | "indicates a value that is used to determine a limit on the transmit power of a spatial reuse transmission". This is only true for SRP-based SR, which does not apply to SU. | Remove "indicates a value that is used to determine a limit on the transmit power of a spatial reuse transmission" | Instruction to Editor: ACCEPTED |

## CID 16806

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 16086 | 464.22 | 28.3.10.7.2 | "indicates a value that is used to determine a limit on the transmit power of a spatial reuse transmission". This is only true for SRP-based SR, which does not apply to MU. | Remove "indicates a value that is used to determine a limit on the transmit power of a spatial reuse transmission" | Instruction to Editor: ACCEPTED |