IEEE P802.11
Wireless LANs

| LB233 CR on MU Cascading |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-07-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F1-17, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |
| David Xun Yang | Huawei |  |  | david.yangxun@huawei.com |

Abstract

This submission proposes resolutions of comments received from TGax LB233. (The proposed change is based on TGax Draft 3.0)

- CIDs: 16076 16702 16869 (3 CIDs)

Revisions:

- Rev 0: Initial version of the document.

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGax Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGax Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGax Editor: Editing instructions preceded by “TGax Editor” are instructions to the TGax editor to modify existing material in the TGax draft.  As a result of adopting the changes, the TGax editor will execute the instructions rather than copy them to the TGax Draft.

| CID | Page No. | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 16076 | 295.09 | 27.5.4 | "An MU cascading sequence is a frame exchange sequence between an AP and a non-AP STA carried in an HE MU PPDU in the downlink and HE TB PPDU in the uplink and characterized by the exchange of Con- trol, Data and/or Management frames in both directions." is far too vague. The key point is the use of an UL MU trigger combined with acknowledgment | Change the cited text at the referenced location to "An MU cascading sequence is a frame exchange sequence between an AP and a non-AP STA where at least one transmission by the AP includes both an acknowledgment and an UL MU trigger." | Rejected The transmitted MPDU’s Ack policy in the MU cascading sequence is allowed to set to “No Ack”, when all the MPDUs’ Ack policies are set to “No Ack”, there is no solicited acknowledgement. The comment failed to identify this case. |
| 16702 | 295.43 | 27.5.4 | We should probably prevent an AP from sending an A-MPDU with QoS Data frame (or anything) soliciting acknowledgement AND a Trigger frame with TIG Agg Limit > 0 to a STA | Add statement to the effect that AP shall not send an A-MPDU with a frame that solicits acknowledgment together with a Trigger frame that has TID Agg Limit > 0 unless the STA supports cascading | Rejected The comment failed to provide the motivation or the reason to have the proposed restriction. Morover, the setting of TID Aggregation Limit subfield in Trigger frame depends on the the value indicated in the Multi-TID Aggregation Tx Support subfield in the HE Capabilities element. |
| 16869 | 295.09 | 27.5.4 | MU cascading sequence may involve several non AP STAs (as in the example of the figure 27-4). Please update the text accordingly. | Change the text "An MU cascading sequence is a frame exchange sequence between an AP and a non-AP STA" by "An MU cascading sequence is a frame exchange sequence between an AP and non-AP STAs" | Revised Agree with the comment to some extent. However, in the MU cascading sequence, the number of non-AP STAs can be either 1 or more. TGax editor please make the changes as shown in 11-18/1512 r0 |

Discussion: …

TGax editor: please change the subsection 27.5.4 of 11ax draft 3.0:

27.5.4 MU cascading sequence

An MU cascading sequence is a frame exchange sequence between an AP and one or more non-AP STAs (#CID 16869) carried in an HE MU PPDU in the downlink and HE TB PPDU in the uplink and characterized by the exchange of Con-trol, Data and/or Management frames in both directions. An example of an MU cascading sequence is shown in Figure 27-4 (An example of an MU cascading sequence).

……

An AP shall not transmit an A-MPDU to a non-AP STA that includes an Ack or BlockAck frame together with a Trigger frame or a frame carrying a TRS Control subfield unless the non-AP STA has indicated sup-port by setting the MU Cascading Support subfield to 1 in the MAC Capabilities Information field in the HE Capabilities element it transmits. The A-MPDU may additionally contain one or more MPDUs and is con-structed following the rules defined in 27.10 (A-MPDU operation).

An AP shall not transmit an A-MPDU that includes an Ack or BlockAck frame together with a Trigger frame or a frame carrying a TRS Control subfield unless it has set the MU Cascading Support subfield to 1 in the MAC Capabilities Information field in the HE Capabilities element it transmits.

The MU cascading sequence may have a different set of transmitters in HE TB PPDUs as compared to the receivers of the HE MU PPDU that immediately follows the HE TB PPDUs within the same TXOP. The MU cascading sequence may have a different set of receivers in the HE MU PPDU as compared to the set of transmitters of HE TB PPDUs that immediately follow the HE MU PPDU within the same TXOP.

If an AP does not receive an immediate response from a STA addressed by a Trigger frame or frame carry-ing a TRS Control subfield contained in the HE MU PPDU sent at the beginning of the TXOP, then the AP follows the backoff procedure described in 10.22.2.2 (EDCA backoff procedure).