IEEE P802.11
Wireless LANs

| 802.11 LB236 - Proposed Resolutions for EDITOR adhoc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-02-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corporation |  |  | Emily.h.qi@intel.com |

Abstract

This document contains proposed resolutions to LB236 editorial comments for the EDITOR ad hoc that require TG’s feedback.

R00: Initial proposal.

R01: Added more comments for discussion and updated some comments based on the feedback received.

R02: Added Clasue 3 alphabetical order checking from Mark Rison

R03: Incorporated feedback from the TGmd teleconference on 2/1/2019.

R04: Incorporated feedback from the TGmd teleconference on 2/8/2019.

| 7101 | 1706.09 | 11.11.10.3 | "the reporting AP has dot11LciCivicInNeighborReport and the neighbor AP has LCI MeasurementCapability (RM Enabled Capabilities element with the LCI Measurement Capability Enabled fieldset to 1) dot11RMLCIMeasurementActivated equal to true"-- this has at least two errors "has dot11LciCivicInNeighborReport" and"Measurement Capability (...) dot11RMLCIMeasurementActivated equal to true"Note that " (RM Enabled Capabilities element with the LCI Measurement Capability Enabled field set to 1)" is also a informal way of anonymously referencing a transmission by the AP)." this can also be improved. This informality occurs in a number of places in this subclause. The proposed changes addresses two of these. | Change cited text to:"the reporting AP has dot11LciCivicInNeighborReport equal to true and the neighbor AP indicates support for LCI measurement(the neighbor AP has transmitted an RM Enabled Capabilities element with the LCI Measurement Capability Enabled field equal to true)"Make matching changes at 1706.32. |
| --- | --- | --- | --- | --- |

| 2136 | 181.00 | 3.2 | Is 3.2 supposed to be in alphabetical order? Is so why is "generic advertisement service" after "geolocation..."? | check order in 3.2 to make it alphabetical order |
| --- | --- | --- | --- | --- |

Discussion

Is the resolution to fix this specific location or fix all unidentified alphabetical order issues?

If it is to fix this specific location, the proposed resolution is:

Revised. Move the defintion of "generic advertisement service" from 181.49 to 181.18

If it is the later, suggest to assign the comment to the commenter to check all 50+ pages to identify the locations.

See Appendix A for Clasue 3 alphabetical order checking (from Mark Rison).

Emily to do: Bring questions (4) to the editor meeting, .

Proposed Resolution:  TBD

Revised. Move the defintion of "generic advertisement service" from 181.49 to 181.18

| 2431 |  |  | Why QSRC QoS short retry counter but SRC short retry count in abbreviations? Similarly unsolicited retry count/er | Change "retry counter" to "retry count" throughout, case-insensitively and with the space optional |
| --- | --- | --- | --- | --- |

Discussion

Both “count” and “counter” are used in REVmd.

600 instances of count.

582 instances of counter.

20  instances of “retry counter”

15 instances of “retry count”.

As long as they are used consistent with their definitions, there is no need to rename throughout.

However, “unsolicited retry count” and “unsolicited retry counter”

SRC should stand for the same word.

Whatever, accept.

Mark Rison to do: Identify the locations:

Mark identified the following locations in D2.1:

212.44: QLDRC  QoS long drop-eligible retry counter(#1505)

212.50: QSDRC  QoS short drop-eligible retry counter

212.41: QSRC QoS STA retry counter(#1505)

1727.61: attempt to transmit an MPDU causes either STA retry counter to increment, until the CW reaches the value of

1802.14: NOTE 1—In the case of rule e), the STA selects a new random number using the current value of CW[AC], and the retry

counters are not updated

1811: 8 instances

1812: 2 instances

1813.46: NOTE 1—In the case of rule c), the STA selects a new random number using the current value of CW[AC], and the retry

counters are not updated

1815.45: (#1505)When there is a transmission failure within a polled TXOP, the short retry counter (as described in

2248.39: in the series). The short retry counter and long retry counter for the MSDU or A-MSDU are not affected. [note 2x]

2310.29: the retry counters are not updated.

2/5/2019: Mark H. will review it offline and see whether he can agree with the resolution.

Proposed Resolution:

Revised.

In D2.1, Change "retry counter" to "retry count" case-insensitivelyat the following locations:

212.44: QLDRC  QoS long drop-eligible retry counter(#1505)

212.50: QSDRC  QoS short drop-eligible retry counter

212.41: QSRC QoS STA retry counter(#1505)

1727.61: attempt to transmit an MPDU causes either STA retry counter to increment, until the CW reaches the value of

1802.14: NOTE 1—In the case of rule e), the STA selects a new random number using the current value of CW[AC], and the retry counters are not updated

1811: 8 instances

1812: 2 instances

1813.46: NOTE 1—In the case of rule c), the STA selects a new random number using the current value of CW[AC], and the retry counters are not updated

1815.45: (#1505) When there is a transmission failure within a polled TXOP, the short retry counter (as described in

2248.39: in the series). The short retry counter and long retry counter for the MSDU or A-MSDU are not affected. [note 2x]

2310.29: the retry counters are not updated.

| 2473 |  |  | 11md: the whole "field" v. "subfield" thing is just a big inconsistent mess (e.g. in 9.4.2.171 Reduced Neighbor Report element some things in the Neighbor AP Information field are fields and some are subfields, and the TBTT Information Set [sub!]field contains one or more TBTT Information fields) | Change "subfield" to "field" throughout |
| --- | --- | --- | --- | --- |

Discussion

“Subfield” is used to indicate a field in another field. There are 5184 instances of subfield in the draft.

If there is any inconsistent, fix inconsistances.

Recommendation: Reject, or assign to commenter.

Emily to create text files and post to the central.

Mark to run the tool.

Proposed Resolution:

TBD

| 2488 |  |  | "member of an IBSS" should canonically be "IBSS STA". Ditto for MBSS | Change "member of an IBSS" to "IBSS STA" throughout, changing any preceding "a" to "an". Change "member of an MBSS" to "MBSS STA" throughout, changing any preceding "a" to "an" |
| --- | --- | --- | --- | --- |

Discussion

“member of an IBSS” is not same as “IBSS STA”.

There is no definition of “IBSS STA”. I guess it would something like:

IBSS STA: a STA that implements the IBSS facility, if we decide to define it.

“member of an IBSS” is a STA that is already a member of an IBSS, not a STA that is joining an IBSS.

For examples,

At 325.19

a) This parameter is adopted by a STA that is joining an IBSS.

b) This parameter is adopted by a STA that is a member of an IBSS that receives a beacon (11ah)or

S1G beacon(Ed) from a STA that is a member of the same IBSS and that has a timestamp value that

is greater than the local TSF value (see 11.1.5 (Adjusting STA timers)).

also simply replacing “member of an IBSS” with “IBSS STA” won’t read well. For example,

At 1791.25:

sent by a STA that is a member of an IBSS to a STA or STAs that are members of an IBSS

sent by a STA that is an IBSS STA to a STA or STAs that are members of an IBSS

As for “MBSS STA”, there is no existing usage for “MBSS STA”. There might be “mesh STA”.

However, “mesh STA” is not same as “member of an MBSS”.

At 162.60:

mesh station (STA): A quality-of-service (QoS) STA that implements the mesh facility.

“member of an MBSS” is mesh STA that is already a member of an MBSS.

Recommendation: Reject, or assign to the commenter to bring a submission.

Okay with the direction of changing to IBSS STA, and no change for MBSS STA

Assign to the commenter to bring a submission.

IBSS STA definition should be included in the contribution.

Proposed Resolution:

TBD

| 2489 |  |  | "SC" stands for "single-carrier", not "sequence control" or anything else | In Subclause 12 change "the SC field" to "the Sequence Control field", " sequence control field SC)" to " Sequence Control field". In 12.5.3.3 change the first bullet to " The PN is composed of the Sequence Control field and the base PN (BPN), where --- The Sequence Control field is present in the MPDU header --- PN0¦¦PN1 = Sequence Control field with the Fragment Number subfield masked to 0 when the PV1 MPDU is carried in an A-MPDU that is not an S-MPDU --- The base PN is retrieved from the local storage at the receiver --- PN2¦¦PN3¦¦PN4¦¦PN5 = BPN --- PN = PN0¦¦PN1¦¦ PN2¦¦PN3¦¦PN4¦¦PN5 (= Sequence Control ¦¦ BPN)" and below this change "SC¦¦" to "Sequence Control ¦¦ " (3x) |
| --- | --- | --- | --- | --- |

Discussion

Agreed with commenter.

CID1280 has deleted the duplicated definition “SC  sequence counter”.

However "SC" is used in multiple locations for "sequence control".

Proposed Resolution:

Accept.

| 2501 |  |  | It is confusing to have "KDE" stand for "key data encapsulation" now that we have a "Key Delivery element" (which, as a bonus, contains KDEs!) | In 3.2 change "key data encapsulation (KDE): A format for data other than elements in the EAPOL-Key Data field." to "key data cryptographic encapsulation (KDE): A format for data other than elements in the EAPOL-Key Data field." In 12.5.4.4 change "IGTK key data encapsulation (KDE)" to "IGTK key data cryptographic encapsulation (KDE)". Add "The" at the start of the first para of 9.4.2.185. Change the caption for Table 12-7 to "KDE selectors" |
| --- | --- | --- | --- | --- |

Discussion

Agreed with commenter. Since this comment is more than an editorial comment, I would like to the group to review it.

In 3.2 change

"key data encapsulation (KDE): A format for data other than elements in the EAPOL-Key Data field." to "key data cryptographic encapsulation (KDE): A format for data other than elements in the EAPOL-Key Data field."

In 12.5.4.4 change "IGTK key data encapsulation (KDE)" to "IGTK key data cryptographic encapsulation (KDE)".

Add "The" at the start of the first para of 9.4.2.185.

Change the caption for Table 12-7 to "KDE selectors"

Discussion from 2/1/2019: there is no confusion with the Key Delivery element.

Proposed Resolution:

Revised

Add "The" at the start of the first para of 9.4.2.185.

Change the caption for Table 12-7 to "KDE selectors"

At 2619.7 , delete “cryptographic” in the sentence “The additional data may be zero or more element(s) (such as the RSNE) and zero or more key data cryptographic encapsulation(s) (KDEs) (such as GTK(s) or PMKID(s)).

| 2568 |  | 1.4 | The terminology "<blah> frame" to refer to a frame of type Action or Action No Ack where the Action/Category fields indicate <blah> is never spelt out | Add in 1.4 "The construction "<name> frame" is sometimes used to refer to an Action or Action No Ack frame whose Category and Action Details fields indicate <name>." |
| --- | --- | --- | --- | --- |

Discussion

Agreed with commenter.

Need to be revised. Assign to Mark Rison.

Proposed Resolution:

Accept.

| 2570 |  |  | 802.11 is allergic to hyphens (cf. "nonzero") | Delete the hyphen in "multi-band" (case-insensitively) throughout |
| --- | --- | --- | --- | --- |

Discussion

In the Editorial Style Guid, we do see a lot of exception of using Hyphenation. Particularly, hyphenated when before a noun.

Copied from style guide:

---

There are exceptions.   The following are OK:

- non-initial

- non-monotonic

- non-negative

- non-null

- pre-robust

- fixed-length (hyphenated when before a noun)

- follow-up

- signal-to-noise

- STA-to-STA

- third-party

- variable-length (hyphenated when before a noun)

- vendor-specific

---

As “multi-band” is defined as a multi-band operation reference model to different physical layers, I think it is okay to use a hyphen. See example above.

Note that there are 570 instances in the draft.  A lot of instances in figures.  Changing it requires a lot of editing effort, but no gain.

Recommendation: Reject

Reason: In the Editorial Style Guid, we do see a lot of exception of using Hyphenation. Particularly, hyphenated when before a noun.  “multi-band” is defined as a multi-band operation reference model to different physical layers, and hyphenated when before a noun. It can be an exception of using Hyphenation.

Option 1: reject comment  6

Option 2: not reject comment.  3

A: 3

The group decided to keep using “multi-band”.

Proposed Resolution:

Revised.

At 3813.50 change “multiband” to “multi-band”

Note to Editor: add “multi-band” to the exception list in the editorial style guide.

Note to the commenter: The TG discussed the options of eliminating the hyphen and keeping it. By straw poll the group preferred to reject proposed changes (6-3-3). The text is changed to have consistent usage. The editorial work to remove hyphens in 570 locations was not viewed as worth the benefit.

| 2677 | 150.00 | 2 | The draft includes 14 references to IEEE Std 802-2014, including one (as IEEE Std 802(R)-2014) in the Normative References [which, incidentally, has a typo in the title]. There is at least one generic (undated) reference to IEEE Std 802. The references should be generic. IEEE Std 802 has been amended twice since 2014 and may be amended or revised in the future. IEEE Std 802.11 must remain consistent with IEEE Std 802. | In the Normative References, change "IEEE Std 802(R)-2014 IEEE Standards" to "IEEE Std 802(R) IEEE Standard". In 13 other places, change "IEEE Std 802(R)-2014" to "IEEE Std 802(R)". |
| --- | --- | --- | --- | --- |

Discussion

I got 16 instances in D2.0. Two instanances are specific to clause 8 of IEEE Std 802(R)-2014 (772.63 and 785.61)

Proposed Resolution:

Revised.

At 151.11, change "IEEE Std 802(R)-2014 IEEE Standards" to "IEEE Std 802(R) IEEE Standard".

In other places (excluding instanances at 772.63, 785.61, 2518.26), change "IEEE Std 802-2014" to "IEEE Std 802".

================================================

| 2658 | 155.00 | 3.1 | The definitions in clause 3.1 don't read the way they are used in the spec, because all acronyms have to be expanded on first use. This complicates searching and proofreading. | Move 3.4 (Abbreviations and acronyms) to before 3.1 (Definitions) so that the definitions may read the way they are used in the spec text. Or add a dummy sentence before 3.1 which includes all acronyms used in 3.1. Or, send a request to the IEEE editor for the definitions section to be exempted from the rule that acronyms have to be expanded on first use. |
| --- | --- | --- | --- | --- |

Discussion

Clause 3.1 definitions will be included in IEEE Standards Dictionary Online.  The definition in 3.1 shall follow IEEE Standard Disctionary guide.

Proposed Resolution:

Reject.

Reason: Clause 3.1 definition will be included in IEEE Standards Dictionary Online.  The definition in 3.1 shall follow IEEE Standard Disctionary guide.

================================================

| 2544 | 176.00 | 3.2 | "China millimeter-wave multi-gigabit (CMMG) duplicate physical layer (PHY) protocol data unit (PPDU) format: The data transmission (TXVECTOR parameter CH_BANDWIDTH equal to CBW1080) duplicates the transmission of a 540 MHz signal over every 540 MHz frequency segment.(11aj)" is not a canonical form of a definition, which should be "A PPDU format that [...]" | Change to "China millimeter-wave multi-gigabit (CMMG) duplicate physical layer (PHY) protocol data unit (PPDU) format: A PPDU format that duplicates the transmission of a 540 MHz signal over two 540 MHz frequency segments." |
| --- | --- | --- | --- | --- |

Discussion

Change

"China  millimeter-wave  multi-gigabit  (CMMG)  duplicate  physical  layer  (PHY)  protocol  data  unit
(PPDU) format: The data transmission (TXVECTOR parameter CH_BANDWIDTH equal to CBW1080)
duplicates the transmission of a 540 MHz signal over every 540 MHz frequency segment.(11aj)" to

"China  millimeter-wave  multi-gigabit  (CMMG)  duplicate  physical  layer  (PHY)  protocol  data  unit (PPDU) format: A PPDU format that duplicates the the transmission of a 540 MHz signal over two 540 MHz frequency segments."

Proposed Resolution:

Accept

================================================

| 2137 | 182.00 | 3.2 | GTKSA is in 3.2 182.10 and in 3.4 208.42. I don't see GTK in both, for example. Should GTKSA be in both? What is the rule? | Delete GTKSA from 3.2 |
| --- | --- | --- | --- | --- |

Discussion

There is definition for PTK, PTKSA, GTKSA in 3.2

Suggest adding a GTK definition.

group temporal key (GTK): a group key that is used to protect group addressed Data frames.

Proposed Resolution:

Revised.

At 182.8, add a definition as follows:

“group temporal key (GTK): a group key that is used protect group addressed Data frames.”

================================================

| 2255 | 327.00 | 6.3.3.3.2 | First occurrence of RSNE is in table(s) in clause 6 (at P327.26). Do such uses have the "spell it out the first time" rule? Or, should that apply to the first 'real' use at P460.36? This applies to MME, FFE, FTE, MDE, PWE, RDE and TIE as well. | As in comment. |
| --- | --- | --- | --- | --- |

Discussion

If the abbreviation is defined in Clause 3, you don’t need to spell it out in the rest of the document.

However, we sometimes do spell it out the first time it appears after Clause 3. And sometimes the first time it is used in each major clause.

Anyway, the general rule is spell it out first time it is used (which is often Clause 3) and then just use the abbreviation.

Proposed Resolution:

Reject.

Reason: the general rule is spell it out first time it is used (which is often Clause 3) and then just use the abbreviation.

================================================

| 2119 | 584.00 | 6.3.71.4.2 | "PeerSTAAddress" shall be "STAAddress" to be consistent with MLME-GAS.indication. In MLME-GAS.request and MLME-GAS.confirm primitives, the first parameter is "STAAddress". In MLME-GAS.indication primitive, the parameter is named as "PeerSTAAddress". To be consistent, "PeerSTAAddress" shall be "STAAddress" in MLME-GAS.indication. | In 6.3.71.4.2, change "PeerSTAAddress" to "STAAddress" at 584.13 and 584.29. |
| --- | --- | --- | --- | --- |

Discussion

Agree with commenter.

11aq changed “PeerSTAAddress” to “STAAddress” for in MLME-GAS.request and MLME-GAS.confirm primitives.

Should be consistent in the same set of primitives.

Proposed Resolution:

Accept.

================================================

| 2337 | 791.00 | 9.2.4.5.4 | "Where the frame does not contain a fragment, or either the originator or the addressed recipient does not support the fragment BA procedure." is spurious, given the following "Otherwise:" | Delete "Where the frame does not contain a fragment, or either the originator or the addressed recipient does not support the fragment BA procedure." at the referenced location |
| --- | --- | --- | --- | --- |

Discussion

Agree with commenter.

Proposed Resolution:

Accept.

================================================

| 2583 | 966.00 | 9.4.2.1 | It is confusing for "Fragmentable" information to be sometimes explicitly "No" and sometimes only implicitly not yes | In Table 9-94 make the Fragmentable cell "No" where empty and where not for a reserved element ID (e.g. Vendor Specific). Also make it empty for a reserved element ID (e.g. 4, 178-180) |
| --- | --- | --- | --- | --- |
| 2584 | 966.00 | 9.4.2.1 | It is confusing for "Extensible" information to be sometimes explicitly "No" and sometimes only implicitly not yes | In Table 9-94 and the "subelement ids for" tables make the Extensible cell empty where "No" (e.g. Vendor Specific) |
| 2585 | 966.00 | 9.4.2.1 | It is confusing for "Extensible" information to be sometimes explicitly "No" and sometimes only implicitly not yes | In Table 9-94 and the "subelement ids for" tables make the Extensible cell "No" where empty and not for a reserved element ID (e.g. . Also make it empty for a reserved element ID (e.g. 178-180) |

Discussion

CID 2583: The instruction is clear. The changes are for Table 9-94.  However, we need to make sure the empty is “No”.

CID 2584: It seems to cover Table 9-94 and all other 42 tables (Optional subelement IDs for XXX”

CID 2585: It seems to cover Table 9-94 and all other 42 tables (Optional subelement IDs for XXX”

CID 2584 and 2585 have the same comment but provide different resolutions. We need to exam each of those tables.

I think that these three comments need a submition, and it should be reviewed as a technical comment.

Proposed Resolution:

================================================

| 2006 | 1435.00 | 9.4.2.237 | The name "Active BSSID Count' is too specific and restrictive. The element was recently defined in REVmd and is extensible. It is very likely to be used by future ammendments for providing more information about a multiple BSSID set. Rename the element to something more generic. | Rename the element to 'Multiple BSSID Configuration element'. Replace all occurrence with the new name. |
| --- | --- | --- | --- | --- |

Discussion

Any issue to change the name?

It is okay to me.

Proposed Resolution:

Accept.

================================================

| 2633 | 1626.00 | 9.6.20.7 | It is confusing to have "OCT MMPDU" being both "an MMPDU that was constructed by a different STA of the same device" and a field in the On-channel Tunnel Request frame | Change the name of the field to "OCT MMPDU descriptor" |
| --- | --- | --- | --- | --- |

Discussion

Any issue to change the name?

It is okay to me.

Proposed Resolution:

Revised.

At 1626.10, 1626.19, and 1626.26: change “OCT MMPDU” to “OCT MMPDU Descriptor”

================================================

| 2020 | 1778.00 | 9.5.6 | The word "slave" to describe a device which does not set its own configurations has become increasingly contentious in technical communities. It should be changed, if for no other reason than to avoid PR problems. Happily for the 802.11 community, this appears to be the only mention of the word "slave" in the entire Tgmd document, and there is no definition of what a "slave" could be in section 3. This should be fairly trivial to resolve. | Replace the word "slave" or the words "is a slave of" with something like "client" (analogous with TVWS in B.4.26, perhaps) or "worker", "recipient", "is passive with respect to", "secondary". |
| --- | --- | --- | --- | --- |

Discussion

The location is 1478.38:

The BeamLink isMaster subfield is set to 1 to indicate that the STA is the master of the data transfer and set

to 0 if the STA is a slave of the data transfer. The STAs use the BeamLink isMaster subfield to negotiate the

dot11BeamLinkMaintenanceTime as specified in Table 9-343 (The Beamformed Link Maintenance

negotiation).

Okay to change to “client”.

Proposed Resolution:

Revised.

At 1478.38: Change “STA is a slave of the data transfer” to “STA is a client of the data transfer”

| 2650 |  |  | Equation (T-39) is corrupted: the thing on the left looks like a valid sigma, but the squared things in the square root are some other glyph (a box with "F073" inside) | Change all proprietary/Adobe Unicode codepoints to the equivalent non-proprietary Unicode codepoints. This may be as simple as not using the Symbol font |
| --- | --- | --- | --- | --- |
| 2649 |  |  | Equation (T-39) is corrupted: the thing on the left looks like a valid sigma, but the squared things in the square root are some other glyph (a box with "F073" inside) | Change all proprietary/Adobe Unicode codepoints in the document to "OOPS" throughout |

Discussion

I couldn’t see what the commenter sees. Need another pair of eyes.

Commenter stated: It might depend on your PDF reader.  Copy one of the two rightmost sigmas and paste into some other application.  In Firefox I get the F073 inside a box.  In Word I get a ? in a box.  In either case it’s not the right character (a lowercase sigma).

I tried both Adobe Acrobat Reader and Foxit reader, and I cannot identify the problem.  I also copied it to the word and outlook emai, I couldn’t duplicate this problem.

Recommendation: Reject.

Reason: There is no issue in the published PDF file. This issue may occur when someone copies and pastes in a word doc, or to other format. However, Editor cannot duplicate the issue.

Proposed Resolution:

TBD

================================================

| 2660 | 148.00 | 1.4 | What is the meaning of "between" in the standard, is it inclusive or exclusive of the boundary values? | Add a definition of "between" in clause 1.4 (Word usage). |
| --- | --- | --- | --- | --- |

Discussion

A submission is required.

Proposed Resolution:

TBD

================================================

| 2587 |  | 9 | "Indicates support for <x>" should be "Indicates whether <x> is supported" in Table 9-184---Subfields of the HT Capability Information field, Table 9-187---Subfields of the HT Extended Capabilities field, Table 9-272---Subfields of the VHT Capabilities Information field, Table 9-301---Subfields of the S1G Capabilities Information field, Table 9-314---Subfields of the CMMG Capabilities Info field | As it says in the comment. Also restore the table borders in Table 9-301 |
| --- | --- | --- | --- | --- |

Discussion

I don’t see a need for these changes. The current usage creates no confusion.  If sentence’s grammar is wrong, fix the specific sentence.

Reject, or assign it to the commenter for submission required.

Proposed Resolution:

TBD

================================================

| 2599 |  |  | Sometimes ASCII apostrophes are used, sometimes a Unicode right quote | Change all ASCII apostrophes to Unicode right quotes, except those in a monospaced font |
| --- | --- | --- | --- | --- |

Discussion

Need a specific location to help understanding the comment.

Proposed Resolution:

TBD

================================================

| 2647 |  |  | Some of the ResultCodes in the SAP have underscores (e.g. JOIN_FAILURE_TIMEOUT, NOT_SUPPORTED) some not (e.g. AUTH FAILURE TIMEOUT, ANTI-CLOGGING TOKEN REQUIRED). There is no justification for this inconsistency. Underscores make it easier to find result codes, so are preferable | Change "AUTH FAILURE TIMEOUT" to "AUTH FAILURE_TIMEOUT" throughout the document |
| --- | --- | --- | --- | --- |

Discussion

It is a similar comment as CID 25, CID 277 and CID 1433. See:

| 1433 |  |  | Some of the ResultCodes in the SAP have underscores (e.g. JOIN_FAILURE_TIMEOUT, NOT_SUPPORTED) some not (e.g. AUTH FAILURE TIMEOUT, ANTI-CLOGGING TOKEN REQUIRED) | Change "JOIN_FAILURE_TIMEOUT" to "JOIN FAILURE TIMEOUT" throughout the document |
| --- | --- | --- | --- | --- |

These comments are rejected.

Reject Reason: There is no rule on whether ResultCode should include underscores or not. Uderscores are used for ResultCode everywhere. No need to remove them. The current usage create no confusion.

Recommendation: Reject

Reject Reason: There is no rule on whether ResultCode should include underscores or not. No need to change them. The current usage creates no confusion.

Proposed Resolution:

Reject

Reject Reason: There is no rule on whether ResultCode should include underscores or not. No need to change them. The current usage creates no confusion.

Appendix A: Clause 3 alphabetical order check from Mark Rison

After saving the 3.1 definitions to a text file, I used the following script:

$ cat c31defs.txt | grep -a -v ^NOTE | cut -f1 -d: | sed -e 's/(#[0-9]*)//g' -e 's/-//g' >/tmp/md$$ ; sort -f </tmp/md$$ | diff -u1 /tmp/md$$ -

This gave the following (- is existing, + is correct alphabetical position, ignoring hyphens):

@@ -14,4 +15,4 @@

association

-authentication

authenticated encryption with associated data (AEAD)

+authentication

Authentication Server (AS)

@@ -78,4 +79,4 @@

frame

-frequency segment

frame exchange sequence

+frequency segment

Gaussian frequency shift keying (GFSK)

@@ -125,4 +126,2 @@

mobility domain identifier (MDID)

-multilevel precedence and preemption (MLPP)

-multiuser multiple input, multiple output (MUMIMO)

multicast

@@ -130,2 +129,3 @@

multicastgroup address

+multilevel precedence and preemption (MLPP)

multiple basic service set identifier (BSSID) capability

@@ -133,2 +133,3 @@

multiple medium access control (MAC) station management entity (SME) (MMSME)

+multiuser multiple input, multiple output (MUMIMO)

neighbor access point (AP)

@@ -158,4 +159,4 @@

peertopeer traffic specification (PTP TSPEC)

-perframe encryption key

perfect forward secrecy (PFS)

+perframe encryption key

physical layer (PHY) frame

@@ -193,5 +194,5 @@

roaming consortium

+service hash

service set transition

serving access point (AP)

-service hash

singleuser (SU) physical layer (PHY) protocol data unit (PPDU)

3.2 needed additional faffery to get rid of lists:

$ cat c32defs.txt | grep -a -v ^NOTE | grep -a -v '^[a-z])' | cut -f1 -d: | sed -e 's/(#[0-9]*)//g' -e 's/(11a.)//g' -e 's/-//g' >/tmp/md$$ ; sort -f </tmp/md$$ | diff -u1 /tmp/md$$ -

to give (I suppose there could be debate as to whether "2" should be

before "10"):

@@ -2,10 +2,8 @@

1 MHz physical layer (PHY) protocol data unit (PPDU)

-2 MHz mask physical layer (PHY) protocol data unit (PPDU)

-2 MHz physical layer (PHY) protocol data unit (PPDU)

-4 MHz mask physical layer (PHY) protocol data unit (PPDU)

-4 MHz physical layer (PHY) protocol data unit (PPDU)

-8 MHz mask physical layer (PHY) protocol data unit (PPDU)

-8 MHz physical layer (PHY) protocol data unit (PPDU)

16 MHz mask physical layer (PHY) protocol data unit (PPDU)

16 MHz physical layer (PHY) protocol data unit (PPDU)

+160 MHz mask physical layer (PHY) protocol data unit (PPDU)

+160 MHz physical layer (PHY) protocol data unit (PPDU)

+2 MHz mask physical layer (PHY) protocol data unit (PPDU)

+2 MHz physical layer (PHY) protocol data unit (PPDU)

20 MHz basic service set (BSS)

@@ -15,2 +13,7 @@

20/40 MHz basic service set (BSS)

+4 MHz mask physical layer (PHY) protocol data unit (PPDU)

+4 MHz physical layer (PHY) protocol data unit (PPDU)

+40 MHz highthroughput (HT)

+40 MHz mask physical layer (PHY) protocol data unit (PPDU)

+40 MHz physical layer (PHY) protocol data unit (PPDU)

40MHzcapable (40MC) highthroughput (HT) access point (AP)

@@ -21,13 +24,9 @@

40MHzcapable (40MC) highthroughput (HT) station (STA) 5G

-40 MHz highthroughput (HT)

-40 MHz mask physical layer (PHY) protocol data unit (PPDU)

-40 MHz physical layer (PHY) protocol data unit (PPDU)

4way handshake

+8 MHz mask physical layer (PHY) protocol data unit (PPDU)

+8 MHz physical layer (PHY) protocol data unit (PPDU)

80 MHz mask physical layer (PHY) protocol data unit (PPDU)

80 MHz physical layer (PHY) protocol data unit (PPDU)

-160 MHz mask physical layer (PHY) protocol data unit (PPDU)

-160 MHz physical layer (PHY) protocol data unit (PPDU)

80+80 MHz mask physical layer (PHY) protocol data unit (PPDU)

80+80 MHz physical layer (PHY) protocol data unit (PPDU)

-attached bridge

access category (AC)

@@ -41,2 +40,3 @@

aggregated schedule

+attached bridge

authentication and key management (AKM) suite

@@ -54,4 +54,4 @@

bufferable unit (BU)

-centralized authentication controller (CAC) access point (AP)

centralized authentication controlled (CAC) station (STA)

+centralized authentication controller (CAC) access point (AP)

centralized coordination service root (CCSR)

@@ -59,10 +59,10 @@

China directional multigigabit (CDMG)

-China directional multigigabit small band beacon interval (CDMG SBBI)

China directional multigigabit (CDMG) physical layer (PHY) protocol data unit (PPDU)

+China directional multigigabit small band beacon interval (CDMG SBBI)

China millimeterwave multigigabit (CMMG)

China millimeterwave multigigabit (CMMG) basic service set (BSS)

-China millimeterwave multigigabit (CMMG) duplicate physical layer (PHY) protocol data unit (PPDU) format

-China millimeterwave multigigabit (CMMG) physical layer (PHY) protocol data unit (PPDU)

China millimeterwave multigigabit (CMMG) beamformee

China millimeterwave multigigabit (CMMG) beamformer

+China millimeterwave multigigabit (CMMG) duplicate physical layer (PHY) protocol data unit (PPDU) format

+China millimeterwave multigigabit (CMMG) physical layer (PHY) protocol data unit (PPDU)

China millimeterwave multigigabit (CMMG) single medium access control (MAC) protocol data unit (CMMG single MPDU)

@@ -74,7 +74,7 @@

deep sleep mode

-deliveryenabled access category (AC)

delivery traffic indication map (DTIM) interval

+deliveryenabled access category (AC)

destination directional multigigabit (DMG) station (STA)

-direct sequence spread spectrum/complementary code keying (DSSS/CCK)

direct sequence spread spectrum (DSSS) physical layer (PHY) protocol data unit (PPDU)

+direct sequence spread spectrum/complementary code keying (DSSS/CCK)

directional multigigabit (DMG)

@@ -95,3 +95,2 @@

EAPOLStart frame

-Extensible Authentication Protocol (EAP) reauthentication protocol (EAPRP)

emergency services association

@@ -109,2 +108,3 @@

extended service set (ESS) link

+Extensible Authentication Protocol (EAP) reauthentication protocol (EAPRP)

fast basic service set (BSS) transition (FT) 4way handshake

@@ -113,6 +113,6 @@

fast initial link setup (FILS)

-fast initial link setup category (FILSC)

-fast initial link setup station (FILS STA)

fast initial link setup (FILS) association

fast initial link setup (FILS) authentication

+fast initial link setup category (FILSC)

+fast initial link setup station (FILS STA)

fine timing measurement (FTM) procedure

@@ -126,4 +126,5 @@

general link (GLK) convergence function

-general link groupcast with retries (GLKGCR)

general link (GLK) station (STA)

+general link groupcast with retries (GLKGCR)

+generic advertisement service (GAS)

geolocation database (GDB)

@@ -136,3 +137,2 @@

geolocation database dependent (GDD) nonaccess point (nonAP) station (STA)

-generic advertisement service (GAS)

global operating class

@@ -196,5 +196,5 @@

modulation and coding scheme (MCS)

-modulation and coding scheme 32 (MCS 32) format

modulation and coding scheme (MCS) feedback (MFB) requester

modulation and coding scheme (MCS) feedback (MFB) responder

+modulation and coding scheme 32 (MCS 32) format

multiple basic service set identifier (BSSID) set

@@ -243,2 +243,3 @@

pairwise master key (PMK)

+pairwise master key (PMK) R0 (PMKR0)

pairwise master key (PMK) R0 key holder (R0KH)

@@ -246,3 +247,3 @@

pairwise master key (PMK) R0 name (PMKR0Name)

-pairwise master key (PMK) R0 (PMKR0)

+pairwise master key (PMK) R1 (PMKR1)

pairwise master key (PMK) R1 key holder (R1KH)

@@ -250,3 +251,2 @@

pairwise master key (PMK) R1 name (PMKR1Name)

-pairwise master key (PMK) R1 (PMKR1)

pairwise master key (PMK) S0 key holder (S0KH)

@@ -269,10 +269,10 @@

power save (PS) mode

+power save (PS) station (STA)

power save multipoll (PSMP)

power save multipoll (PSMP) burst

-power save multipoll downlink transmission time (PSMPDTT)

power save multipoll (PSMP) sequence

power save multipoll (PSMP) session

+power save multipoll downlink transmission time (PSMPDTT)

power save multipoll uplink transmission time (PSMPUTT)

power save multipoll uplink transmission time (PSMPUTT) spacing

-power save (PS) station (STA)

prerobust security network association (preRSNA)

@@ -281,11 +281,12 @@

primary 2 MHz channel

-primary 4 MHz channel

-primary 8 MHz channel

primary 20 MHz channel

+primary 4 MHz channel

primary 40 MHz channel

+primary 8 MHz channel

primary 80 MHz channel

primary access category (AC)

+Protected Dual of Public Action frame

protocol version 0 (PV0) medium access control (MAC) protocol data unit (MPDU)

protocol version 1 (PV1) medium access control (MAC) protocol data unit (MPDU)

-Protected Dual of Public Action frame

+qualityofservice (QoS) frame

qualityofservice management frame (QMF)

@@ -295,3 +296,2 @@

qualityofservice management frame (QMF) station (STA)

-qualityofservice (QoS) frame

quasiomni antenna pattern

@@ -305,2 +305,3 @@

robust Management frame

+robust security network (RSN)

robust security network association (RSNA)

@@ -308,3 +309,2 @@

robust security network association (RSNA) key management

-robust security network (RSN)

S1G band

@@ -313,6 +313,6 @@

secondary 2 MHz channel

-secondary 4 MHz channel

-secondary 8 MHz channel

secondary 20 MHz channel

+secondary 4 MHz channel

secondary 40 MHz channel

+secondary 8 MHz channel

secondary 80 MHz channel

@@ -338,3 +338,2 @@

spatial sharing (SPSH)

-synthetic receiver address (SYNRA)

staggered preamble

@@ -353,6 +352,8 @@

synchronization access point (AP) (SAP)

-synchronizing access point (AP) or personal basic service set (PBSS) control point (PCP)

-synchronized access point (AP) or personal basic service set (PBSS) control point (PCP)

synchronization personal basic service set (PBSS) control point (PCP) (SPCP)

+synchronized access point (AP) or personal basic service set (PBSS) control point (PCP)

+synchronizing access point (AP) or personal basic service set (PBSS) control point (PCP)

+synthetic receiver address (SYNRA)

target wake time (TWT)

+target wake time (TWT) requester

target wake time (TWT) responder

@@ -360,3 +361,3 @@

target wake time (TWT) service period (SP) start time

-target wake time (TWT) requester

+television very high throughput (TVHT) basic service set (BSS)

television very high throughput 2W (TVHT_2W)

@@ -364,3 +365,2 @@

television very high throughput 4W (TVHT_4W)

-television very high throughput (TVHT) basic service set (BSS)

television very high throughput W (TVHT_W)

@@ -413,5 +413,5 @@

very high throughput (VHT) physical layer (PHY) protocol data unit (PPDU)

+very high throughput (VHT) singleuser (SU) physical layer (PHY) protocol data unit (PPDU)

very high throughput (VHT) singleuseronly (SUonly) beamformee

very high throughput (VHT) singleuseronly (SUonly) beamformer

-very high throughput (VHT) singleuser (SU) physical layer (PHY) protocol data unit (PPDU)

white space map (WSM)

Or maybe you think stuff in parens should be ignored?  Then:

$ cat c32defs.txt | grep -a -v ^NOTE | grep -a -v '^[a-z])' | cut -f1 -d: | sed -e 's/(#[0-9]*)//g' -e 's/([^)]*) *//g' -e 's/-//g' >/tmp/md$$ ; sort -f </tmp/md$$ | diff -u1 /tmp/md$$ -

gives:

@@ -2,10 +2,8 @@

1 MHz physical layer protocol data unit

-2 MHz mask physical layer protocol data unit

-2 MHz physical layer protocol data unit

-4 MHz mask physical layer protocol data unit

-4 MHz physical layer protocol data unit

-8 MHz mask physical layer protocol data unit

-8 MHz physical layer protocol data unit

16 MHz mask physical layer protocol data unit

16 MHz physical layer protocol data unit

+160 MHz mask physical layer protocol data unit

+160 MHz physical layer protocol data unit

+2 MHz mask physical layer protocol data unit

+2 MHz physical layer protocol data unit

20 MHz basic service set

@@ -15,2 +13,7 @@

20/40 MHz basic service set

+4 MHz mask physical layer protocol data unit

+4 MHz physical layer protocol data unit

+40 MHz highthroughput

+40 MHz mask physical layer protocol data unit

+40 MHz physical layer protocol data unit

40MHzcapable highthroughput access point

@@ -21,13 +24,9 @@

40MHzcapable highthroughput station 5G

-40 MHz highthroughput

-40 MHz mask physical layer protocol data unit

-40 MHz physical layer protocol data unit

4way handshake

+8 MHz mask physical layer protocol data unit

+8 MHz physical layer protocol data unit

80 MHz mask physical layer protocol data unit

80 MHz physical layer protocol data unit

-160 MHz mask physical layer protocol data unit

-160 MHz physical layer protocol data unit

80+80 MHz mask physical layer protocol data unit

80+80 MHz physical layer protocol data unit

-attached bridge

access category

@@ -41,2 +40,3 @@

aggregated schedule

+attached bridge

authentication and key management suite

@@ -54,4 +54,4 @@

bufferable unit

-centralized authentication controller access point

centralized authentication controlled station

+centralized authentication controller access point

centralized coordination service root

@@ -59,10 +59,10 @@

China directional multigigabit

-China directional multigigabit small band beacon interval

China directional multigigabit physical layer protocol data unit

+China directional multigigabit small band beacon interval

China millimeterwave multigigabit

China millimeterwave multigigabit basic service set

-China millimeterwave multigigabit duplicate physical layer protocol data unit f

ormat

-China millimeterwave multigigabit physical layer protocol data unit

China millimeterwave multigigabit beamformee

China millimeterwave multigigabit beamformer

+China millimeterwave multigigabit duplicate physical layer protocol data unit f

ormat

+China millimeterwave multigigabit physical layer protocol data unit

China millimeterwave multigigabit single medium access control protocol data un

it

@@ -74,7 +74,7 @@

deep sleep mode

-deliveryenabled access category

delivery traffic indication map interval

+deliveryenabled access category

destination directional multigigabit station

-direct sequence spread spectrum/complementary code keying

direct sequence spread spectrum physical layer protocol data unit

+direct sequence spread spectrum/complementary code keying

directional multigigabit

@@ -95,3 +95,2 @@

EAPOLStart frame

-Extensible Authentication Protocol reauthentication protocol

emergency services association

@@ -109,2 +108,3 @@

extended service set link

+Extensible Authentication Protocol reauthentication protocol

fast basic service set transition 4way handshake

@@ -113,6 +113,6 @@

fast initial link setup

-fast initial link setup category

-fast initial link setup station

fast initial link setup association

fast initial link setup authentication

+fast initial link setup category

+fast initial link setup station

fine timing measurement procedure

@@ -128,2 +128,3 @@

general link station

+generic advertisement service

geolocation database

@@ -136,3 +137,2 @@

geolocation database dependent nonaccess point station

-generic advertisement service

global operating class

@@ -243,2 +243,3 @@

pairwise master key

+pairwise master key R0

pairwise master key R0 key holder

@@ -246,3 +247,3 @@

pairwise master key R0 name

-pairwise master key R0

+pairwise master key R1

pairwise master key R1 key holder

@@ -250,3 +251,2 @@

pairwise master key R1 name

-pairwise master key R1

pairwise master key S0 key holder

@@ -281,11 +281,12 @@

primary 2 MHz channel

-primary 4 MHz channel

-primary 8 MHz channel

primary 20 MHz channel

+primary 4 MHz channel

primary 40 MHz channel

+primary 8 MHz channel

primary 80 MHz channel

primary access category

+Protected Dual of Public Action frame

protocol version 0 medium access control protocol data unit

protocol version 1 medium access control protocol data unit

-Protected Dual of Public Action frame

+qualityofservice frame

qualityofservice management frame

@@ -295,3 +296,2 @@

qualityofservice management frame station

-qualityofservice frame

quasiomni antenna pattern

@@ -305,2 +305,3 @@

robust Management frame

+robust security network

robust security network association

@@ -308,3 +309,2 @@

robust security network association key management

-robust security network

S1G band

@@ -313,6 +313,6 @@

secondary 2 MHz channel

-secondary 4 MHz channel

-secondary 8 MHz channel

secondary 20 MHz channel

+secondary 4 MHz channel

secondary 40 MHz channel

+secondary 8 MHz channel

secondary 80 MHz channel

@@ -338,3 +338,2 @@

spatial sharing

-synthetic receiver address

staggered preamble

@@ -343,3 +342,2 @@

station 5G

-sub 1 GHz physical layer protocol data unit

sub 1 GHz 1M physical layer protocol data unit

@@ -347,2 +345,3 @@

sub 1 GHz modulation and coding scheme

+sub 1 GHz physical layer protocol data unit

sub 1 GHz short physical layer protocol data unit

@@ -353,6 +352,8 @@

synchronization access point

-synchronizing access point or personal basic service set control point

-synchronized access point or personal basic service set control point

synchronization personal basic service set control point

+synchronized access point or personal basic service set control point

+synchronizing access point or personal basic service set control point

+synthetic receiver address

target wake time

+target wake time requester

target wake time responder

@@ -360,3 +361,2 @@

target wake time service period start time

-target wake time requester

television very high throughput 2W

@@ -413,5 +413,5 @@

very high throughput physical layer protocol data unit

+very high throughput singleuser physical layer protocol data unit

very high throughput singleuseronly beamformee

very high throughput singleuseronly beamformer

-very high throughput singleuser physical layer protocol data unit

white space map

Please tell me the sorting rules, and I'll try to make my script do the

right thing.

As for 3.4, it's a bit easier:

$ cat c34defs.txt | sed 's/-//g' | cut -f1 >/tmp/md$$ ; sort -f </tmp/md$$ | diff -u1 /tmp/md$$ -

giving:

@@ -102,5 +103,5 @@

DA

+DBC

DBI

DBPSK

-DBC

DCF

@@ -234,2 +235,3 @@

HWMP SN

+I/Q

IBSS

@@ -248,3 +250,2 @@

IQMF

-I/Q

ISM

@@ -297,4 +298,4 @@

MIDC

-MIS

MIMO

+MIS

MLME

@@ -309,3 +310,2 @@

MRQ

-MSSAP

MSB

@@ -315,2 +315,3 @@

MSK

+MSSAP

MTK

@@ -329,4 +330,4 @@

OCB

-OCI

OCEF

+OCI

OCT

@@ -449,2 +450,3 @@

SA

+SA Query

SAE

@@ -453,3 +455,2 @@

SAPSD

-SA Query

SBBI