IEEE P802.11
Wireless LANs

| 802.11 REVmd LB236 comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | BlackBerry Ltd | 4701 Tahoe Blvd, Mississauga, ON. CANADA. L4W 0B4 | +1-289-261-4183 | mmontemurro@blackberry.com |

Abstract

This document contains some proposed resolutions to REVmd LB236 comments.

# Comments

### CID 2542

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2542 |  | 25 |  |  | A non-directional multi-gigabit PHY was not seen as necessary/practicable in the 60G band, so doesn't seem necessary/practicable in the 55G band | Delete Clause 25 |

#### Discussion:

- This clause was added on the roll-in of IEEE 802.11aj-2018.

- With respect to feasibility, here is the justification that was given in the approved 5C submission (https://mentor.ieee.org/802.11/dcn/12/11-12-0141-07-cmmw-ieee-802-11-cmww-sg-5c.doc):

“The 60 GHz has been referenced for quite some time as a technology used for > 1 Gbps communication.  802.11 is a mature technology which has a wide variety of legacy devices and a proven track record, with hundreds of millions of devices shipping each year. The silicon prototype demonstration based on 802.11ad is available in the market.

The 45 GHz has been referenced as a technology used for > 1 Gbps backhaul communication. It is a similar technology to 60 GHz.”

- The task group reached consensus was that a non-directional PHY was necessary/practical.

- Given that the standard was published less than a year ago, it’s premature to conclude whether the non-directional PHY in these bands are necessary/practical.

#### Proposed Resolution:

REJECTED. The IEEE 802.11 working group consensus in creating Clause 25 was that a non-directional, multi-gigabit PHY was both practical and necessary. Given that the standard was published less than a year ago, it’s premature to conclude whether the technology is necessary/practical.

### CID 2643

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2643 |  | 12.3.3.3 |  |  | WEP is obsolete and has not been maintained (comments on it in previous ballots were rejected on the basis it was obsolete and was going to be deleted), so implementations based on the current wording are likely to be erroneous | Delete the referenced subclause |

#### Discussion:

- Removal of WEP has been considered in past ballot rounds:

- CC25 – CID 63

| Comment | Proposed Resolution | Resolution |
| --- | --- | --- |
| Time to remove all pre-RSNA security mechanisms other than Open System authentication? WEP | Remove | REJECTED (MAC: 2017-11-07 20:09:31Z): There are known implementations of these features in the market, so we choose not to remove them at this time. The Group did not come to consensus on removal of these two features. |

- LB232 – CID 1006

| Comment | Proposed Resolution | Resolution |
| --- | --- | --- |
| "Except for Open System authentication, all pre-RSNA security mechanisms are obsolete." This was considered on D0.1 CID 63 and rejected as "Known implementations.... in the market". 17/1504 details deletion changes. It was also noted that it should be re-considered in January 2018 (i.e. D1.0). Security is IMPORTANT and discouraging WEP and TKIP is IMPORTANT. Implementations using WEP are compliant to 802.11 - 2012 and maybe to 2016 so the reference and specification is available even though we are not keeping WEP and TKIP up to date by addressing comments for them. Latest developments in WFA on WPA3 are clear, DO NOT TEST WEP or TKIP and fail devices that allow WEP and TKIP. If vendors wish to support outdated security, that's fine, they can't be tested anyhow. Let's get rid of WEP and TKIP in our standard - why keep poor security in our Std. that is irresponsible. | Incorporate changes as described in 17/1504. Note Page and Line numbers will need to be checked to comply with D1.0 | REJECTED. “The task group discussed removal of WEP and/or TKIP from the standard and decided to not change the standard based on strawpolls on the direction for the resolution. The strawpolls were held during the Warsaw meeting (2018-05-08) and the option to keep WEP and TKIP text as-is received most support. See https://mentor.ieee.org/802.11/dcn/18/11-18-0616-00-000m-minutes-revmd-may-2018-warsaw.docx. |

- Removal of shared-key authentication has been considered in past ballot rounds. See LB 232 – CID 1323

| Comment | Proposed Resolution | Resolution |
| --- | --- | --- |
| It is confusing to have something called "FILS Shared Key authentication", as it sounds as if this is a special case of "Shared Key authentication" (a.k.a. WEP) | Delete Subclause 12.3.3.3 | REJECTED (PHY: 2018-07-11 23:28:38Z) - With a resolution of “The term "FILS Shared Key" is unambiguous. The commenter proposes to remove WEP. The task group discussed removal of WEP and/or TKIP from the standard and decided to not change the standard based on strawpolls on the direction for the resolution. The strawpolls were held during the Warsaw meeting(2018-05-08) and the option to keep WEP and TKIP text as-is received most support. See https://mentor.ieee.org/802.11/dcn/18/11-18-0616-00-000m-minutes-revmd-may-2018-warsaw.docx . |

- This comment is a duplicate of a previously resolved comment. See LB232 – CID 1410

| Comment | Proposed Resolution | Resolution |
| --- | --- | --- |
| WEP is obsolete and has not been maintained (comments on it in previous ballots were rejected on the basis it was obsolete and was going to be deleted), so implementations based on the current wording are likely to be erroneous | Delete Subclause 12.3.3.3 | REJECTED. “The task group discussed removal of WEP and/or TKIP from the standard and decided to not change the standard based on strawpolls on the direction for the resolution. The strawpolls were held during the Warsaw meeting (2018-05-08) and the option to keep WEP and TKIP text as-is received most support. See https://mentor.ieee.org/802.11/dcn/18/11-18-0616-00-000m-minutes-revmd-may-2018-warsaw.docx . |

#### Proposed Resolution:

REJECTED. This comment is a duplicate of LB 232 – CID 1410. No further justification for a technical change has been given. The task group discussed removal of WEP and/or TKIP from the standard and decided to not change the standard based on strawpolls on    the direction for the resolution. The strawpolls were held during the Warsaw meeting (2018-05-08) and the option to keep WEP and TKIP text as-is received most support. See https://mentor.ieee.org/802.11/dcn/18/11-18-0616-00-000m-minutes-revmd-may-2018-warsaw.docx

### CID 2662

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2662 | 4352.00 | Annex G |  |  | Annex G adds little value and a lot of overhead. And its contents should be a repeat of normative requirements elsewhere in other sections. | Delete Annex G and its references. |

#### Discussion:

- Similar comments have come up in CC25 and LB232:

- CC25 – comment 72:

| Comment | Proposed Resolution | Resolution |
| --- | --- | --- |
| I note that Annex G is Normative. Does anyone actually refer to this? Checking it against the main text I would say is impossible. Does it serve any practical purpose? What would be the effect of removing it? All I see in the text are generality statements "see Annex G" never do I see a specific place in Annex G which is 15 pages long! | Consider removing Annex G or form a task group to check all the hieroglyphics contained therein as to their accuracy and compliance with the main text. Then make the references in the main text specific to a place in the 15 pages of Annex G. If no-one volunteers, back to deleting? | REJECTED (MAC: 2017-12-07 17:49:06Z): A Normative definition of Frame exchange sequence is required and Annex G provides this Definition. |

- LB232 – comment 1277

| Comment | Proposed Resolution | Resolution |
| --- | --- | --- |
| The definition of "frame exchange sequence" which simply states that they are specified by Annex G, does not seem adequate. | Provide a definition for "frame exchange sequence". e.g. frame exchange sequence: a sequence of frames which are exchanged between STAs to change the state, configuration, or status of the STAs participating in the exchange. The state, configuration, or status of the participating STAs does not change until the frame exchange sequence is complete. Frame exchange sequences are specified in Annex G. | REJECTED (GEN: 2018-04-12 21:08:02Z) The descriptin in Annex G is sufficient, and the only purpose of Annex G is to give this definition. |

- The task group has discussed this comment multiple times and the comment does not provide any additional justification.

#### Proposed Resolution:

REJECTED: A Normative definition of Frame exchange sequence is required, and Annex G provides this definition.

### CID 2053

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2053 | 3532.00 | Annex A |  |  | Bibliography items: B1, B4, B5, B8, B10, B12, B17, B19, B36, B38, B43, B45, B52, B54, B57, B60 are orphaned: there is no reference to them outside the bibligography | Consider removing these items. |

#### Discussion:

- Summary of cross-references:

  - B1 – Referenced in Annex R, clause 5.1 (4562.28) – 3GPP reference to latest spec is: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=799

  - B4 – 3GPP2 X.S0060-0 IMS emergency sessions architecture: http://www.3gpp2.org/Public_html/specs/X.S0060-0_v1.0_080729.pdf.  No cross-reference.

  - B5 – ANSI Z136.1-1993, American National Standard for the Safe Use of Lasers. No cross-reference

  - B8 – ARIB STD-T71 (5.0), Broadband Mobile Access Communication System (CSMA), ARIB, Dec. 2007. No cross-reference.

  - B10 – Code of Federal Regulations, Title 47, Telecommunication, Part 90, Private Land Mobile Radio Services, Section 90.210(m), Emission masks. Referenced in E2.2 (4344.7)

  - B12 – Engwer, D., and Zweig, J., “Algorithmically Derived Hop Sequences,” submission 99/195 to the IEEE P802.11 Working Group, Sept. 1999. No cross-reference.

  - B17 – GSMA, IR.34 v4.6, Inter-Service provider IP Backbone Guidelines, http://gsmworld.com/documents/IR3446.pdf, Apr. 2009. Annex R 3.1 (4555.9) – should be [B17], not [B16]

  - B19 –IEC 60825-1:1993, Safety of laser products—Part 1: Equipment classification, requirements and user’s guide. No cross-reference.

  - B36 – [B36] IETF RFC 2898, PKCS #5: Password-Based Cryptography Specification Version 2.0. RFC 2898 is cited in J2.4.1. (4465.7)

  - B38 – [B38] IETF RFC 3222, Terminology for Forwarding Information Base (FIB) based Router Performance. No cross-reference.

  - B43 – IETF RFC 3693, Geopriv Requirements, Feb. 2004. No cross-reference

  - B45 – IETF RFC 4493, The AES-CMAC Algorithm. RFC is referenced in clause 12.7.2 (2616.22)

  - B52 – ITU Radio Regulations, volumes 1–4. No cross-reference.

  - B54 –ITU-T Recommendation V.41 (11/88), Code-independent error-control system. No cross-reference.

  - B57 – Maric, S. V., and Titlebaum, E. L., “A Class of Frequency Hop Codes with Nearly Ideal Characteristics for Use in Multiple-Access Spread-Spectrum Communications and Radar and Sonar Systems,” IEEE Transactions on Communications , vol. 40, no. 9, pp. 1442–1447, Sept. 1992. No cross-reference.

  - B60 – Tarkoma, S., Rothenberg, C. E., and Lagerspetz, E., “Theory and Practice of Bloom Filters for Distributed Systems,” IEEE Communications Surveys and Tutorials , vol. 14, no. 1, pp. 131–155, Feb. 2011. No cross reference.

  - Some cross-references exist from this list. Others reference the document in the bibliography.

  - The bloom filter reference is missed in the text introducing bloom filter.

#### Proposed Resolution:

REVISED. With respect to D2.0,

Remove the following references from the Bibliography: [B4], [B5], [B8], [B12], [B19], [B38], [B43], [B52], [B54], and [B57].

Replace the bibliography entry for [B1] with:

“[B1] 3GPP TS 23.167, IMS emergency sessions architecture: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=799.”

At 4434.37, change “Part 90 Subpart Z” to “Part 90 Subpart Z [B10]”

At 4555.9, change “IR.34 v4.6 [B16]” to “IR.34 v4.6 [B17]”

At 2616.22, change “IETF RFC 4493” to “IETF RFC 4493 [B45]”

At 4465.7, change “IETF RFC 2898” to “IETF RFC 2898 [B36]”

At 1435.60, change “stochastic characteristics of a Bloom filter” to “stochastic characteristics of a Bloom filter [B60]”

### CID 2194

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2194 | 3537.00 | B.2 |  |  | WS is defined as wakeup schedule in most of the specification, but in B.2 it is defined as white spaces 3217.39 and is then is not used in B only TVWS is used and hence the definition in B.2 for WS should be deleted. WS also shows up in the MIB, where I believe it is used as an example and means work station. WS is also used in E 3983.30 where it means white spaces. Also it would be useful to add WS into the 3.4: WS wakeup schedule. | Delete: "WS white spaces" And in E.2.5, page 4347 line 42, replace "WS" with "White Space" |

#### Discussion:

- In 9.4.2.130, The acronym WS is introduced (1284.23): “The DMG Wakeup Schedule element is used to communicate the wakeup schedule (WS) of DMG STAs.”

- Adding the WS acronym to clause 3 would also be a good idea.

#### Proposed Resolution:

ACCEPTED. In addition to the Proposed Change, add “WS wakeup schedule” to clause 3.4.

### CID 2199

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2199 | 2658.00 | 12.9.2 |  |  | There has been number of attempts to fix issues in the RSNA frame pseudocode subclauses over the years, but this has never really succeeded in getting sufficient contributions and review. Consequently, the current state of clause 12.9 is not exactly ideal and it is difficult to change anything in frame encryption without making these subclauses even less in sync with rest of the standard. Taken into account the inaccuracies in the current pseudocode for TKIP/CCMP/GCMP and BIP processing, it is questionable whether these pseudocode descriptions are of any real value to implementers. In fact, they may result in incorrect implementation. Based on lack of progress in cleaning this up in the past, it does not look likely that this would get sufficient work done within the REVmd schedule. Since the related requirements on the STAs should be covered elsewhere in the standard, the time to remove these pseudocode subclauses may have arrived. If there is anything within 12.9 and its subclauses that is not covered elsewhere in the standard, such contents should be added in the other security subclauses to keep it more maintainable. | Delete 12.9.2 and all its subclauses (page 2658 line 42 - page 2669 line 21). In 5.1.2 (page 298 line 45), replace "the decision tree for CCMP, GCMP, and BIP defined in 12.9 is driven by MIB attributes" with "the decision tree for CCMP, GCMP, and BIP is driven by MIB attributes". In B.4.4.1 (page 3553 lines 24-39) PC34, delete References column items 12.9.2, 12.9.2.2, 12.9.2.4, 12.9.2.6, 12.9.2.8. In B.4.1.1 (page 3557 lines 43-56) PC34.1.10, delete References column items 12.9.2.3, 12.9.2.5, 12.9.2.7, 12.9.2.9. |

#### Discussion:

- Clause 12.9.2 is not normative and could be removed.

- Changes in 5.1.2:

“The MAC sublayer security services provided by CCMP, GCMP, and BIP rely on information from non

MAC sublayer management or system entities. Management entities communicate information to CCMP,

GCMP, and BIP through a set of MAC sublayer management entity (MLME) interfaces and MIB attributes;

in particular, the decision tree for CCMP, GCMP, and BIP defined in 12.9 (Per-frame pseudocode) is driven

by MIB attributes.”

- The changes to the PICS look correct.

- No changes are proposed to 2583.54, but since TKIP is no longer maintained, there should be no changes to that sub-clause.

#### Proposed Resolution:

ACCEPTED.

### CID 2295

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2295 | 3778.00 | C.3 |  |  | dot11AuthenticationResponseTimeout is deprecated, but it doesn't say why (by convention, it should). There is no obvious reason why this is deprecated. | Change "Deprecated" to "current" |

#### Discussion:

- 802.11 Authentication frames are used for more than just WEP protocols. For instance, they are used for Open, SAE, FT.

- dot11AuthenticationResponseTimeout behavior is described for FILS, see 2678.34:

“If the STA was attempting EAP-RP Authentication and did not successfully receive an Authentication frame within the time of dot11AuthenticationResponseTimeout, then the STA should perform retransmission procedure as defined in IETF RFC 6696. If the retransmission procedure fails, then the STA shall abandon the FILS authentication and should perform full EAP authentication via IEEE 802.1X authentication.”

- dot11AuthenticationResponseTimeout behavior should be described for SAE and FT. It be associated with the transaction sequence number in authentication frames (for instance, it could be related to a transaction sequence number greater than 1).

- Bottom line is that this parameter cannot be deprecated.

#### Proposed Resolution:

ACCEPTED

### CID 2341

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2341 | 4110.00 | C.3 |  |  | dot11RTSThreshold is limited to 65536. This is a threshold on PSDU size. This in turn means that it is not possible to not enable RTS/CTS for medium-size or bigger A-MPDUs | Change 65536 to 6500631 (looking forward to HE) |

#### Discussion:

- Mib specification text:

dot11FragmentationThreshold OBJECT-TYPE

SYNTAX Unsigned32 (256..65535)

UNITS "octets"

MAX-ACCESS read-write

- The maximum MPDU size is 4692480

- Not sure about HE (?) but it likely makes sense to align the threshold with the maximum MPDU size.

#### Proposed Resolution:

REVISED. Change 65536 to 4692480.

### CID 2352

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2352 | 2564.00 | 12.5.3.4.3 |  |  | "The AAD does not include the header Duration field, because the Duration field value might change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission). The AAD includes neither the Duration/ID field nor the HT Control field because the contents of these fields might change during normal operation (e.g., due to a rate change preceding retransmission)." -- duplication | Delete the first of the cited sentences at the referenced location |

#### Discussion:

- Text at the cited location:

“The AAD is constructed from the MPDU header. The AAD does not include the header Duration field, because the Duration field value might change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission). The AAD includes neither the Duration/ID field nor the HT Control field because the contents of these fields might change during normal operation (e.g., due to a rate change preceding retransmission). The HT Control field might also be inserted or removed during normal operation (e.g., retransmission of an A MPDU where the original A MPDU included an MRQ that has already generated a response). For similar reasons, several subfields in the Frame Control field are masked to 0.  For PV0 MPDUs, the AAD construction is performed as follows.”

#### Proposed Resolution:

ACCEPTED.

### CID 2353

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2353 | 2571.00 | 12.5.4.3 |  |  | "The Duration field in the AAD shall be masked to 0." - there is no Duration field in the AAD, so this statement is meaningless | Delete the cited text at the referenced location |

#### Discussion:

- The context of the text is as follows

The BIP Additional Authentication Data (AAD) shall be constructed from the MPDU header. The Duration

field in the AAD shall be masked to 0. The AAD construction shall use a copy of the IEEE 802.11 header

without the SC field for the MPDU, with the following exceptions:

a) FC—MPDU Frame Control field, with:

1) Retry subfield (bit 11) masked to 0

2) Power Management subfield (bit 12) masked to 0

3) More Data subfield (bit 13) masked to 0

b) A1—MPDU Address 1 field.

c) A2—MPDU Address 2 field.

d) A3—MPDU Address 3 field.

- The normative statement about the duration field should be included but could be worded more accurately.

- It would be better to include Duration in the letter list.

- Accept the editing instruction for the change but add the text at the beginning of the bulleted list.

#### Proposed Resolution:

REVISED. Delete the cited text as the referenced location.

Replace:

“with the following exceptions:

a) FC—MPDU Frame Control field, with:”

with

“with the following exceptions:

a) Duration – MPDU duration field masked to 0

b) FC—MPDU Frame Control field, with:

### CID 2354

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2354 | 2571.00 | 12.5.4.3 |  |  | "The AAD construction shall use a copy of the IEEE 802.11 header without the SC field for the MPDU" -- no, it doesn't, it uses the fields shown in Figure 12-25---BIP AAD Construction | Change the cited text at the referenced location to "The AAD construction shall use fields copied from the MAC header, with the following modifications:" |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2403

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2403 |  | C.3 |  |  | In a few places in the MIB, ^ is used for exponentiation. Everywhere else it's for XOR | Change ^ to ** throughout C.3 |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2408

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2408 | 2593.00 | 12.6.10.2 |  |  | This is the only "EAP-Packet" in the whole document. 802.1X-2010 states that "The EAPOL-EAP Packet Type was referred to as the EAP-Packet Packet Type in previous revisions of this standard." | Change the cited text at the referenced location to "EAPOL-EAP" |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2493

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2493 |  | 12 |  |  | The resolution to CID 1365 failed to fix the two Figures referenced (and F12-47 also needs fixing in fact and F13-3 in 13.4.2 etc.) | In Figure 12-47 change "Anonce, 0, 0, 0" to "0, Anonce, 0, {}", ", RSNE" to ", { RSNE }", "MIC, 0" to "MIC, {}" and put curly brackets around the last three arguments of the third EAPOL-Key frame. In Figure 12-48 put curly brackets around the last two arguments of the first EAPOL-Key frame and make the fifth argument of each of the EAPOL-Key frames be "G" not "0" and change "0)" to "{} )". Put curly brackets around the last 4 arguments of StaProcessEAPOL-Key in 12.7.9.4. In 12.7.9.4 change "RSNE)" to "{RSNE})". In Figure 12-50 change "0)" to "{})" in the first EAPOL-Key argument list and put curly brackets around the last 3 arguments of the second EAPOL-Key argument list. In Figure 12-52 put curly brackets around the last argument of the EAPOL-Key argument list. In Figure 13-2 insert ", {}" as the last argument of the first and last EAPOL-Keys; ", 0" as the sixth argument of all the EAPOL-Keys; put curly brackets around the arguments of the second and third EAPOL-Keys from the "RSNE" (included) to the closing paren (excluded). In Figure 13-15 change "ANonce, 0)" to "0, Anonce, 0, {})" in the first EAPOL-Key argument list and put curly brackets around the last 3 arguments and insert "0," as the sixth argument of the second EAPOL-Key argument list. In Figure 13-18 put curly brackets around the last 3 arguments and insert "0," as the sixth argument of the first EAPOL-Key argument list and change "MIC-KCK" to "0, MIC-KCK, {}" in the second EAPOL-Key argument list |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2512

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2512 | 2587.00 | 12.6.3 |  |  | 12.6.3 RSNA policy selection in an infrastructure BSS has lots of variation on whether it's just an AP/STA or an RSNA capable AP/STA or an RSNA-enabled AP/STA etc. | Insert a new para at the start of the referenced subclause add "The requirements in this subclause apply to all RSNA enabled APs/STAs." then delete "RSNA capable" and "RSNA-enabled" and "RSNA enabled" throughout |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2522

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2522 |  | C.3 |  |  | The MIB has some variables that are "It is written by the SME or external management entity.", some that are "It is written by an external management entity." and some that are "It is written by the SME." This implies that the SME is not considered an "external management entity", but this is not clear | At the end of 4.9.2 add a "NOTE---The SME is not considered an external management entity." |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2523

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2523 |  | C.3 |  |  | The MIB has some variables that are "It is written by the STA". It is not clear what this means, regarding whether the SME is part of the STA, i.e. whether if a MIB variable is set by the STA, that includes being set by the STA's SME, or whether necessarily means the MLME, MAC or lower layers | Change all instances of "It is written by the STA" in C.3 to "It is written by the MAC" |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2541

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2541 | 2625.00 | 12.7.6.1 |  |  | The "{blah} or {boo}" notation is not clear and not all args (e.g. PMKID) are mentioned in the "Here," below | At the end of the list in the referenced subclause add "- PMKID represents the appropriate PMKID" and "- "{a} or {b}" means that exactly one of {a} and {b} is present" |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2552

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2552 |  | 12.12.2.1 |  |  | "To prevent key reinstallation attacks, the non-AP STA shall maintain a copy of the most recent GTK and most recent IGTK installed as part of the FILS authentication protocol as if they were installed as a result of receipt of EAPOL-Key frames" -- but the requirements in 6.3.19 only apply to keys installed as a result of receipt of EAPOL-Key frames, those installed "as if". Same issue in 13.5.1 | In 6.3.19.1.4 change "installed as a result of receipt of EAPOL-Key frames" to "installed as a result of, or as if as a result of, receipt of EAPOL-Key frames" |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2626

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2626 |  | 12 |  |  | "Hash is" wording is inconsistent ("Hash is the hash algorithm identified by the AKM suite selector (see Table 9-151)", "Hash is the hash algorithm identified by the negotiated AKM suite selector specified in Table 9-151", "Hash is the AKM-specific hash algorithm", "Hash is the hash algorithm(#307) specific to the negotiated AKM") | Change each of the instances of the text cited in the parenthesis in the comment to "Hash is the hash algorithm specific to the negotiated AKM (see Table 9-151)" |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2669

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2669 | 2518.00 | 12.2.10 |  |  | The requirements on a STA behavior described in the cited paragraph could be improved. The paragraph should clearly indicate that a STA needs to determine what MAC address to use when it selects a BSS for Association. | Commenter will provide a submission to update the cited text. |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2051, 2670

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2051 | 2906.00 | 17.3.5.5 |  |  | "If dot11MACPrivacyActivated is true, the initial state of the scrambler shall be reset when the STA's MAC address is changed." - There are a few problems here: 1) How does the PHY know that the MAC address have chagned. 2) If it is supposed to be pseudo-random, what does resetting it mean? 3) sometimes the scrambler initialization conveys information - how does that reconcile with resetting? | See submission 11-18-2165 |
| 2670 | 2518.00 | 12.2.10 |  |  | The reference to changing scrambler settings is PHY dependent. The text needs to be update to indicate so. | Commenter will provide a submission to update the cited text. |

#### Discussion:

- []

#### Proposed Resolution:

[]

### CID 2722

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2722 | 2691.00 | 12.6.22.1 |  |  | It doesn't make a sense, in the last sentence of 12.6.22.1, to reference 12.6.22, which leads right back to 12.6.22.1. Since the sentence states that separates RSNA is needed for cases other than the transparent FST, it would make a sense to reference 12.6.22.2 (Nontransparent multi-band RSNA), instead. | Change "see 12.6.22 (Multi-band RSNA)" to "see 12.6.22.2 (Nontransparent multi-band RSNA)". |

#### Discussion:

- []

#### Proposed Resolution:

[]