IEEE P802.11
Wireless LANs

| 802.11 REVmd LB236 comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-04-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | BlackBerry Ltd | 4701 Tahoe Blvd, Mississauga, ON. CANADA. L4W 0B4 | +1-289-261-4183 | mmontemurro@blackberry.com |

Abstract

This document contains some proposed resolutions to REVmd LB236 comments.

# Comments

### CID 2053

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2053 | 3532.00 | Annex A |  |  | Bibliography items: B1, B4, B5, B8, B10, B12, B17, B19, B36, B38, B43, B45, B52, B54, B57, B60 are orphaned: there is no reference to them outside the bibligography | Consider removing these items. |

#### Discussion:

- Summary of cross-references:

  - B1 – Referenced in Annex R, clause 5.1 (4562.28) – 3GPP reference to latest spec is: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=799

  - B4 – 3GPP2 X.S0060-0 IMS emergency sessions architecture: http://www.3gpp2.org/Public_html/specs/X.S0060-0_v1.0_080729.pdf.  No cross-reference.

  - B5 – ANSI Z136.1-1993, American National Standard for the Safe Use of Lasers. No cross-reference

  - B8 – ARIB STD-T71 (5.0), Broadband Mobile Access Communication System (CSMA), ARIB, Dec. 2007. No cross-reference.

  - B10 – Code of Federal Regulations, Title 47, Telecommunication, Part 90, Private Land Mobile Radio Services, Section 90.210(m), Emission masks. Referenced in E2.2 (4344.7)

  - B12 – Engwer, D., and Zweig, J., “Algorithmically Derived Hop Sequences,” submission 99/195 to the IEEE P802.11 Working Group, Sept. 1999. No cross-reference.

  - B17 – GSMA, IR.34 v4.6, Inter-Service provider IP Backbone Guidelines, http://gsmworld.com/documents/IR3446.pdf, Apr. 2009. Annex R 3.1 (4555.9) – should be [B17], not [B16]

  - B19 –IEC 60825-1:1993, Safety of laser products—Part 1: Equipment classification, requirements and user’s guide. No cross-reference.

  - B36 – [B36] IETF RFC 2898, PKCS #5: Password-Based Cryptography Specification Version 2.0. RFC 2898 is cited in J2.4.1. (4465.7)

  - B38 – [B38] IETF RFC 3222, Terminology for Forwarding Information Base (FIB) based Router Performance. No cross-reference.

  - B43 – IETF RFC 3693, Geopriv Requirements, Feb. 2004. No cross-reference

  - B45 – IETF RFC 4493, The AES-CMAC Algorithm. RFC is referenced in clause 12.7.2 (2616.22)

  - B52 – ITU Radio Regulations, volumes 1–4. No cross-reference.

  - B54 –ITU-T Recommendation V.41 (11/88), Code-independent error-control system. No cross-reference.

  - B57 – Maric, S. V., and Titlebaum, E. L., “A Class of Frequency Hop Codes with Nearly Ideal Characteristics for Use in Multiple-Access Spread-Spectrum Communications and Radar and Sonar Systems,” IEEE Transactions on Communications , vol. 40, no. 9, pp. 1442–1447, Sept. 1992. No cross-reference.

  - B60 – Tarkoma, S., Rothenberg, C. E., and Lagerspetz, E., “Theory and Practice of Bloom Filters for Distributed Systems,” IEEE Communications Surveys and Tutorials , vol. 14, no. 1, pp. 131–155, Feb. 2011. No cross reference.

  - Some cross-references exist from this list. Others reference the document in the bibliography.

  - The bloom filter reference is missed in the text introducing bloom filter.

  - [B21] – The cross-reference was incorrectly labeled as [B22] at 922.22.

  - [B43] – The Geopriv requirements are not referenced, but the Geopriv RFC (RFC 4119) is.

#### Proposed Resolution:

REVISED (PHY: 2019-04-03 00:41:49Z)

With respect to D2.0,

Remove the following references from the Bibliography: [B4], [B5], [B8], [B12], [B19], [B38], [B43], [B47], [B52], [B54], and [B57].

Replace the bibliography entry for [B1] with:

“[B1] 3GPP TS 23.167, IMS emergency sessions architecture: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=799.”

At 4434.37, change “Part 90 Subpart Z” to “Part 90 Subpart Z [B10]”

At 4555.9, change “IR.34 v4.6 [B16]” to “IR.34 v4.6 [B17]”

At 2616.22, change “IETF RFC 4493” to “IETF RFC 4493 [B45]”

At 4465.7, change “IETF RFC 2898” to “IETF RFC 2898 [B36]”

At 1435.60, change “stochastic characteristics of a Bloom filter” to “stochastic characteristics of a Bloom filter [B60]”

At 922.22, change [B22] to [B21]

### CID 2194

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2194 | 3537.00 | B.2 |  |  | WS is defined as wakeup schedule in most of the specification, but in B.2 it is defined as white spaces 3217.39 and is then is not used in B only TVWS is used and hence the definition in B.2 for WS should be deleted. WS also shows up in the MIB, where I believe it is used as an example and means work station. WS is also used in E 3983.30 where it means white spaces. Also it would be useful to add WS into the 3.4: WS wakeup schedule. | Delete: "WS white spaces" And in E.2.5, page 4347 line 42, replace "WS" with "White Space" |

#### Discussion:

- In 9.4.2.130, The acronym WS is introduced (1284.23): “The DMG Wakeup Schedule element is used to communicate the wakeup schedule (WS) of DMG STAs.”

- Adding the WS acronym to clause 3 would also be a good idea.

- WS is used as an acronym for ‘white spaces” in the PICS.

- Need to change name of RFC7545 to be correct.  “Protocol to Access White-Space (PAWS) Databasses.

#### Proposed Resolution:

REVISED. Replace with the WS acronym with TVWS in B.4.26 and delete the WS acronym in B.2.2. Also, add “WS wakeup schedule” to clause 3.4. At 2179.14, remove the italics for the word STA. And in E.2.5, page 4347 line 42, replace " IETF Protocol to Access WS database ‘paws-protocol’ with “see IETF RFC 7545 [B??]”

Add  bibliography entry in Annex A: “[B??] IETF RFC 7545 “Protocol to Access White-Space (PAWS) Databases”,

### CID 2199

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2199 | 2658.00 | 12.9.2 |  |  | There has been number of attempts to fix issues in the RSNA frame pseudocode subclauses over the years, but this has never really succeeded in getting sufficient contributions and review. Consequently, the current state of clause 12.9 is not exactly ideal and it is difficult to change anything in frame encryption without making these subclauses even less in sync with rest of the standard. Taken into account the inaccuracies in the current pseudocode for TKIP/CCMP/GCMP and BIP processing, it is questionable whether these pseudocode descriptions are of any real value to implementers. In fact, they may result in incorrect implementation. Based on lack of progress in cleaning this up in the past, it does not look likely that this would get sufficient work done within the REVmd schedule. Since the related requirements on the STAs should be covered elsewhere in the standard, the time to remove these pseudocode subclauses may have arrived. If there is anything within 12.9 and its subclauses that is not covered elsewhere in the standard, such contents should be added in the other security subclauses to keep it more maintainable. | Delete 12.9.2 and all its subclauses (page 2658 line 42 - page 2669 line 21). In 5.1.2 (page 298 line 45), replace "the decision tree for CCMP, GCMP, and BIP defined in 12.9 is driven by MIB attributes" with "the decision tree for CCMP, GCMP, and BIP is driven by MIB attributes". In B.4.4.1 (page 3553 lines 24-39) PC34, delete References column items 12.9.2, 12.9.2.2, 12.9.2.4, 12.9.2.6, 12.9.2.8. In B.4.1.1 (page 3557 lines 43-56) PC34.1.10, delete References column items 12.9.2.3, 12.9.2.5, 12.9.2.7, 12.9.2.9. |

#### Discussion:

- Clause 12.9.2 is not normative and could be removed.

- Changes in 5.1.2:

“The MAC sublayer security services provided by CCMP, GCMP, and BIP rely on information from non

MAC sublayer management or system entities. Management entities communicate information to CCMP,

GCMP, and BIP through a set of MAC sublayer management entity (MLME) interfaces and MIB attributes;

in particular, the decision tree for CCMP, GCMP, and BIP defined in 12.9 (Per-frame pseudocode) is driven

by MIB attributes.”

- The changes to the PICS look correct.

- No changes are proposed to 2583.54, but since TKIP is no longer maintained, there should be no changes to that sub-clause.

- Straw poll: Should we delete the pseudocode be deleted?

  - Result: 7 – Yes; 2 – No; 0 – Abstain.

#### Proposed Resolution:

REVISED.  (D2.0) Delete 12.9.2 and all its subclauses (page 2658 line 42 - page 2669 line 21).
In 5.1.2 (page 298 line 45), replace "the decision tree for CCMP, GCMP, and BIP defined in 12.9 is driven by MIB attributes" with "the decision tree for CCMP, GCMP, and BIP is driven by MIB attributes".
In B.4.4.1 (page 3553 lines 24-39) PC34, delete References column items 12.9.2, 12.9.2.2, 12.9.2.4, 12.9.2.6, 12.9.2.8.
In B.4.4.1 (page 3557 lines 43-56) PC34.1.10, delete References column items 12.9.2.3, 12.9.2.5, 12.9.2.7, 12.9.2.9.

Additionally, at 2583.54, delete the “(See … frames.)”

CID 2341

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2341 | 4110.00 | C.3 |  |  | dot11RTSThreshold is limited to 65536. This is a threshold on PSDU size. This in turn means that it is not possible to not enable RTS/CTS for medium-size or bigger A-MPDUs | Change 65536 to 6500631 (looking forward to HE) |

#### Discussion:

- Mib specification text:

dot11FragmentationThreshold OBJECT-TYPE

SYNTAX Unsigned32 (256..65535)

UNITS "octets"

MAX-ACCESS read-write

- The maximum PSDU size is 4692480

- Not sure about HE (?) but it likely makes sense to align the threshold with the maximum PSDU size.

#### Proposed Resolution:

REVISED. Change 65536 to 4692480 at 4110.7 and 4110.27. Note to commenter, the P802.11ax amendment is not within the scope of Revmd. See table 9-25 (807.42 relative to D2.0).

### CID 2352

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2352 | 2564.00 | 12.5.3.4.3 |  |  | "The AAD does not include the header Duration field, because the Duration field value might change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission). The AAD includes neither the Duration/ID field nor the HT Control field because the contents of these fields might change during normal operation (e.g., due to a rate change preceding retransmission)." -- duplication | Delete the first of the cited sentences at the referenced location |

#### Discussion:

- Text at the cited location:

“The AAD does not include the header Duration field, because the Duration field value might change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission). The AAD includes neither the Duration/ID field nor the HT Control field because the contents of these fields might change during normal operation (e.g., due to a rate change preceding retransmission). The HT Control field might also be inserted or removed during normal operation (e.g., retransmission of an A MPDU where the original A MPDU included an MRQ that has already generated a response). For similar reasons, several subfields in the Frame Control field are masked to 0.  For PV0 MPDUs, the AAD construction is performed as follows.”

#### Proposed Resolution:

ACCEPTED.

### CID 2353

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2353 | 2571.00 | 12.5.4.3 |  |  | "The Duration field in the AAD shall be masked to 0." - there is no Duration field in the AAD, so this statement is meaningless | Delete the cited text at the referenced location |

#### Discussion:

- The context of the text is as follows

The BIP Additional Authentication Data (AAD) shall be constructed from the MPDU header. The Duration

field in the AAD shall be masked to 0. The AAD construction shall use a copy of the IEEE 802.11 header

without the SC field for the MPDU, with the following exceptions:

a) FC—MPDU Frame Control field, with:

1) Retry subfield (bit 11) masked to 0

2) Power Management subfield (bit 12) masked to 0

3) More Data subfield (bit 13) masked to 0

b) A1—MPDU Address 1 field.

c) A2—MPDU Address 2 field.

d) A3—MPDU Address 3 field.

- The normative statement about the duration field should be included but could be worded more accurately.

- It would be better to include Duration in the letter list.

- Accept the editing instruction for the change but add the text at the beginning of the bulleted list.

- Comment from Yunsong Yang:

“For CID 2353, the commenter is correct that the Duration field is not used for constructing the AAD. Therefore, the cited sentence should be simply deleted. And the resolution should be Accepted.

Your revised resolution text is incorrect in that it implies that the Duration field is included in the AAD construction, which would contradict to Figure 12-25. And the length of the AAD would not add up to 20 octets with the Duration field.”

#### Proposed Resolution:

ACCEPTED

### CID 2354

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2354 | 2571.00 | 12.5.4.3 |  |  | "The AAD construction shall use a copy of the IEEE 802.11 header without the SC field for the MPDU" -- no, it doesn't, it uses the fields shown in Figure 12-25---BIP AAD Construction | Change the cited text at the referenced location to "The AAD construction shall use fields copied from the MAC header, with the following modifications:" |

#### Discussion:

- The context of the cited text is as follows:

The BIP Additional Authentication Data (AAD) shall be constructed from the MPDU header. The Duration field in the AAD shall be masked to 0. The AAD construction shall use a copy of the IEEE 802.11 header without the SC field for the MPDU, with the following exceptions fields copied from the MAC header, with the following modifications:

a) FC—MPDU Frame Control field, with:

1) Retry subfield (bit 11) masked to 0

2) Power Management subfield (bit 12) masked to 0

3) More Data subfield (bit 13) masked to 0

b) A1—MPDU Address 1 field.

c) A2—MPDU Address 2 field.

d) A3—MPDU Address 3 field.

Figure 12-25 (BIP AAD Construction) depicts the format of the AAD. The length of the AAD is 20 octets.

#### Proposed Resolution:

REVISED.

Change the cited text at the referenced location to "The AAD construction shall use the following fields copied from the MPDU header:

a) FC—MPDU Frame Control field, with the following modifications:”

### CID 2403

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2403 |  | C.3 |  |  | In a few places in the MIB, ^ is used for exponentiation. Everywhere else it's for XOR | Change ^ to ** throughout C.3 |

#### Discussion:

- In the front matter of Annex C, the draft states: “Follow the guidelines for writing MIB modules in http://www.rfc-editor.org/rfc/rfc4181.txt (including any updates thereto).” This document uses “^”.

- https://tools.ietf.org/html/rfc3418 describes a mib definition. This document uses “^”.

- https://tools.ietf.org/html/rfc2579 describes textual conventions. This document uses “^”.

- From these documents “^” should be used to express an exponent.

- In annex C, the occurrences of “**” are:

  - 3794.55

  - 3795.9

  - 3969.34

  - 3973.39

  - 4045.5

  - 4045.7

  - 4045.23

  - 4045.24 (2 times)

  - 4045.39

  - 4045.40

  - 4055.32

  - 4056.58

  - 4112.38

  - 4112.54

  - 4135.11

  - 4135.31

  - 4137.44

  - 4137.64

- In Annex C, the occurrences of “^” are:

  - 4075.23

  - 4077.55

  - 4086.39

  - 4101.20

- IETF seems to have adopted the convention of using “^”

#### Proposed Resolution:

REVISED. According to IETF document RFC 4181, RFC 3418, RFC 2579 regarding MIB modules, the convention for exponent is to use “^”. At the following locations, change “**” to “^”:

  - 3794.55

  - 3795.9

  - 3969.34

  - 3973.39

  - 4045.5

  - 4045.7

  - 4045.23

  - 4045.24 (2 times)

  - 4045.39

  - 4045.40

  - 4055.32

  - 4056.58

  - 4112.38

  - 4112.54

  - 4135.11

  - 4135.31

  - 4137.44

  - 4137.64

### CID 2408

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2408 | 2593.00 | 12.6.10.2 |  |  | This is the only "EAP-Packet" in the whole document. 802.1X-2010 states that "The EAPOL-EAP Packet Type was referred to as the EAP-Packet Packet Type in previous revisions of this standard." | Change the cited text at the referenced location to "EAPOL-EAP" |

#### Discussion:

- For context, here is the text:

“Preauthentication uses the IEEE 802.1X protocol and state machines with EtherType 88-C7, rather than the

EtherType 88-8E. Only IEEE 802.1X frame types EAP-Packet and EAPOL-Start are valid for

preauthentication.”

- There is no “frame type” in IEEE 802.1X-2010, the field is defined as a “packet type”

#### Proposed Resolution:

REVISED.

At cited location, replace “Only IEEE 802.1X frame types EAP-Packet and EAPOL-Start are valid …” with “Only IEEE 802.1X packet types EAPOL-EAP and EAPOL-Start are valid …”

### CID 2493

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2493 |  | 12 |  |  | The resolution to CID 1365 failed to fix the two Figures referenced (and F12-47 also needs fixing in fact and F13-3 in 13.4.2 etc.) | In Figure 12-47 change "Anonce, 0, 0, 0" to "0, Anonce, 0, {}", ", RSNE" to ", { RSNE }", "MIC, 0" to "MIC, {}" and put curly brackets around the last three arguments of the third EAPOL-Key frame. In Figure 12-48 put curly brackets around the last two arguments of the first EAPOL-Key frame and make the fifth argument of each of the EAPOL-Key frames be "G" not "0" and change "0)" to "{} )". Put curly brackets around the last 4 arguments of StaProcessEAPOL-Key in 12.7.9.4. In 12.7.9.4 change "RSNE)" to "{RSNE})". In Figure 12-50 change "0)" to "{})" in the first EAPOL-Key argument list and put curly brackets around the last 3 arguments of the second EAPOL-Key argument list. In Figure 12-52 put curly brackets around the last argument of the EAPOL-Key argument list. In Figure 13-2 insert ", {}" as the last argument of the first and last EAPOL-Keys; ", 0" as the sixth argument of all the EAPOL-Keys; put curly brackets around the arguments of the second and third EAPOL-Keys from the "RSNE" (included) to the closing paren (excluded). In Figure 13-15 change "ANonce, 0)" to "0, Anonce, 0, {})" in the first EAPOL-Key argument list and put curly brackets around the last 3 arguments and insert "0," as the sixth argument of the second EAPOL-Key argument list. In Figure 13-18 put curly brackets around the last 3 arguments and insert "0," as the sixth argument of the first EAPOL-Key argument list and change "MIC-KCK" to "0, MIC-KCK, {}" in the second EAPOL-Key argument list |

#### Discussion:

- The EAPOL-Key notation in the figures is not consistent with the text with the new Key Data notation (the use of curly brackets):

  - After the MIC, place the key data arguments inside curly brackets “{…}”, and replace “0” with “{}”

- The proposed resolution updates figure 12-47, 12-48, 12-50, 12-52, 13-2, 13-15, 13-18 and text in 12.7.9.4

- For figures 13-2, 13-15, and 13-18 the resolution contains a proposal adding the missing key data argument as {}, to make the notation consistent throughout the standard.

- The changes for 12.7.9.4 are at 2647.15 and 2648.45, respectively.

#### Proposed Resolution:

ACCEPTED

### CID 2512

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2512 | 2587.00 | 12.6.3 |  |  | 12.6.3 RSNA policy selection in an infrastructure BSS has lots of variation on whether it's just an AP/STA or an RSNA capable AP/STA or an RSNA-enabled AP/STA etc. | Insert a new para at the start of the referenced subclause add "The requirements in this subclause apply to all RSNA enabled APs/STAs." then delete "RSNA capable" and "RSNA-enabled" and "RSNA enabled" throughout |

#### Discussion:

- Paragraphs 3 and 5 of the cited clause use AP and STA without any qualifier.

- Note there is one case at 2588.27 where “RSNA enabled” is used. If we are removing “RSNA-enabled”, we should also remove “RSNA enabled”

#### Proposed Resolution:

REVISED. Insert a new para at the start of the referenced subclause add "The requirements in this subclause apply to a STA when dot11RSNAActivated is true." Delete "RSNA capable" and "RSNA-enabled" and "RSNA enabled" throughout.

### CID 2522

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2522 |  | C.3 |  |  | The MIB has some variables that are "It is written by the SME or external management entity.", some that are "It is written by an external management entity." and some that are "It is written by the SME." This implies that the SME is not considered an "external management entity", but this is not clear | At the end of 4.9.2 add a "NOTE---The SME is not considered an external management entity." |

#### Discussion:

- The SME is clearly defined in clause 4.9.2 and its evident from figure 4-24 that a SME is is defined as part of the IEEE 802.11 reference model.

- There are normative requirements specified for an SME in the IEEE 802.11 specification. There are no requirements specified on an external management entity in the IEEE 802.11 standard.

- The use of “external management entity” in Annex C.3 describes which MIB variables are written by the SME as opposed to an external management entity.

- Consider the example of dot11DesiredSSID:

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-SCAN.request primitive.

This attribute reflects the Service Set ID (SSID) used in the SSID

parameter of the most recent MLME-SCAN.request primitive. This value may

be modified by an external management entity and used by the local SME to

make decisions about the Scanning process."

- Clearly an external management entity is distinct from the SME.

- Given that external management entity is only used in the MIB (Annex C), a note could be added in the MIB.

#### Proposed Resolution:

REVISED. The SME is a required component to operate an IEEE 802.11 STA.  The SME is not an external management entity. At the end of C.1, add the following as a list bullet at the end of the bulleted list:

At 3770.19 (D2.0) “NOTE--An external management entity is a component that is beyond the scope of this standard. The SME is not an external management entity.”

### CID 2523

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2523 |  | C.3 |  |  | The MIB has some variables that are "It is written by the STA". It is not clear what this means, regarding whether the SME is part of the STA, i.e. whether if a MIB variable is set by the STA, that includes being set by the STA's SME, or whether necessarily means the MLME, MAC or lower layers | Change all instances of "It is written by the STA" in C.3 to "It is written by the MAC" |

#### Discussion:

- There are two occurences of “It is written by the STA”: 3872.50 (dot11RMLCIConfigured) and 3873.6 (dot11RMCivicConfigured).

- For dot11RMLCIConfigured, the description states:

“It is written by an external management entity which sets the Value to true after it configures dot11STALCIEntry.

It is written by the STA when an external management entity configures dot11STALCIEntry.

Changes take effect as soon as practical in the implementation. This attribute, when true, indicates that that the station is configured with an LCI location (LCI is not Unknown). false indicates the station is not configured with an LCI location or the configured LCI Location is set to Unknown (as defined in 9.4.2.21.10 (LCI report (Location configuration information report))).”

- For dot11RMCivicConfigured), the description states:

“It is written by an external management entity which sets the Value to true when it configures dot11STACivicLocationEntry.

It is written by the STA when an external management entity configures dot11STALCIEntry.

Changes take effect as soon as practical in the implementation. This attribute, when true, indicates that that the station is configured with a civic location (civic location is not Unknown). false indicates the station is not configured with and civic location or the configured civic Location is set to Unknown (as defined in 9.4.2.21.13 (Location Civic

report)).”

- The two boolean variables are mutually exclusive so when dot11RMLCIConfigured is set to true, dot11RMCivicConfigured would need to be false, and visa versa.

- For dot11RMLCIConfigured, the second “dot11STALCIEntry” should be “dot11RMCivicLocationEntry”

#### Proposed Resolution:

REVISED.

At 3872.47, change “It is written by an external management entity which sets the Value to true after it configures dot11STALCIEntry.” To “It is written by an external management entity, which sets it to true after the external management entity configures dot11STALCIEntry.”

At 3873.3, change “It is written by an external management entity which sets the Value to true when it configures dot11STACivicLocationEntry.” To “It is written by an external management entity, which sets it to true after the external management entity configures dot11STACivicLocationEntry.”

At 3872.50, change “It is written by the STA when an external management entity configures dot11STALCIEntry.” to “It is written by the SME, which sets it to false when an external management entity configures dot11RMCivicLocationEntry.”

At 3873.6, change “It is written by the STA when an external management entity configures dot11STALCIEntry.” to “It is written by the SME, which sets it to false when an external management entity configures dot11STALCIEntry.”

### CID 2541

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2541 | 2625.00 | 12.7.6.1 |  |  | The "{blah} or {boo}" notation is not clear and not all args (e.g. PMKID) are mentioned in the "Here," below | At the end of the list in the referenced subclause add "- PMKID represents the appropriate PMKID" and "- "{a} or {b}" means that exactly one of {a} and {b} is present" |

#### Discussion:

- The assumptions in the current draft do not include a reference to the PMKID or an explanation of the “or” notation.

- Not sure what is meant in the proposed resolution by “appropriate PMKID”, so it would be better to word the assumption more consistently with the description in message 1.

#### Proposed Resolution:

REVISED. At 2626.35 (D2.0), add:

"- PMKID identifies the PMKSA selected by the Authenticator

- "{a} or {b}" means that exactly one of either {a} or {b} is present as the {Key Data}”

### CID 2552

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2552 |  | 12.12.2.1 |  |  | "To prevent key reinstallation attacks, the non-AP STA shall maintain a copy of the most recent GTK and most recent IGTK installed as part of the FILS authentication protocol as if they were installed as a result of receipt of EAPOL-Key frames" -- but the requirements in 6.3.19 only apply to keys installed as a result of receipt of EAPOL-Key frames, those installed "as if". Same issue in 13.5.1 | In 6.3.19.1.4 change "installed as a result of receipt of EAPOL-Key frames" to "installed as a result of, or as if as a result of, receipt of EAPOL-Key frames" |

#### Discussion:

- The original text is:

“When the Key Type is Group or IGTK, and the key matches the existing GTK or IGTK installed as a result of exiting WNM sleep mode (see 11.2.3.16.1 (WNM sleep mode capability)), if any, or matches the existing GTK or IGTK installed as a result of receipt of EAPOL-Key frames (see 12.7.7.4 (Group key handshake implementation considerations)) receipt of this primitive shall have no effect.”

- The key piece is “matches the existing GTK or IGTK installed” – so why do we need the “as a result…” bit.

- What about:

“When the Key Type is Group or IGTK, and the key matches the most recent GTK or IGTK installed, receipt of this primitive shall have no effect.”

#### Proposed Resolution:

ACCEPTED.

### CID 2626

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2626 |  | 12 |  |  | "Hash is" wording is inconsistent ("Hash is the hash algorithm identified by the AKM suite selector (see Table 9-151)", "Hash is the hash algorithm identified by the negotiated AKM suite selector specified in Table 9-151", "Hash is the AKM-specific hash algorithm", "Hash is the hash algorithm(#307) specific to the negotiated AKM") | Change each of the instances of the text cited in the parenthesis in the comment to "Hash is the hash algorithm specific to the negotiated AKM (see Table 9-151)" |

#### Discussion:

- Here is the context for each of the quoted text:

- 2612.39:

“where

— Hash is the hash algorithm identified by the AKM suite selector (see Table 9-151 (AKM suite selectors)).(

— "FT-R0N" is treated as an ASCII string.

The PMKR0Name is used to identify the PMK-R0.”

- 2613.11:

“where

— Hash is the hash algorithm identified by the AKM suite selector (see Table 9-151 (AKM suite selectors))

— "FT-R1N" is treated as an ASCII string.

PMKR1Name is used to identify the PMK-R1.”

- 2640.51:

“Hash 		is the hash algorithm identified by the negotiated AKM suite selector specified in Table 9-151 (AKM suite selectors)”

- But there are more:

- 2681.58:

“Hash 		is the AKM-specific hash algorithm”

- 2683.25

“Hash 		is the hash algorithm specific to the negotiated AKM”

- 2685.50

“Hash 		is the hash algorithm specific to the negotiated AKM”

#### Proposed Resolution:

ACCEPTED. Note to editor, the changes are listed below.

At 2612.39, 2613.11

Change

“is the hash algorithm identified by the AKM suite selector (see Table 9-151)”

To

“is the hash algorithm specific to the negotiated AKM (see Table 9-151)”

At 2640.51,

Change

“is the hash algorithm identified by the negotiated AKM suite selector specified in Table 9-151”

To

“is the hash algorithm specific to the negotiated AKM (see Table 9-151)”

At 2681.58,

Change

“is the AKM-specific hash algorithm”

To

“is the hash algorithm specific to the negotiated AKM (see Table 9-151)”

At 2683.25 and 2685.50,

Change

“is the hash algorithm specific to the negotiated AKM”

To

“is the hash algorithm specific to the negotiated AKM (see Table 9-151)”

### CID 2722

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2722 | 2691.00 | 12.6.22.1 |  |  | It doesn't make a sense, in the last sentence of 12.6.22.1, to reference 12.6.22, which leads right back to 12.6.22.1. Since the sentence states that separates RSNA is needed for cases other than the transparent FST, it would make a sense to reference 12.6.22.2 (Nontransparent multi-band RSNA), instead. | Change "see 12.6.22 (Multi-band RSNA)" to "see 12.6.22.2 (Nontransparent multi-band RSNA)". |

#### Discussion:

- Cited text:

“If the pairwise and group cipher suites used by a pair of multi-band capable devices to communicate with

each other in the current operating band/channel is also supported after the transfer to another band/channel

that was performed using transparent FST, the devices shall continue using the same cipher suites to

communicate with each other after the transfer. In all other cases, a separate RSNA has to be established for

the other band/channel (see 12.6.22 (Multi-band RSNA)).”

12.6.22.2 Nontransparent multi-band RSNA

An RSNA initiator can establish a nontransparent (11.32 (Multi-band operation)) multi-band RSNA with an

RSNA responder for a supported band/channel other than the current operating band/channel. The two STAs

use the same PMKSA for both the supported band/channel and the current operating band/channel and

create different PTKSAs for different bands/channels.”

- Commenter is correct.

#### Proposed Resolution:

REVISED. Change "see 12.6.22 (Multi-band RSNA)" to "see 12.6.22.2 (Nontransparent multi-band RSNA) and 12.6.22.4".

### CID 2140

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2140 |  |  |  |  | WEP is obsolete but we still have it referrred to as "deprecated". Similarly other obsolete mechanisms are refered to as deprecated. Let's correct this. | At 203.35 replace "A deprecated" with "An obsolete" . At 298.48 change "deprecated" to "obsolete". At 298.52 change "TKIP is deprecated" to "TKIP" is obsolete". At 1166.50 (dual beacon) change "deprected" to "obsolete". At 1166.62 (dual CTS) change "deprecated" to "obsolete". At 1735.32 change "deprecated" to "obsolete". At 2513.24 change "deprecated" to "obsolete". At 2513 change "deprecated" to "obsolete". At 3546.38 change "deprecated" to "obsolete". At 3664.18 change "deprecated" to "obsolete". |

#### Discussion:

- According to 11md Draft 2.0, WEP is deprecated, not obsolete.

- 1166.62, 1735.32 – Is dual CTS obsoleted or deprecated?

- 3554.7 (TKIP) – should be marked deprecated

- Discussion during 11md adhoc (2 April 2019)

  - Straw poll on preferred disposition WEP and TKIP

    - 0 – deprecated

    - 9 – obsolete

    - 4 – abstain

- Make changes to make dual-CTS obsolete consistently throughout the draft.

- Make changes to dual-Beacon obsolete consistently throughout the draft.

#### Proposed Resolution:

REVISED. (WEP and TKIP)

At 203.35 replace "A deprecated" with "An obsolete"

At 298.48 change "deprecated" to "obsolete".

At 298.52 change "TKIP is deprecated" to "TKIP is obsolete".

At 2513.24 change "deprecated" to "obsolete".

At 2513.28 change "deprecated" to "obsolete".

At 3546.38 change "deprecated" to "obsolete".

At 3664.18 change "deprecated" to "obsolete".

At 3554.7. Add the following sentence at the end of the existing text. “This capability is obsolete. Support for this mechanism might be removed in a later revision of the standard.”

At 3546.36, Change “Wired equivalent privacy (WEP) algorithm. This capability is deprecated (applicable only to systems that are backward compatible).” To “Wired equivalent privacy (WEP) algorithm. This capability is obsolete. Support for this mechanism might be removed in a later revision of the standard.”

(dual CTS) At 1166.62 (dual CTS) change "deprecated" to "obsolete".

At 1735.32 change “deprecated” to “obsolete”.

At 3664.18 change “deprecated” to “obsolete”.

(dual beacon) At 1166.50 (dual beacon) change "deprecated" to "obsolete".

Note to commenter: This resolution adds to the changes suggested by the commenter.

### CID 2141

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2141 |  |  |  |  | We have 7 mechanisms that are 'obsolete', Including WEP. We have made the decision, or did we, that these would not be maintained. If this is true we should say so. Not sure where but we should insert somewhere text such as "This revision marks some features as obsolete. It should be noted that obsolete features are not maintained." | Add "This revision marks some features as obsolete. It should be noted that obsolete features are not maintained." somewhere appropriate. |

#### Discussion:

- This text should likely go in the introduction. Here is the most likely place:

“Technical corrections, clarifications, and enhancements

In addition, this revision specifies technical corrections and clarifications to IEEE Std 802.11 as well as

enhancements to the existing medium access control (MAC) and physical layer (PHY) functions. In

addition, this revision removes some features previously marked as obsolete and adds new indications of

other obsolete features.”

#### Proposed Resolution:

REVISED. At the end of the paragraph at 9.25, add the following sentence: “Generally, features that are marked deprecated or obsolete are not maintained.”

### CID 2243

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2243 | 298.00 |  |  |  | 5.1.2 and 12.2.1 say WEP and TKIP are deprecated. 12.3.1 says all pre-RSNA are obsolete. These need to be aligned. | TGm needs to discuss and decide on a direction, then a submission will be provided with detailed changes. |

#### Discussion:

- The only place where an obsolete statement is made with respect to WEP and TKIP is shown below. All other statements describe WEP and TKIP as deprecated.

“12.3.1 Status of Pre-RSNA security methods

Except for Open System authentication, all pre-RSNA security mechanisms are obsolete. Support for them

might be removed in a later revision of the standard.”

- Assuming the group still agrees to maintain WEP and TKIP as deprecated or obsolete, this statement should be updated.

#### Proposed Resolution:

REVISED.

Note to editor: This resolution is the same as CID 2140.

At 203.35 replace "A deprecated" with "An obsolete"

At 298.48 change "deprecated" to "obsolete".

At 298.52 change "TKIP is deprecated" to "TKIP is obsolete".

At 2513.24 change "deprecated" to "obsolete".

At 2513.28 change "deprecated" to "obsolete".

At 3546.38 change "deprecated" to "obsolete".

At 3664.18 change "deprecated" to "obsolete".

At 3554.7. Add the following sentence at the end of the existing text. “This capability is obsolete. Support for this mechanism might be removed in a later revision of the standard.”

At 3546.36, Change “Wired equivalent privacy (WEP) algorithm. This capability is deprecated (applicable only to systems that are backward compatible).” To “Wired equivalent privacy (WEP) algorithm. This capability is obsolete. Support for this mechanism might be removed in a later revision of the standard.”

(dual CTS) At 1166.62 (dual CTS) change "deprecated" to "obsolete".

At 1735.32 change “deprecated” to “obsolete”.

At 3664.18 change “deprecated” to “obsolete”.

(dual beacon) At 1166.50 (dual beacon) change "deprecated" to "obsolete".

### CID  2572

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2572 |  |  |  |  | Per CID 1188, TGmd has agreed that changes to fix issues with material associated with deprecated features such as WEP and TKIP | Make the changes requested in previous ballots that were rejected on the basis that no change was going to be made for text related to deprecated features (e.g. CID 1441, 1518) |

#### Discussion:

- Review adhoc notes from CID 1188.

#### Proposed Resolution:

REJECTED. Generally, features that are marked deprecated or obsolete are not maintained. For CID 1188, the following rationale was considered in accepting the comment: “This is not a deprecated feature, it is a valid frame and field, with a possible value that indicates reference to a deprecated item.”

### CID 2669

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| 2669 | 2518.00 | 12.2.10 |  |  | The requirements on a STA behavior described in the cited paragraph could be improved. The paragraph should clearly indicate that a STA needs to determine what MAC address to use when it selects a BSS for Association. | Commenter will provide a submission to update the cited text. |

#### Discussion:

- []

#### Proposed Resolution:

[]