IEEE P802.11
Wireless LANs

| Text proposal on ISTA-2-RSTA LMR feedback |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-05-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ali Raissinia | Qualcomm |  |  | alirezar@qti.qualcomm.com |
| Chittabrata Ghosh | Intel corporation |  |  | chittabrata.ghosh@intel.com |
| Christian Berger | Marvell |  |  | crberger@marvell.com |
| Ganesh Venkatesan | Intel corporation |  |  | ganesh.venkatesan@intel.com |
| Roy Want | Google |  |  | roywant@google.com |

Abstract

This submission contains a proposal to resolve CID-2295, 2296, 2297, 2298, 2299, 2300, xxxx, xxxx, received during TGaz LB# 240.  The proposal clarifies that an RSTA can request an ISTA to transmit the time measurement feedback and optionally the AoA measurement feedback to the RSTA only if the ISTA has indicated that it is willing to share its data.

Introduction

Introduction

This submission contains a proposal to resolve CID-2295, xxxx, xxxx, xxxx,  received during the 11az LB#240. The proposal clarifies that, the procedure to negotiate ISTA2RSTA LMR reporting.

| CID | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 2295 | 48.14 | "The ISTA2RSTA LMR Feedback subfield in the Ranging Parameters field is set to 1 in the Initial Fine Timing Measurement Request frame indicates that the ISTA is willing to report the estimated LMR to the RSTA; when included in the Initial Fine Timing Measurement frame indicates that the RSTA requires a LMR report from the ISTA at the end of each ranging exchange. Otherwise the ISTA2RSTA LMR Feedback subfield is set to 0. See 11.22.6.4.2.4 (TB 14 19 Measurement Reporting Part) and 11.22.6.4.3.3 (Measurement Report)". The text needs to be clarified that only when the ISTA has set the ISTA2RSTA LMR Feedback field to 1 (i.e., willing to share its location information) in the initial FTM Request frame, then the RSTA may set the ISTA2RSTA LMR Feedback field to 1 in the initial FTM frame. | Modify the text to clarify that, only when the ISTA has set the ISTA2RSTA LMR Feedback field to 1 (i.e., willing to share its location information) in the initial FTM Request frame, then the RSTA may set the ISTA2RSTA LMR Feedback field to 1 in the initial FTM frame; the RSTA shall set the ISTA2RSTA LMR Feedback field to 0 in the initial FTM frame if ISTA has set the ISTA2RSTA LMR Feedback field to 0 (i.e., not willing to share its location information) in the initial FTMRequest frame. See submission. | Revised per document 11-19-0481-lb240-cr-for-cid2295,cid2296,cid2297,cid2298,cid2299,cid2300 |
| 2296 | 48.14 | "The I2R ToA Type subfield is set to 1 in the initial Fine Timing Measurement Request frame to indicate that the ISTA supports phase shift type ToA feedback in the ISTA-to-RSTA LMR. The I2R ToA type subfield is set to 1 in the initial Fine Timing Measurement frame to set the ToA feedback type in the ISTA-to-RSTA LMR to phase shift, corresponding to the average linear phase across the subcarriers. Otherwise, the I2R ToA Type is set to 0 and the ISTA-to-RSTA LMR ToA feedback type will be first path reporting." The spec needs to specify that when the ISTA-to-RSTA LMR Feedback field in the initial FTM Request frame is set to 0 (i.e., not willing to share its location information), then the I2R ToA Type subfield in both the initial FTM Request frame and the initial FTM frame shall have no meaning. | Modify the spec to specify that when the ISTA-to-RSTA LMR Feedback field in the initial FTM Request frame is set to 0 (i.e., not willing to share its location information), then the I2R ToA Type subfield in both the initial FTM Request frame and the initial FTM frame is reserved. See submission. | Revised per document 11-19-0481-lb240-cr-for-cid2295,cid2296,cid2297,cid2298,cid2299,cid2300 |
| 2297 | 48.29 | "The I2R AoA Requested subfield is set to 1 in the initial Fine Timing Measurement Request frame to indicate that the ISTA supports AoA measurement feedback in the ISTA-to-RSTA LMR. The I2R AoA Requested subfield is set to 1 in the initial Fine Timing Measurement frame by the RSTA to request the ISTA to include AoA measurements in the ISTA-to-RSTA LMR in the AoA feedback field." The spec needs to specify that when the ISTA-to-RSTA LMR Feedback field in the initial FTM Request frame is set to 0 (i.e., not willing to share its location information), then the I2R AoA Requested subfield in both the initial FTM Request frame and the initial FTM frame shall have no meaning. | Modify the spec to specify that when the ISTA-to-RSTA LMR Feedback field in the initial FTM Request frame is set to 0 (i.e., not willing to share its location information), then the I2R AoA Requested subfield in both the initial FTM Request frame and the initial FTM frame is reserved. See submission. | Revised per document 11-19-0481-lb240-cr-for-cid2295,cid2296,cid2297,cid2298,cid2299,cid2300 |
| 2298 | 86.32 | In this section, text needs to be added to specify that The ISTA shall set the ISTA-to-RSTA LMR feedback field in the Ranging Parameters field in the initial Fine Timing Measurement Request frame to 0 if the ISTA is not willing to share its time measurement or AoA measurement to the RSTA. As a result, the RSTA shall set the ISTA-to-RSTA LMR feedback subfield field in the Ranging Parameters field in the initial Fine Timing Measurement frame to 0. An RSTA shall not reject an ISTA's request because the ISTA has set the ISTA-to-RSTA LMR feedback field in the Ranging Parameters field in the initial Fine Timing Measurement Request frame to 0. | For Rev_mc ranging (i.e., the legacy FTM protocol), the timestamps are transmitted only from the rSTA to iSTA. In the base spec, there is a separate feature that a STA 1(AP, or STA) can request a another STA 2 to send the location of STA1 (self), STA 2 (peer) or STA 3 ( 3r party); STA 2 can refuse to share the requested location information by transmitting a response frame indicating "refused". The protocol was designed precisely for the privacy protection. The 11az spec needs to follow the same principle, so that the the end users, not the networks, have the control of the users' location privacy. The use of the 11az ranging protocol shall not require a user to give up its location privacy. Alternatively, the iSTA-2-rSTA LMR report is not needed and can be removed, because the existing feature of Location LCI Report in the 802.11 base spec can be used to obtain another STA's location. | Revised per document 11-19-0481-lb240-cr-for-cid2295,cid2296,cid2297,cid2298,cid2299,cid2300 |
| 2299 | 86.32 | In this section, text needs to be added to specify that when the ISTA-to-RSTA LMR Feedback field in the initial FTM Request frame is set to 0 (i.e., not willing to share its location information), then the I2R ToA Type subfield in both the initial FTM Request frame and the initial FTM frame is reserved. | As in comment. | Revised per document 11-19-0481-lb240-cr-for-cid2295,cid2296,cid2297,cid2298,cid2299,cid2300 |
| 2300 | 86.32 | Modify the spec to specify that when the ISTA-to-RSTA LMR Feedback field in the initial FTM Request frame is set to 0 (i.e., not willing to share its location information), then the I2R AoA Requested subfield in both the initial FTM Request frame and the initial FTM frame is reserved. | As in comment. | Revised per document 11-19-0481-lb240-cr-for-cid2295,cid2296,cid2297,cid2298,cid2299,cid2301 |

Instruction to the editor: The proposed modifications are in reference to the text in IEEE P802.11802.11az_D1.0, and are indicated by the change marks as follows:

      - Ranging Parameters

TGaz Editor: Modify the following paragraph as shown below:

The ISTA2RSTA LMR Feedback subfield in the Ranging Parameters field is set to 1 in the Initial Fine Timing Measurement Request frame is set to 0 to indicate that the ISTA is requesting a FTM session in which the ISTA is not required to transmit ISTA2RSTA LMRs, and is set to 1 to indicate that the ISTA, when requested by the RSTA, transmits the ISTA2RSTA LMR. See 11.22.6.4.2.4 (TB Measurement Reporting Part) and 11.22.6.4.3.3 (Measurement Report)

TGaz Editor: Modify the following paragraph as shown below:

If the ISTA2RSTA LMR Feedback subfield in the Ranging Parameters field in the Initial Fine Timing Measurement Request frame is set to 1, the I2R ToA Type subfield in the initial Fine Timing Measurement Request frame is set to 1 to indicate that the ISTA supports phase shift type ToA feedback and is set to 0 to indicate that the ISTA supports the first path reporting in the ISTA2RSTA LMR. The I2R ToA type subfield in the initial Fine Timing Measurement frame is set to 1 to indicate that the ToA feedback type in the ISTA2RSTA LMR to be phase shift type ToA corresponding to the average linear phase across the subcarriers, and is set to 0 to indicate that the ISTA2RSTA LMR ToA feedback type to be be first path reporting.

TGaz Editor: Modify the following paragraph as shown below:

If the ISTA2RSTA LMR Feedback subfield in the Ranging Parameters field in the initial Fine Timing Measurement Request frame is set to 1, the I2R AoA Requested subfield in the initial Fine Timing Measurement Request frame is set to 1 to indicate that the ISTA supports AoA measurement feedback in the ISTA-to-RSTA LMR,and is set to 0 otherwise. If the ISTA2RSTA LMR Feedback subfield and the I2R AoA Requested subfield in the initial Fine Timing Measurement Request frame are set to 1, the I2R AoA Requested subfield in the initial Fine Timing Measurement frame is set to 1 to request the ISTA to include AoA measurements in the ISTA2RSTA LMR in the AoA feedback field, and is set to 0 otherwise.

11.22.6.3.3 Trigger-based and non-Trigger-based Ranging Measurement Negotiation

— maximum number of LTF repetitions it is capable of receiving in the preamble of the DL NDP frames, in the Max DL Rep subfield of the Ranging Parameters field.

— maximum number of LTF repetitions it is capable of transmitting in the preamble of the  UL NDP frames in the Max UL Rep subfield of the Ranging Parameters field.

The ISTA shall set the Max DL Rep and Max UL Rep subfields to a value greater than 0 if the Secure LTF Required subfield of the Ranging Parameters field is equal to 1.

TGaz Editor: Insert the following text after the above paragraph.

The ISTA2RSTA LMR Feedback subfield in the Initial Fine Timing Measurement frame is set to 0 to indicate that the RSTA does not require the ISTA to transmit a ISTA2RSTA LMR Report frame to the RSTA at the end of every measurement exchange; it is set to 1 to indicate that the RSTA requires the ISTA to transmit a ISTA2TSTA LMR Report frame to the RSTA, when requested by the RSTA, at the end of every measurement exchange. In case the RSTA sets the value in ISTA2RSTA LMR Feedback subfield in the Ranging Parameter field to 1 in the Initial Fine Timing Measurement frame, the ISTA may either proceed with the ranging operation or terminate the FTM session as described in Subclause 11.22.6.6.

By setting the ISTA2RSTA LMR Feedback field in the Ranging Parameter field in the initial Fine Timing Measurement Request Frame to 0 the ISTA shall request an FTM session without transmission of ISTA2RSTA LMR Report frame, and by setting the ISTA2RSTA LMR Feedback field in the Ranging Parameters field in the initial Fine Timing Measurement Request to 1 it shall request an FTM session with transmission of ISTA2RSTA LMR frame.

If an ISTA has set the ISTA2RSTA LMR Feedback field in the Ranging Parameters field in the initial Fine Timing Measurement Request frame to 1, then the RSTA may set the ISTA2RSTA LMR Feedback field in the Ranging Parameters field in the initial Fine Timing Measurement frame to 1 to request an ISTA2RSTA LMR reporting, and to 0 otherwise.

For TB ranging and non-TB ranging, the Ranging Priority subfield of the Ranging Parameters field of the Ranging Parameters element in the initial Fine Timing Measurement Request frame contains the ISTA’s Ranging Priority request which indicates the time sensitivity of a ranging operation, and it is set according to Table 9-281a in 9.4.2.167.

References:

1. IEEE Draft P802.11az_D1.0, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications, Amendment 8: Enhancements for locationing