IEEE P802.11
Wireless LANs

|  | Proposed requirements for 802.11bd Functional Requirements Document |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Date: 2019-03-14 |  |  |  |  |  |  |  |  |  |
|  | Author(s): |  |  |  |  |  |  |  |  |  |
|  | Name |  | Affiliation |  | Address |  | Phone |  | email |  |
|  | Bahar SADEGHI |  | Intel |  |  |  | +1-503-803-2471 |  | bahareh.sadeghi@intel.com |  |
| Joseph LEVY |  | InterDigital, Inc. |  |  |  | +1.631.622.4139 |  | jslevy@ieee.org |  |  |

### Abstract

This document describes the 802.11bd Functional Requirements.

- Introduction

- Purpose

This document summarizes requirements, agreed by the Task Group, for solutions addressing functionality to be provided by the 802.11bd amendment, referred to as the TGbd Functional Requirements (FRs).

- Scope

The scope for deriving functional requirements is primarily set by the P802.11bd PAR Error: Reference source not found and CSD Error: Reference source not found, as well as by the TGbd use cases Error: Reference source not found. Additional requirements may be added if agreed by  the task group.

- Notation

Requirements are identified by a preceding unique number in the format of “TGbd Rn”, where n is an integer number representing the ID of the requirements.

- Functional Requirements

  - System Performance

    - Throughput measured at MAC SAP

- The 802.11bd amendment shall provide at least one mode of operation capable of achieving at least two times higher throughput, compared to IEEE Std 802.11™-2016 operating at maximum mandatory data rate as defined in the 5.9 GHz band (12 Mb/s in a 10 MHz channel), in high mobility channel environments at vehicle speeds up to 250 km/h (closing speeds up to 500 km/h).
 (from Ref-1)

    - Transmission range

- The 802.11bd amendment shall provide at least one mode of operation that achieves at least 3dB lower sensitivity level (longer range), than that of the lowest data rate defined in IEEE Std 802.11™-2016 operating in 5.9 GHz band (3 Mb/s in a 10 MHz channel) (from Ref-1)

  - Bands of Operation

- The 802.11bd amendment shall define operations for 5.9 GHz band as defined in clauses E.2.3 and E.2.4 of IEEE Std 802.11™-2016. (from Ref-1)

- The 802.11bd amendment may define operations for the 60GHz band (57 GHz to 71 GHz) as defined in clause E.1 of IEEE Std 802.11™-2016. (from Ref-1)

  - Coexistence, interoperability and backward compatibility

- The 802.11ba amendment shall provide interoperability, coexistence, backward compatibility, and fairness with OCB (Outside the Context of a BSS) devices as specified in IEEE Std 802.11™-2016. (from Ref-1)

  - Compliance to PAR

- The 802.11bd amendment shall comply with the PAR [Ref-1] and the CSD [Ref-2].

- References

- https://mentor.ieee.org/802.11/dcn/18/11-18-0861-09-0ngv-ieee-802-11-ngv-sg-proposed-par.docx

- https://mentor.ieee.org/802.11/dcn/18/11-18-0862-03-0ngv-ieee-802-11-ngv-sg-proposed-csd.docx

- https://mentor.ieee.org/802.11/dcn/18/11-18-1323-02-0ngv-ngv-sg-use-cases.pptx