IEEE P802.11
Wireless LANs

| Comment Resolutions for TXVECTOR and RXVECTOR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-03-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Bo Sun | ZTE | ZTE R&D center, #9 Wuxingduan, Xifeng Rd., Chang’an district, Xi’an, China | +86-29-68700944 | Sun.bo1@zte.com.cn |
| Yonggang Fang |  | ZTETX |  |  |

Abstract

This submission provisions with resolutions to the following comments on sub-clause 31.1.2 (TXVECTOR and RXVECTOR parameters) in IEEE P802.11ba D2.0, and suggested spec text modification to IEEE P802.11ba D2.0 to TGba editor.

- CIDs: 2621, 2574, 2780, 2590, 2591, 2779, 2068, 2495

Revisions:

- R0, initial comment resolution draft.

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGba Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGba Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TG ba Editor: Editing instructions preceded by “TGba Editor” are instructions to the TGba editor to modify existing material in the TGba draft.  As a result of adopting the changes, the TGba editor will execute the instructions rather than copy them to the TGba Draft.

Resolution for CID #2621/2574/2780/2590/2591/2779

| CID | Clause | P.L | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 2621 | 31.1.2 | 84.40 | The way to define "WUR PPDU format" using "WUR" may cause confusing since it may mean both single (or non-FDMA) WUR PPDU format and WUR FDMA PPDU. | Change "WUR indicates WUR PPDU format" to "WUR_SIGNAL indicates WUR SIGNAL PPDU format and the corresponding rows in the second column of Table 32.1 (This is the same comment and solution I gave for LB235. It was "rejected" due to "unable to reach consensus on a resolution"). | Revised Instruction to TGba tech editor: Please implement the proposed modification to IEEE P802.11ba D2.0 under resolution to CID 2621 as in 11-19/0570r0 Note to TGba tech editor: in current spec there’re several terms used to mean WUR FDMA PPDU, e.g. FDMA transmission, WUR FDMA, FDMA PPDU. And there’re several places using WUR PPDU referring to WUR SU PPDU or WUR SU PPDU + WUR FDMA PPDU. It’s suggested to go through the whole spec to use dedicated term for WUR SU PPDU and WUR FDMA PPDU respectively. |
| 2574 | 31.3.2 | 85.4 | Some of the entires in this table have no description in the value column. A note is paranthetical -- added as additional information on another statement -- an not a substitute for a description. | Add descriptions where they are missing. 85.4, 85.9, 85.21. | Revised Instruction to TGba tech editor: Please implement the proposed modification to IEEE P802.11ba D2.0 under resolution to CID 2574 as in 11-19/0570r0 |
| 2780 | 31.1.2 | 85.4 | Why do L_LENGTH and L_DATARATE parameters have only the NOTE? Please change it to the normative text. | As in comment. | Revised Instruction to TGba tech editor: Please implement the proposed modification to IEEE P802.11ba D2.0 under resolution to CID 2780 as in 11-19/0570r0 |
| 2590 | 31.1.2 | 85.30 | Unlike 11ax, in WUR the preamble puncturing in 80 MHz also allows the case where both the secondary 20 MHz as well as one of the two 20 MHz sub-channel in secondary 40 MHz are punctured. | Add one more enumerated type: WUR_CBW_PUNC80-PRI+SEC for preamble puncturing in 80 MHz, where in the preamble the secondary 20 MHz as well as one of the two 20 MHz sub-channels in the secondary 40 MHz are punctured. | Revised Instruction to TGba tech editor: Please implement the proposed modification to IEEE P802.11ba D2.0 under resolution to CID 2590 as in 11-19/0570r0 |
| 2591 | 31.1.2 | 85.49 | WUR FDMA PPDU may carry WUR frames using different data rates in different sub-channels. | Define means for specifying the data rate of WUR frame on each 20 MHz subchannel. One option is to use a 4 bits bitmap, 1 bit per 20 MHz channel, 0 = LDR, 1 = HDR. | Rejected Reason The parameter WUR_DATARATE for TXVECTOR indicates the only value when FORMAT is WUR and indicates the WUR data rate per user when FORMAT is WUR_FDMA. It allows different data rate for different user. |
| 2779 | 31.1.2 | 86.4 | Regarding CID 1152, The resolution is "REJECTED (MAC: 2019-01-20 06:57:45Z) - RSSI reflects the signal strength of received wake-up signal. It is necessary to support several WUR MAC functions, including but not limited to receiving error analytic." Please clarify which WUR MAC functions need the RSSI. Otherwise, please remove the RSSI parameter. | As in comment. | Rejected Reason The parameter RSSI could be used by Non-AP STA to detect the network environment while main radio is closed. For example, during the WUR discovery process, the WUR non-AP STA can measure the WUR AP’s discovery channels with RSSI. |

Discussion

CID 2621: the author agrees with the commenter that the term “WUR PPDU” as one value of parameter “FORMAT” may cause misunderstanding in places referring “WUR PPDU” to differentiate from Non-WUR PPDUs. Proposed to change the term “WUR PPDU” here to “WUR_SU”.

CID 2574 and CID 2780: the author agrees with the commenter that the description of value of TXVECTOR and RXVETOR parameters should not be replaced by notes. A normative description for range of value and the interpretation of different value is needed.

CID 2590: the author agrees with the commenter that 11ba has more flexible puncturing mode in a WUR_FDMA transmission. It allows to puncture the secondary20 channel and a 20 MHz sub-channel of the secondary40 channel.

Proposed Spec Modification

TGba Editor: please apply the following modification to the first sentence of the 2nd paragraph under sub-clause 31.2.1 (Introduction) at pg86/ln50 in 802.11ba spec draft D2.0 as resolution to CID 2621/2574/2780/2590.

31.1.2 WUR_TXVECTOR and WUR_RXVECTOR parameters

Table 31-1—WUR_TXVECTOR and WUR_RXVECTOR parameters

| Parameter | Value |  | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- | --- |
| FORMAT | Determines the format of the PPDU. Enumerated type: -- WUR_SU indicates WUR SU PPDU format. -- WUR_FDMA indicates WUR FDMA PPDU format. NOTE: for RXVECTOR, FORMAT only has the value of “WUR_SU”. [CID #2621] |  | Y | Y |
| L_ LENGTH | FORMAT is WUR_SU | Indicates the value used to set the LENGTH field in L-SIG. [CID #2574/2780] NOTE—the length field of the L-SIG in WUR PPDU is defined in Equation (31-14) using the TXTIME value defined by Equation (31-15) | Y | N |
|  | FORMAT is WUR_FDMA | Indicates the value used to set the LENGTH field in L-SIG. [CID #2574/2780] NOTE—the length field of the L-SIG in WUR FDMA PPDU is defined in Equation (31-14) using the TXTIME value defined by Equation (31-15) |  |  |
| L_ DATARATE | FORMAT is WUR_SU or WUR FDMA | Indicates the value used to set the RATE field in L-SIG. [CID #2574/2780] NOTE—the rate field in the L-SIG field in WUR PPDU is set to value representing 6Mb/s in the 20MHz channel spacing column of Table 17-6 (Contents of the Signal field). | Y | N |
| CHANNEL_BANDWIDTH | FORMAT is WUR_SU or WUR_FDMA | Indicates whether the packet is transmitted using 20 MHz or 40 MHz or 80 MHz channel width. Enumerated type: WUR_CBW_20 for 20 MHz. WUR_CBW_40 for 40 MHz. WUR_CBW_80 for 80 MHz. WUR_CBW_PUNC80-PRI for subchannel puncturing in 80 MHz, where in the preamble only the secondary 20 MHz is punctured. WUR_CBW_PUNC80-SEC for subchannel puncturing in 80 MHz, where in the preamble only one of the two 20 MHz subchannel in secondary 40 MHz is punctured. WUR_CBW_PUNC80-PRI-SEC-1 for subchannel puncturing in 80 MHz, where in the preamble only the secondary 20 MHz and the lower 20 MHz subchannel in secondary 40 MHz is punctured. WUR_CBW_PUNC80-PRI-SEC-2 for subchannel puncturing in 80 MHz, where in the preamble only the secondary 20 MHz and the higher 20 MHz subchannel in secondary 40 MHz is punctured. [CID #2590] | Y | N |
| WUR_ DATA RATE | FORMAT is WUR_SU or WUR_FDMA | Determines the transmission rate of the WUR_SU PPDU and WUR FDMA PPDU. Enumerated type: LDR indicates WUR low data rate for the data rate 62.5 kb/s. HDR indicates WUR high data rate for the data rate 250 kb/s | MU | Y |
| PSDU_LENGTH | FORMAT is WUR_SU or WUR_FDMA | Indicates the number of octets in the WUR MPDU in the range of 0 to a MPDUMaxLength octets | MU | Y |
| RSSI | FORMAT is WUR_SU or WUR_FDMA | The allowed values for the RSSI parameter are in the range 0 to RSSI maximum. This parameter is a measure by the PHY of the power observed at the antennas used to receive the current WUR PPDU. RSSI shall be measured during the reception of the WUR-Sync. RSSI is intended to be used in a relative manner, and it is a monotonically increasing function of the received power. | N | Y |
| Note 1—For TXVECTOR, we have the following: — “Y” means that the parameter is present. - “N” means that the parameter is not present. - “MU” means that the parameter is present for WUR SU PPDU and is present per user for WUR FDMA PPDU. Note 2—For RXVECTOR, we have the following: — “Y” means that the parameter is present for WUR SU PPDU. - “N” means that the parameter is not present. |  |  |  |  |

--------------------------------End of resolution to CID 2621/2574/2780/2590-----------------------------------------

| CID | Clause | P.L | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 2068 | 31.2.1 | 86.50 | "legacy" is used several times, what is legacy? | replace legacy with "non-HT" or something similar | Revised Instruction to TGba tech editor: Please implement the proposed modification to IEEE P802.11ba D2.0 under resolution to CID 2068 as in 11-19/0570r0 |
| 2495 | 31.2.1 | 86.50 | Using "legacy preamble" is not a well-defined term for the spec. It is not used anywhere in 802.11REVmd/D2.0. | Change "legacy preamble" to "pre-HT preamble" or "non-HT preamble". | Revised Instruction to TGba tech editor: Please implement the proposed modification to IEEE P802.11ba D2.0 under resolution to CID 2495 as in 11-19/0570r0 |

Resolution for CID # 2068/2495

Discussion:

CID 2068 and CID 2495: the author agrees with the commenters that the term “legacy” is unclear. Considering the detailed PHY preamble format will be introduced in later chapter and the commented content is only for a general introduction, it’s proposed to remove the unclear part instead of offering some detailed but non-stable words at this stage. With a further consideration, the commented sentence is not completed that it doesn’t cover the WUR_FDMA PPDU case. Therefore the resolution further propose a modification to include the WUR_FDMA PPDU case.

TGba Editor: please apply the following modification to the first sentence of the 2nd paragraph under sub-clause 31.2.1 (Introduction) at pg86/ln50 in 802.11ba spec draft D2.0 as resolution to CID 2068/2495.

31.2 WUR PHY

31.2.1 Introduction

This subclause provides the procedure by which PSDUs are converted to and from transmissions on the wireless medium.

During transmission, a PSDU (in the WUR PPDU case) or one or more PSDUs (in the WUR_FDMA PPDU case) isare processed and appended to the PHY preamble including legacy preamble, BPSK-Mark, and WUR-Sync field to create the WUR PPDU. At the legacy receivers, the legacy preamble is accordingly processed to aid in protection of the WUR-Sync and the PSDU. At the wake-up receiver, the WUR-Sync field is accordingly processed to aid in the detection, demodulation, and delivery of the PSDU.

---------------------------------------End of proposed modification as resolution to CID 2068/2495-----------------------