IEEE P802.11
Wireless LANs

| Proposed Comment Resolutions 2309, 2310 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-06-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Calcev | Futurewei Technologies | Rolling Meadows, Illinois, USA |  | gcalcev@futurewei.com |
| Mark Rison | Samsung |  |  |  |

Abstract

This document proposes resolutions to REVmd D2.0 for the following CIDs: 2309, 2310

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 2309 | 8.3.5 | 760 | 3 | "The effect of receipt of this primitive by the MAC is unspecified." -- it is not useful to have a primitive whose receipt has unspecified results | Delete subclauses 8.3.5.3/11/12/13/16 |
| 2310 | 8.3.5 | 760 | 3 | "The effect of receipt of this primitive by the MAC is unspecified." -- it is not useful to have a primitive whose receipt has unspecified results | Add something useful (e.g. for the first could talk about adding to nascent PSDU, for third that it potentially updates the overall CS state) |

Discussion

The comments are valid for Clauses: 8.3.5.3, 8.3.5.11, 8.3.5.12, 8.3.5.13, 8.3.5.16. We note that the comments refer to the same issue but offer two different possible resolutions. We revised 2309, which suggests the deletion of the clauses because we consider that the text could be improved to offer some useful alternative as suggested in the proposed resolution of CID2310  .

We revised the proposed resolution of CID 2310 as follows:

Proposed Revised Resolution

CID 2309- Revised

CID 2310- Revised

Proposed Changes

Instruct the editor to replace in Clause 8.3.5.3.4 (PHY-DATA.indication):

“The effect of receipt of this primitive by the MAC is unspecified.”

With

“The receipt of this primitive by the MAC entity causes the MAC to collect the data octet provided by the PHY in the current  receive flow  (see Figure 5-1).”

Instruct the editor to replace in Clause 8.3.5.11.1 (PHY-CCARESET.confirm)

“This primitive is issued by the PHY to the local MAC entity to confirm that the PHY has reset the CCA state machine and to provide observed IPI values when the IPI reporting is turned on.”

With

“This primitive is issued by the PHY to the local MAC entity to provide observed IPI values if IPI reporting was turned on prior to the receipt of the latest PHY-CCARESET.request primitive.”

Instruct the editor to replace in Clause 8.3.5.11.2 (PHY-CCARESET.confirm)

“The primitive provides the following parameters:

PHY-CCARESET.confirm(

IPI-STATE,

IPI-REPORT

)

The IPI-STATE parameter is present if dot11RadioMeasurementActivated is true. The IPI-STATE parameter can be one of two values: IPI-ON or IPI-OFF. The IPI-STATE value shall be set to the value of IPI-STATE received by the PHY entity in the most recent PHY-CCARESET.request primitive.”

With

“The primitive provides the following parameters:

PHY-CCARESET.confirm(

IPI-REPORT

)

”

Instruct the editor to replace in Clause 8.3.5.11.3 (PHY-CCARESET.confirm)

“This primitive is issued by the PHY to the MAC entity when the PHY has received a PHY-CCARESET request primitive.”

With

“This primitive is issued by the PHY to the MAC entity when IPI reporting was turned on prior to the receipt of the latest PHY-CCARESET.request primitive.”

Instruct the editor to replace in Clause 8.3.5.11.4 (PHY-CCARESET.confirm):

“The effect of receipt of this primitive by the MAC is unspecified.”

With

“The receipt of this primitive by the MAC entity causes the MAC to collect IPI values when IPI reporting was turned on prior to the receipt of the latest PHY-CCARESET.request primitive.

Instruct the editor to replace in Clause 8.3.5.12.4 (PHY-CCA.indication):

“The effect of receipt of this primitive by the MAC is unspecified”

With

“The receipt of this primitive by the MAC entity causes the MAC to update the physical  CS  state,  and to collect IPI values when IPI reporting was turned on.”

Instruct the editor to replace in Clause 8.3.5.13.4 (PHY-RXSTART.indication):

“The effect of receipt of this primitive by the MAC is unspecified”

With

“The receipt of this primitive by the MAC entity causes the MAC to prepare for a new receive flow (see Figure 5-1).”

Instruct the editor to delete the entire Clause 8.3.5.16 (PHY-CONFIG.confirm):