IEEE P802.11
Wireless LANs

| 802.11bc Functional Requirements Document Proposed Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-06-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | BlackBerry | The Pearce Building, West Street, Maidenhead, SL6 1RL, UK |  | smccann@blackberry.com |

### Abstract

This document describes a proposed update to the 802.11bc Functional Requirements Document 11-19-0151r3.

- Introduction

  - Purpose

This document summarizes requirements, which agreed to by the Task Group, for solutions addressing functionality to be provided by the 802.11bc amendment, referred to as the TGbc Functional Requirements (FRs).

  - Scope

The scope for deriving functional requirements is primarily set by the P802.11bc PAR Error: Reference source not found and CSD Error: Reference source not found, as well as by the TGbc usage models Error: Reference source not found. Additional requirements may be added if agreed by and considered essential by the task group.  The following figure illustrates the workflow on how to derive functional requirements and how to add them to the Functional Requirements Document owned by the task group following the steps of the 802.11bc Selection Procedure Error: Reference source not found.

  - Notation

Requirements are identified by a preceding unique number in the format of “TGbc Rn”, where n is an integer number representing the ID of the requirements.

- Definitions

TGbc D2.1:	“Consuming”:	By “consuming a broadcast service”, it is meant that STA receives a corresponding frame and forwards its contents to the upper layer.

- Functional Requirements

  - Compliance to PAR

TGbc R3.1.1:	 The 802.11bc amendment shall comply with the PAR Error: Reference source not found and the CSD Error: Reference source not found.

  - Restrictions

TGbc R3.2.1:	 The 802.11bc amendment shall not change the IEEE 802.11 PHY specifications. [Ref-1 5.2.b]

  - Security

TGbc R3.3.1:	 The 802.11bc amendment shall provide origin authenticity protection for broadcast data frames. [Ref-1 5.2.b]

TGbc R3.3.2:	 The 802.11bc amendment shall provide a broadcast mechanism in which e2e security w/ confidentiality and integrity between STA and the server is assumed. [Ref-5]

TGbc R3.3.3:	 The 802.11bc amendment shall provide a mechanism for the AP to verify the STA before forwarding the packets. [Ref-5]

  - Association state

TGbc R3.4.1:	 The 802.11bc amendment shall support modes of operation:

- in an infrastructure BSS where there is an association between the transmitter and the receiver(s)

- in cases where there is no association between transmitter(s) and receiver(s). [Ref-1 5.2.b]

TGbc R3.4.2:	 The 802.11bc amendment shall have a mechanism to facilitate power-efficient broadcasting.

TGbc R3.4.3:	 The 802.11bc amendment assumes that there is no association and/or handshake required between STA and the AP. [Ref-5]

  - Discovery

TGbc R3.5.1: The 	802.11bc amendment shall have a mechanism to facilitate discovery of 11bc-capable STAs.

TGbc R3.5.1.1:	 The 802.11bc amendment shall describe the use of an extended capability bit to indicate eBCS capability.

TGbc R3.5.2: The 	802.11bc amendment shall have a mechanism to facilitate the discovery of STAs consuming a particular broadcast service.

TGbc R3.5.2.1	The 802.11bc amendment shall describe a query/response mechanism for eBCS services.

TGbc R3.5.2.2	The 802.11bc amendment shall be capable of indicating 0, 1, or multiple eBCS services within an eBCS response frame/message.

  - Broadcast service

TGbc R3.6.1:	 Within the 802.11bc amendment, STAs should be capable of consuming multiple broadcast services simultaneously.

TGbc R3.6.1.1:	 Within the 802.11bc amendment, STAs should be capable of receiving multiple broadcast services from the same STA (i.e. non-AP STA or AP) or many STAs simultaneously.

TGbc R3.6.2:	 Within the 802.11bd amendment, 		all eBCS streams shall be treated equally.

TGbc R3.6.3:	 The 		802.11bc amendment shall provide a mechanism for a STA to signal to the AP to provide additional service information (e.g. date, time, location, RSSI) when delivering the SDU via the MAC SAP.

TGbc R3.6.4:	 The 		802.11bc amendment shall provide a mechanism for an AP to provide additional service information locally available (e.g. date, time, location, RSSI) when delivering the SDU via the MAC SAP.

  - Air time consumption

TGbc R3.7.1:	802.11bc amendment shall have a mechanism to facilitate the sharing of airtime between all broadcast services and all non-broadcast services communications.

- TGbc R3.7.2:	802.11bc amendment shall have a mechanism for starting a broadcast service.

- TGbc R3.7.3:	802.11bc amendment shall have a mechanism for stopping a broadcast service.

  - Relationship between AP and DS

TGbc R3.8.1:	At the AP, there shall be a mapping of a received frame from the DS in deciding whether or not to use eBCS for forwarding the frame towards the STA.