IEEE P802.11
Wireless LANs

| MAC CR for |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-06-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F1-17, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |
| Ross Jian Yu | Huawei |  |  | ross.yujian@huawei.com |
| Dandan Liang | Huawei |  |  | dandan.liang@huawei.com |

Abstract

This submission proposes resolutions of comments received from TGax LB238. (The proposed change is based on TGax Draft 4.2)

- 20937 21034 and 20777 (3 CIDs)

Revisions:

- Rev 0: Initial version of the document.

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGax Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGax Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGax Editor: Editing instructions preceded by “TGax Editor” are instructions to the TGax editor to modify existing material in the TGax draft.  As a result of adopting the changes, the TGax editor will execute the instructions rather than copy them to the TGax Draft.

CID 20937

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 20937 |  |  | Re CID 16139: the discussion in 18/1459r3 states "11ax allows transmitting a full BW HE MU PPDU with SIGB compression to a single user. Hence, the statement "HE-SIG-B compression is used if and only if full BW MU-MIMO is being used" is not true." | In 27.3.2.5 fix "A full bandwidth MU-MIMO transmission using the HE MU PPDU format shall have a value of 1 for the SIGB Compression field in HE-SIG-A, the Common field in HE-SIG-B is not present, the HE modulated fields of the PPDU consists of one RU that spans the entire PPDU bandwidth and the preamble is not punc- tured." (also fix the grammar!). In 27.3.10.8.2 fix "If the SIGB Compression field in the HE-SIG-A field of an HE MU PPDU is set to 1 (indicating full band- width MU-MIMO transmission)" (2x). In 27.3.10.8.5 fix "If the SIGB Compression field in the HE-SIG-A field of an HE MU PPDU is set to 1 (indicating full band- width MU-MIMO transmission)" | Revised- Agree with the comment in principle. Proposed resolution accounts for the suggested change. TGax editor please make the changes as shown in 11-19/1185 r0 under the CID 20937 |

Discussion: …

TGax Editor: Modify the following paragraph of subclause 27.3.2.5

27.3.2.5 Resource indication and User identification in an HE MU PPDU

An AP that transmits an HE MU PPDU shall set the UL/DL field in the HE-SIG-A field to 0.

A full bandwidth MU-MIMO transmission using the HE MU PPDU format shall have a value of 1 for the SIGB Compression field in HE-SIG-A, where the Common field in HE-SIG-B is not present, the HE modulated fields of the PPDU consists of one RU that spans the entire PPDU bandwidth and the preamble is not punc-tured. The number of users in the MU-MIMO group is indicated in the Number Of HE-SIG-B Symbols Or MU-MIMO Users field in HE-SIG-A. The allocated spatial streams for each user and the total number of spatial streams are indicated in the Spatial Configuration field of User field in HE-SIG-B containing the STA-ID of the designated MU-MIMO STA as defined in Table 27-30 (Spatial Configuration subfield encoding). (#20937)

TGax Editor: Modify the following paragraph of subclause 27.3.10.8.2

27.3.10.8.2 HE-SIG-B content channels

…

Page 553 Lin 1

If the SIGB Compression field in the HE-SIG-A field of an HE MU PPDU is set to 1 (indicating full bandwidth MU-MIMO or SU transmission), the Common field is not present and the HE-SIG-B content channel consists of only the User Specific field. If the SIGB Compression field in the HE-SIG-A field of an HE MU PPDU is set to 0, the Common field is present in HE-SIG-B content channel. (#20937)

TGax Editor: Modify the following paragraph of subclause 27.3.10.8.4

27.3.10.8.4 User Specific field

…

Page 561 Line 1

If the SIGB Compression field in the HE-SIG-A field of an HE MU PPDU is set to 1 (indicating full bandwidth MU-MIMO or SU transmission) and the Number Of HE-SIG-B Symbols Or MU-MIMO Users field in the HE-SIG-A field of an HE MU PPDU is set to 0 (indicating 1 user), the User Specific field in the HE-SIG-B field consists of a single User Block field containing one User field for a non-MU-MIMO allocation as shown in Table 27-28 (User field format for a non-MU-MIMO allocation). (#20937)

CID 21034 and 20777

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 21034 | 549.61 | 27.3.10.8.4 | Re CID 16308: ": the number of users multiplexed in the RUs indicated by the arrangement; for RUs with less than 106 subcarriers, there is only one user; for RUs with 106 or more subcarriers that support MU-MIMO, it indicates one user if MU-MIMO is not used and the number of users multiplexed using MU-MIMO." is not clear | Change the cited text at the referenced location to ", for RUs that support MU-MIMO (which is only ever the case for RUs with 106 or more subcarriers)" | Revised- The confusion has been partly resolved in Doc 18-1774r18 under the CID 21229 and21233. However, there is still something unclear to be fixed. Proposed resolution to make it clear TGax editor please make the changes as shown in 11-19/1185 r0 under the CID 21034 |
| 20777 | 549.57 | 27.3.10.8.4 | Re CID 16307: OK, then that xref should be given | After "indexes the size of the RUs and their placement in the frequency domain" at the referenced location add " (see Table 27-25)" | Revised- The confusion has been partly resolved in Doc 18-1774r18 under the CID 21229 and21233. However, there is still something unclear to be fixed. Proposed resolution to make it clear TGax editor please make the changes as shown in 11-19/1185 r0 under the CID 20777 |

Discussion: …

TGax Editor: Modify the Table 27-24 and paragraphs of subclause 27.3.10.8.3

27.3.10.8.3 Common field

Table 27-24—Common field

| Subfield | Number of subfields | Number of bits per subfield | Description |
| --- | --- | --- | --- |
| RU Allocation | N | 8 | N RU Allocation subfields in an HE-SIG B content channel are present, where: N = 1 if the Bandwidth field in the HE-SIG-A field is set to 0 or 1 (indicating a 20 MHz or 40 MHz HE MU PPDU) N = 2 if the Bandwidth field in the HE-SIG-A field is set to 2, 4, or 5 (indicating an 80 MHz HE MU PPDU) N = 4 if the Bandwidth field in the HE-SIG-A field is set to 3, 6, or 7 (indicates a 160 MHz or 80+80 MHz HE MU PPDU) Each RU Allocation subfield in an HE-SIG-B content channel corresponding to a 20 MHz frequency segment indicates the RU assignment, including the size of the RU(s) and their placement in the frequency domain, to be used in the HE modulated fields of the HE MU PPDU in the frequency domain, also indicates the information on the number of users allocated to each RU, where the subcarrier indices of the RU(s) meet the conditions in Table 27-25 . (#CID 21034 and 20777) |

…

For the first or only RU Allocation subfield in an HE-SIG-B content channel that refers to an RU of size greater than or equal to 106-tone, the RU Allocation subfield encodes the number of User fields per RU contributed to the User Specific field in the same HE-SIG-B content channel as the RU Allocation subfield. This number is labeled Nuser(r, c) for RU r and HE-SIG-B content channel c.

For the second RU Allocation subfield, if present, in an HE-SIG-B content channel that refers to an RU of size equal to 996-tone, the RU Allocation subfield encodes zero additional User fields per RU contributed to the User Specific field in the same HE-SIG-B content channel as the RU Allocation subfield.

The number of users allocated to RU r referred to by RU Allocation subfield except for broadcast RU and unallocated RU is determined from the RU size and Nuser(r, c) as follows:

— If RU r is a 26-tone or 52-tone RU, then one user is allocated to the RU

— If RU r is a 106-tone or 242-tone RU, then the number of users allocated to the RU is Nuser(r, c).

— If RU r is a 484-tone or larger RU, then the number of users allocated to the RU equals the number of User fields for the RU summed across both HE-SIG-B content channels, i.e., Nuser(r, 1) + Nuser(r, 2).

If RU r referred to by the RU Allocation subfield is a broadcast RU, then the users allocated to the RU are all the STAs except for a STA that is identified as the recipient of another RU in the same HE MU PPDU.  If RU r referred to by the RU Allocation subfield is an unallocated RU, then the number of users allocated to the RU is zero.  (#CID 21034 and 20777)

TGax Editor: Replace the Table 27-25 of subclause 27.3.10.8.3 with the following table

Table 27-25—RUs associated with each RU Allocation subfield for each HE-SIG-B content channel and PPDU bandwidth (#CID 21034 and 20777)

| PPDU bandwidth | RU Allocation subfield and Center 26-tone RU subfield (if present) | RUs in the subcarrier range, or overlapping with the subcarrier range if the RU is larger than a 242-tone RU |
| --- | --- | --- |
| 20 MHz | The RU Allocation subfield in a single HE SIG-B content channel | [–122: 122] |
| 40MHz | The RU Allocation subfield in HE-SIG-B content channel 2 | [3:244] |
|  | The RU Allocation subfield in HE-SIG-B content channel 1 | [–244: –3] |
| 80 MHz | The second RU Allocation subfield in HE-SIG-B content channel 2 | [259:500] |
|  | The second RU Allocation subfield in HE-SIG-B content channel 1 | [17:258] |
|  | The first RU Allocation subfield in HE-SIG-B content channel 2 | [–258:–17] |
|  | The first RU Allocation subfield in HE-SIG-B content channel 1 | [–500:–259] |
|  | Center 26-tone RU subfield in HE-SIG-B content channel 1 and 2 | [–16:–4, 4:16] |
| 160 MHz or 80+80 MHz | The fourth RU Allocation subfield in HE-SIG-B content channel 2 | [771:1012] |
|  | The fourth RU Allocation subfield in HE-SIG-B content channel 1 | [529:770] |
|  | The third RU Allocation subfield in HE-SIG-B content channel 2 | [254:495] |
|  | The third RU Allocation subfield in HE-SIG-B content channel 1 | [12:253] |
|  | The second RU Allocation subfield in HE-SIG-B content channel 2 | [–253:–12] |
|  | The second RU Allocation subfield in HE-SIG-B content channel 1 | [–495:–254] |
|  | The first RU Allocation subfield in HE-SIG-B content channel 2 | [–770:–529] |
|  | The first RU Allocation subfield in HE-SIG-B content channel 1 | [–1012:–771] |
|  | Center 26-tone RU subfield for higher frequency 80 MHz in HE-SIG-B content channel 2 | [496:508, 16:528] |
|  | Center 26-tone RU subfield for lower frequency 80 MHz in HE-SIG-B content channel 1 | [–528:–516,–508:–496] |