IEEE P802.11
Wireless LANs

| CIDs 2537 – 2539 proposed resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-08-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes resolutions to REVmd D2.0 for the following CIDs:

2537, 2538 and 2539 regarding “AdvertisementProtocolID and Query”

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 2537 | 6.3.71 | 580 | 32 | "Query string formatted using protocol identified in AdvertisementProtocolID." -- but that doesn't work if AdvertisementProtocolID is a list (with >1 member) | Delete "or Sequence of integers" in the tables in 6.3.71.2.2 and 6.3.71.4.2 |

Discussion

This appears to be an error that has existed for several years within the IEEE 802.11 standard. It’s not possible to format a suitable Query string if there is more than one AdvertisementProtocolID.

Proposed Resolution

Accept

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 2538 | 6.3.71 | 580 | 32 | "E.g., if the AdvertisementProtocolID value is 0, then Query is formatted according to the Query List ANQP- element (see 9.4.5.2 (Query List ANQP- element))(#1240). " -- we really don't need an example on how to look up a row in a table! | Delete the cited text in 6.3.71.2/3/4/5.2 |

Discussion

The comment is correct, as this example is not required.

Proposed Resolution

Accept

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 2539 | 6.3.71 | 580 | 32 | "formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted according to the Query List ANQP-element (see 9.4.5.2 (Query List ANQP-element))(#1240). " -- there is no AdvertisementProtocolID or Query in the primitve | Delete the cited text in 6.3.71.3/5.2 |

Discussion

The proposed change cites 6.3.71.3 MLME-GAS.confirm and 6.3.71.5 MLME-GAS.response. Within these primitives, AdvertisementProtocolID or Query are not locally defined. References to their definitions should be added and indeed small changes to 6.3.71.2 and 6.3.71.4 are also required for consistency.

Also see the resolution to CID 2538.

Proposed Resolution

Revised:

Clause 6.3.71.2.2, page 581, line 21 replace

“Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the

AdvertisementProtocolID value is 0, then Query is formatted according to the Query List ANQP-element (see 9.4.5.2 (Query List ANQPelement)).”

with

“GAS Query Request (see 9.6.7.12) formatted using the protocol identified in AdvertisementProtocolID.”

Clause 6.3.71.3.2, page 583, line 3 replace

“Query Response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted according to the

Query List ANQP-element (see 9.4.5.2 (Query List ANQP-element).”

with

“GAS Query Response (see 9.6.7.13) formatted using the protocol identified in the AdvertisementProtocolID parameter of the MLME-GAS.request primitive (see 6.3.71.2.2).”

Clause 6.3.71.4.2 page 584, line 39, replace

“Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the

AdvertisementProtocolID value is 0, then Query is formatted according to the Query List ANQP-element (see 9.4.5.2 (Query List ANQPelement))”

With

“GAS Query Request (see 9.6.7.12) formatted using the protocol identified in AdvertisementProtocolID.”

Clause 6.3.71.5.2 page 586, line 17, replace

“Query Response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted according to the Query List ANQP-element (see 9.4.5.2 (Query List ANQPelement))”

With

“GAS Query Response (see 9.6.7.13) formatted using the protocol identified in the AdvertisementProtocolID parameter of the MLME-GAS.indication primitive (see 6.3.71.4.2).”