IEEE P802.11
Wireless LANs

| Text clarification on the “ISTA2RSTA LMR Feedback Policy” bit in the Extended Capabilities” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-08-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Qi Wang | Apple, Inc. |  |  | qi_wang2@apple.com |

Abstract

This submission proposes the text to clarify that the “ISTA2RSTA LMR Feedback Policy” bit in the Extended Capabilities applies only to an RSTA and is reserved for an ISTA.

Introduction

Introduction

This submission proposes the text to clarify that the “ISTA2RSTA LMR Feedback Policy” bit in the Extended Capabilities applies only to an RSTA and is reserved for an ISTA.

Proposed 11az spec modification:

Instruction to the editor: Change the text as shown below.  The proposed changes are in reference to the text in IEEE 802.11az_D1.4, and are indicated by the change marks as follows:

Editor Note: Please change the last row of Table 9-153 of 11az_D1.4 as shown below:

9.4.2.26 Extended Capabilities element

| Bits | Information | Notes |
| --- | --- | --- |
| … | … | … |
| <ANA> | ISTA2RSTA LMR Feedback Policy | A STA sets the ISTA2RSTA LMR Feedback Policy field to 1 if dot 11 ISTA2RSTALMR FeedbackPolicy is true. Otherwise the STA sets the ISTA2RSTA LMR Feedback Policy field to 0. See 11.22.6.3.3 (Trigger-based and non-Trigger-based Ranging Measurement Negotiation) The ISTA2RSTA LMR Feedback Policy field indicates the policy of the STA in the role of an RSTA, and it is reserved for a STA in the role of an ISTA. |

Editor Note: Please change Line27/Page 110 and Line33/Page 110 of 11.2.6.3.3 of 11az_D1.4 as shown below:

11.22.6.3.3 Trigger-based and non-Trigger-based Ranging Measurement Negotiation

…

If the ISTA2RSTA LMR Feedback subfield in the Initial Fine Timing Measurement Request frame is set to 0, and the ISTA2RSTA LMR Feedback Policy field in the Extended Capabilities element is set to 1 by the RSTA, then the RSTA shall not reject the request because the ISTA2RSTA LMR Feedback subfield was set to 0, and shall set the ISTA2RSTA LMR Feedback in the Initial Fine Timing Measurement frame to 0.

If the ISTA2RSTA LMR Feedback subfield in the Initial Fine Timing Measurement Request frame is set to 0, and the ISTA2RSTA LMR Feedback Policy field in the Extended Capabilities element is set to 0 by the RSTA, then the RSTA shall set the ISTA2RSTA LMR Feedback subfield to 1 to indicate it requests the ISTA to transmit the ISTA2RSTA LMR or to 0 otherwise. In the former case, the ISTA may either proceed with the ranging operation or terminate the FTM session using the procedure described in Subclause 11.22.6.6.

References:

1. IEEE Draft P802.11az_D1.4, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications, Amendment 8: Enhancements for locationing