IEEE P802.11
Wireless LANs

| MAC CR MU cascading |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-09-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F3-06, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |

Abstract

This submission proposes resolutions of comments received from TGax LB238. (The proposed change is based on TGax Draft 4.0)

- 20732 20733 21450 (3 CIDs)

Revisions:

- Rev 0: Initial version of the document.

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGax Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGax Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGax Editor: Editing instructions preceded by “TGax Editor” are instructions to the TGax editor to modify existing material in the TGax draft.  As a result of adopting the changes, the TGax editor will execute the instructions rather than copy them to the TGax Draft.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 20732 | 341.40 | 26.5.4 | Re CID 16076: the comment was valid. The normative requirement is that cascading is defined by the use of "an A-MPDU to a non-AP STA that includes an Ack or BlockAck frame together with a Trigger frame or a frame carrying a TRS Control subfield" (342.33) | Change the first sentence of the referenced subclause to "An MU cascading sequence is a frame exchange sequence between an AP and a non-AP STA where at least one transmission by the AP is a triggering PPDU that includes an acknowledgment." | Rejected- After the discussion in the TGax group, the group agreed that the current of the definition of MU cascading sequence is clear. |
| 20733 | 341.40 | 26.5.4 | Re CID 16076: the comment was valid. The normative requirement is that cascading is defined by the use of "an A-MPDU to a non-AP STA that includes an Ack or BlockAck frame together with a Trigger frame or a frame carrying a TRS Control subfield" (342.33). The resolution suggests this is wrong | Change the first sentence of the referenced subclause to "An MU cascading sequence is a frame exchange sequence between an AP and a non-AP STA where a triggering PPDU follows after SIFS an UL MU transmission." At 342.33 change "An AP shall not transmit an A-MPDU to a non-AP STA that includes an Ack or BlockAck frame together with a Trigger frame or a frame carrying a TRS Control subfield unless" to "An AP shall not transmit a triggering PPDU a SIFS after an HE TB PPDU the AP has triggered unless" | Rejected- After the discussion in the TGax group, the group agreed that the current of the definition of MU cascading sequence is clear. |
| 21450 | 341.40 | 26.5.4 | "It says ""An MU cascading sequence is a frame exchange sequence between an AP and one or more non-AP STAs carried in an HE MU PPDU in the downlink and HE TB PPDU in the uplink and characterized by the exchange of Control, Data and/or Management frames in both directions."" Is "Control" frame in the above a must? From my understanding of the cascading sequence, there are both UL and DL exchange of Data and/or Management frames. For acknowledging, as a result, there may be Control frames in both sides. But that is not essential from the definition point of view. | Delete "Control, " from the cited sentence. Add a note as follows, if necessary. "NOTE - There are Trigger frames in downlink and Control frames such as Ack or BlockAck frames may be present in downlink and/or uplink accordingly to achieve exchanging Data/Management frames in both directions." | Rejected- Control frame is one kind of frame for exchange in MU cascading. Besides Trigger, ACK and Block Ack frames, there are other control frames, such as PS-Poll. |

Discussion: …