IEEE P802.11
Wireless LANs

| MAC CR Fragment Flush |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-09-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F3-06, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |

Abstract

This submission proposes resolutions of comments received from TGax LB238. (The proposed change is based on TGax Draft 4.0)

- 20176 20177 21050 21587 21588 (5 CIDs)

Revisions:

- Rev 0: Initial version of the document.

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGax Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGax Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGax Editor: Editing instructions preceded by “TGax Editor” are instructions to the TGax editor to modify existing material in the TGax draft.  As a result of adopting the changes, the TGax editor will execute the instructions rather than copy them to the TGax Draft.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 20176 | 310.00 | 26.3.3.3 | W.r.t. dynamic defragmentation, it is mentioned that a recipient STA shall discard incomplete fragments when receiving a BlockAckReq to move the BA window. When the STA receives a DELBA to tear down the BA agreement, the STA should/shall do the same. Furthermore, an explicit mechanism to request the recipient STA to discard all pending fragments in current BA window should be introduced for various benefits. | 1) add texts to state: the recipient STA shall discard incomplete fragments when receiving a DELBA request. 2) Add explicit 'discard' request. E.g. using the proposal described in doc 11-18-0218-03-00ax-fragment-flushing-blockackreq. | Rejected- The TGax group failed to reach consensus after full discussion. Moreover, there is no necessity to add Fragment Flushing mechanism because the existing Block Ack Request already provides a flushing mechanism. |
| 20177 | 311.00 | 26.3.3.4 | Same as above | Same as above | Rejected- The TGax group failed to reach consensus after full discussion. Moreover, there is no necessity to add Fragment Flushing mechanism because the existing Block Ack Request already provides a flushing mechanism. |
| 21050 | 305.47 | 26.3.1 | Need a mechanism to allow the transmitter of fragments to re-partition an MSDU for which some fragments have been transmitted. This requires a fragment flush command. | Add a mechanism to allow the transmitter of fragments to re-partition an MSDU by creating the ability to signal a fragment flush command to its recipient STA. | Rejected- The TGax group failed to reach consensus after full discussion. Moreover, there is no necessity to add Fragment Flushing mechanism because the existing Block Ack Request already provides a flushing mechanism. |
| 21587 | 310.00 | 26.3.3.3 | W.r.t. dynamic defragmentation, it is mentioned that a recipient STA shall discard incomplete fragments when receiving a BlockAckReq to move the BA window. When the STA receives a DELBA to tear down the BA agreement, the STA should/shall do the same. Furthermore, an explicit mechanism to request the recipient STA to discard all pending fragments in current BA window should be introduced for various benefits. | 1) add texts to state: the recipient STA shall discard incomplete fragments when receiving a DELBA request. 2) Add explicit 'discard' request. E.g. using the proposal described in doc 11-18-0218-03-00ax-fragment-flushing-blockackreq. | Rejected- The TGax group failed to reach consensus after full discussion. Moreover, there is no necessity to add Fragment Flushing mechanism because the existing Block Ack Request already provides a flushing mechanism. |
| 21588 | 311.00 | 26.3.3.4 | Same as above | Same as above | Rejected- The TGax group failed to reach consensus after full discussion. Moreover, there is no necessity to add Fragment Flushing mechanism because the existing Block Ack Request already provides a flushing mechanism. |

Discussion: …