IEEE P802.11
Wireless LANs

| SFD Proposal for R3.6.3 & 3.6.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-10-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Abhishek Patil | Qualcomm Inc |  |  | appatil@qti.qualcomm.com |
| George Cherian |  |  |  |  |
| Alfred Asterjadhi |  |  |  |  |
| Duncan Ho |  |  |  |  |

Abstract

This document proposes draft text eBCS SFD text for R3.6.3 and R3.6.4.

TGbc R3.6.3:			802.11bc amendment shall provide a mechanism for a STA to signal to the AP to provide additional service information (e.g. date, time, location, RSSI) when delivering the SDU via the MAC SAP.

TGbc R3.6.4:			802.11bc amendment shall provide a mechanism for an AP to provide additional service information locally available (e.g. date, time, location, RSSI) when delivering the SDU via the MAC SAP.

9 Frame Formats

- Beacon frame format

| - Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA> | E-BCS Capabilities | This element is optionally present if dot11eBCSSupportImplemented is true. |

- Probe Response frame format

| - Probe Response frame body |  |  |  |
| --- | --- | --- | --- |
| Order | Information | Notes |  |
| <ANA> | E-BCS Capabilities | This element is optionally present if dot11eBCSSupportImplemented is true. |  |

## 9.4.2 Elements

#### 9.4.2.1 General

Insert the following new rows into Table 9-77 (Element IDs) (header row shown for convenience):

Table 9-77 -- Element IDs

| Element | Element ID | Element ID Extension | Extensible |
| --- | --- | --- | --- |
| E-BCS Capabilities | 255 | <ANA> | Yes |
| E-BCS Container | 255 | <ANA> | Yes |

Insert the following subclauses:

9.4.2.<ANA> E-BCS Capabilities

An eBCS STA advertises its capabilities in this element

The element carries:

- A bit field indicating if the transmitting (AP) STA supports forwarding to a remote server

- A bit field indicating if the transmitting (AP) STA supports embedding information (such as location, date, time) when sending a frame to a remote server

- A bit field indicating if transmitting (non-AP) STA requires embedding information (such as location, date, time) when sending a frame to a remote server

- Other fields TBD

9.4.2.<ANA> E-BCS Container

The element carries:

- A field for carrying higher layer data

- Other fields TBD

9.6 Action frame format details

9.6.7 Public Action details

9.6.7.1 Public Action frames

Add E-BCS UL Table 9-363.

9.6.7.<ANA5> E-BCS UL frame format

This GAS frame includes:

- Destination URI element (9.4.2.89)

  - Carries the URI to the destination server

- E-BCS Capabilities element (Optionally present)

  - Provides capabilities of the transmitting non-AP STA

- E-BCS Container

  - Carries the data to be forwarded to the remote server

# 11 MLME

Insert the following subclauses:

### 11.<ANA> eBCS procedures

11.<ANA>.<ANA> eBCS UL Service

The eBCS UL Service procedure allow a non-AP STA to transmit an UL frame with the expectation that one or more eBCS AP(s) in the neighborhood forward the contents of the frame to a remote destination specified in the frame.

The eBCS spec shall provide a mode of operation that prevents unauthenticated unsigned packet to be forwarded to the remote destination.

An eBCS AP may enforce rate control to prevent unlimited ULs to a remote destination

Describe the procedure in detail – including discussion on eBCS AP / non-AP STA capabilities and actions on eBCS AP if a STA requests embedding of information.