IEEE P802.11
Wireless LANs

| MAC CR on Fragmentation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-11-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F3-06, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |

Abstract

This submission proposes resolutions of comments received from TGax LB244. (The proposed change is based on TGax Draft 5.0)

- 22064  22274

Revisions:

- Rev 0: Initial version of the document.

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGax Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGax Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGax Editor: Editing instructions preceded by “TGax Editor” are instructions to the TGax editor to modify existing material in the TGax draft.  As a result of adopting the changes, the TGax editor will execute the instructions rather than copy them to the TGax Draft.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 22064 | 320.16 | 26.3.2.2 | The 2 bullets miss a condition when the recipient indicates that it is capable of level 2 or 3, and in the BA agreement explicitly/implictly neogotiated level 2 or 3. Under the Note in P318.37, the originator is allowed to send level 1 dynamic fragements in this case. | add the missing condition | Revised- Agree with the commenter. Proposed resolution accounts for the suggested change. TGax editor please make the changes as shown in 11-19/1947r0 under the CID 22064 |
| 22274 |  | 10.4 | The baseline says "A fragment is an MPDU, the Frame Body field of which carries (#1452)only a portion of an MSDU or MMPDU." However, in HE a fragment can carry a portion of an A-MSDU, which might be more than a portion of an MSDU | As it says in the comment | Revised- Agree with the commenter. Proposed resolution accounts for the suggested change. TGax editor please make the changes as shown in 11-19/1947r0 under the CID 22274 |

Discussion: …

TGax editor, please change the subclause 26.3.2.2:

26.3.2.2 Level 1 dynamic fragmentation

An originator STA may transmit one dynamic fragment of an MSDU, A-MSDU (if supported by the recipi-ent) or MMPDU that is a non-A-MPDU and that is not sent under a block ack agreement to a recipient STA using level 1 dynamic fragmentation if the Dynamic Fragmentation Support subfield in the HE MAC Capa-bilities Information field of the HE Capabilities element received from the recipient STA is 1, 2 or 3.

An originator STA may transmit a fragmented MSDU or A-MSDU (if supported by the recipient) under a block ack agreement to a recipient STA using level 1 dynamic fragmentation provided one of the following conditions is met:

— The Dynamic Fragmentation Support subfield in the HE Capabilities element received from the recipient STA is 1, 2 or 3, and for the block ack agreement associated with the TID of the MSDU or A-MSDU, the HE Fragmentation Operation subfield is 1 if the ADDBA Extension element is present in the ADDBA Response frame received from the recipient STA.

. (#CID 22064)

10.4 MSDU, A-MSDU, and MMPDU fragmentation

Change the 4th paragraph as follows:

A fragment is an MPDU, the Frame Body field of which carries (#1452) only a portion of an MSDU, A-MSDU (#CID 22274) or MMPDU. When data are to be transmitted, the number of octets in the fragment (before processing by the security mechanism) shall be limited by dot11FragmentationThreshold and the number of octets in the MPDU that have yet to be assigned to a fragment at the instant the fragment is constructed for the first time. Once a fragment is transmitted for the first time, its frame body content and length shall be fixed until it is successfully delivered to the immediate receiving STA.