IEEE P802.11
Wireless LANs

| Proposed resolution to LB240 CID - 2291 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Qi Wang | Apple, Inc. |  |  | qi_wang2@apple.com |

Abstract

This submission contains a proposal to resolve 11az LB#240 CID-2291.

The reference document is 11az_D1.5.

Introduction

Introduction

This submission contains a proposal to resolve 11az LB#240 CID-2291.

The reference document is 11az_D1.5.

| Comments: CID; Page/Line; Clause; Comment; Proposed change; Resolution. 2291; 89/28; 11.22.6.3.4; "When an RSTA has set the Secure LTF Support field to 1 in the Extended Capabilities element it transmits, an ISTA with dot11SecureLTFImplemented equal to true may set the Secure LTF Support subfield in the Ranging Parameters field in an initial Fine Timing Measurement Request frame to 1 to activate a secure LTF measurement exchange mode between the ISTA and the RSTA." Please add text to specify that when the iSTA activates a secure LTF measurements, the initial FTM Request frame and the initial FTM frames shall be protected using PMF.; Please add text to specify that when the iSTA activates a secure LTF measurements, the initial FTM Request frame and the initial FTM frames shall be protected using PMF.; Revised. This comment is already resolved and incorporated in 11az_D1.5 (Line 15-19, page 112, see below under “Discussion”), which specifies that the FTM request and response frames used to establish a secure ranging session shall use protected Dual of Public Action frame format. TGaz editor: no further text changes required.. |  |
| --- | --- |

Discussion:

11az_D1.5 (line 15-19/page 112 in 11az_D1.5) already specifies that the ranging request frame and the corresponding response frame that are used to establish a secure ranging session shall use the protected Dual of Public Action frame format, as shown below:

11.22.6.3 Fine timing measurement procedure negotiation

11.22.6.3.1 General

…

A Secure Fine Timing Measurement Session is established when an ISTA and a RSTA establish a security context and use it to exchange the initial Fine Timing Measurement Request frame and the corresponding initial Fine Timing Measurement frame in the Protected Dual of Public Action frame format (see Cl. 9.6.10 Protect Dual of Public Action frames) and the negotiation completes successfully.

---

References:

1. IEEE Draft P802.11az_D1.5, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications, Amendment 8: Enhancements for locationing