IEEE P802.11
Wireless LANs

| Additional SFD text for requirements R3.6.3 & 3.6.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-01-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Abhishek Patil | Qualcomm Inc |  |  | appatil@qti.qualcomm.com |
| George Cherian |  |  |  |  |
| Jouni Malinen |  |  |  |  |
| Alfred Asterjadhi |  |  |  |  |

Abstract

This document proposes updates to the draft text in the eBCS SFD document (11-19/1429r2) for requirements R3.6.3 and R3.6.4. The contribution builds on top of document 11-19/1801r6.

- Rev0 – initial version

- Rev1 – revised based on feedback from the group when the document was presented Tuesday PM2 (1/14/20).

  - Remove E-BCS Container element and integrated the contents it into the E-BCS UL frame as a field

    - Additional fields were added based on additional (offline) feedback

  - Mandate source authentication at AP for all UL frames

  - Separately advertise AP’s capability for each embedding service

  - General text updates to align with the above changes

TGbc editor, please update the SFD document to incorporate the addition and changes as shown in this document

9 Frame Formats

- Beacon frame format

Insert the following new row into Table 9-34 (Beacon frame body) in numeric order

| - Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA> | E-BCS UL Capabilities | This element is optionally present if dot11eBCSSupportImplemented is true. |

- Probe Response frame format

Insert the following new row into Table 9-41 (Probe Response frame body) in numeric order

| - Probe Response frame body |  |  |  |
| --- | --- | --- | --- |
| Order | Information | Notes |  |
| <ANA> | E-BCS UL Capabilities | This element is optionally present within a broadcast Probe Response frame if dot11eBCSSupportImplemented is true. |  |

## 9.4.2 Elements

#### 9.4.2.1 General

Insert the following new rows into Table 9-77 (Element IDs) in numeric order:

Table 9-77 - Element IDs

| Element | Element ID | Element ID Extension | Extensible | Fragmentable |
| --- | --- | --- | --- | --- |
| E-BCS UL Capabilities | 255 | <ANA> | Yes | No |

Insert the following new subclauses:

9.4.2.X E-BCS UL Capabilities element

9.4.2.X.1 General

The E-BCS UL Capabilities element contains fields that are used to advertise the capabilities of an eBCS  STA related to forwarding service to a remote server.

An eBCS AP declares support for forwarding service and related capabilities by including the E-BCS UL Capabilities element in a Beacon and broadcast Probe Response frame.

An eBCS non-AP STA can include the E-BCS UL Capabilities element in an E-BCS UL frame if it intends to provide its capabilities to an AP that forwards its data to a remote server.

|  | Element ID | Length | Element ID Extension | E-BCS UL Capabilities |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 9-<BC-1> - E-BCS UL Capabilities element format |  |  |  |  |

The format of E-BCS UL Capabilities element is shown in Figure 9-<BC-1> (E-BCS UL Capabilities element format).

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1 (General).

The contents of an E-BCS UL Capabilities field is defined in 9.4.2.X.2 when the element is transmitted by an eBCS AP and defined in 9.4.2.X.3 when the element is transmitted by an eBCS non-AP STA.

9.4.2.<ANA>.2 E-BCS AP UL Capabilities

The format of an E-BCS UL Capabilities field when transmitted by an eBCS AP is shown in Figure 9-<BC-2> (Format of E-BCS UL Capabilities field for an AP).

|  | UL AP Control |  |  |
| --- | --- | --- | --- |
| Octet: | 2 |  |  |
|  |  | 9-<BC-2> - Format of E-BCS UL Capabilities field for an AP |  |

The format of UL AP Control is shown in Figure 9-<BC-3> (UL AP Control field format).

|  |  | B0 B1 | B2 B3 | B4 | B5 |  | B6 | B7 B15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Authentication Mode | Limiting Mode | Location Embedding Supported |  | Data-Time Embedding Supported | IP Address Embedding Supported | Reserved |  |
| Bits: |  | 2 | 2 | 1 | 1 |  | 1 | 9 |  |
|  | 9-<BC-3> - UL AP Control field format |  |  |  |  |  |  |  |  |

The encoding of Authentication Mode subfield is shown in Table 9-<BC-4> (Encoding of Authentication Mode subfield).

| Table 9-<BC-4> - Encoding of Authentication Mode subfield |  |  |
| --- | --- | --- |
| Subfield value | Definition | Encoding |
| 0 | Reserved |  |
| 1 | Per Destination | AP forwards contents of an E-BCS UL frame only if it is able to successfully able to authenticate the transmitter of the frame based on an established relationship with the remote server identified in the frame. |
| 2 – 3 | Reserved |  |

The encoding of Limiting Mode subfield is shown in Table 9-<BC-5> (Encoding of Limiting Mode subfield).

| Table 9-<BC-5> - Encoding of Limiting Mode subfield |  |  |  |
| --- | --- | --- | --- |
| Subfield value | Definition | Encoding |  |
| 0 | No Throttling | AP applies no restrictions on the amount/frequency of ULs from a non-AP STA destined to a remote server. |  |
| 1 | Per Destination | AP applies forwarding limits as specified by the remote server with whom it has established a relationship. |  |
| 2 – 3 | Reserved |  |  |

Location Embedding Supported subfield is set to 1 if the AP supports embedding of location information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an E-BCS UL frame to the remote server. Otherwise, the subfield is set to 0.

Date-Time Embedding Supported subfield is set to 1 if the AP supports embedding of date and time information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an E-BCS UL frame to the remote server. Otherwise, the subfield is set to 0.

IP Address Embedding Supported subfield is set to 1 if the AP supports embedding of IP address information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an E-BCS UL frame to the remote server. Otherwise, the subfield is set to 0.

Additional fields related to security/authentication and throttling TBD.

9.4.2.X.3 E-BCS non-AP UL Capabilities

The format of an E-BCS UL Capabilities field when transmitted by an eBCS non-AP STA is shown in Figure 9-<BC-6> (Format of E-BCS UL Capabilities field for a non-AP STA).

|  | UL Non-AP STA Control |  |
| --- | --- | --- |
| Octets: | 1 |  |
| 9-<BC-6> - Format of E-BCS UL Capabilities field for a non-AP STA |  |  |

The format of UL Non-AP STA Control is shown in Figure 9-<BC-7> (UL Non-AP STA Control field format).

|  | B0 |  | B1 | B2 | B3 B7 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Location Embedding Requested |  | Date-Time Embedding Requested | IP Address Embedding Requested | Reserved |  |
| Bits: | 1 |  | 1 | 1 | 5 |  |
|  |  | 9-<BC-7> - UL Non-AP STA Control field format |  |  |  |  |

The Security Information Present subfield is set to 1 to indicate that a field carrying Security Information is present in the element. Otherwise the subfield is set to 0.

The Location Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append location information before forwarding. Otherwise the subfield is set to 0.

The Date-Time Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append date and time information before forwarding. Otherwise the subfield is set to 0.

The IP Address Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append AP’s IP address information before forwarding. Otherwise the subfield is set to 0.

9.6 Action frame format details

9.6.7 Public Action details

9.6.7.1 Public Action frames

Insert the following new row into Table 9-363 (Public Action field values) in numeric order:

| Table 9-363 – Public Action field values |  |  |
| --- | --- | --- |
| Public Action field value | Description |  |
| <ANA> | E-BCS UL |  |

Insert the following new subclause:

9.6.7.X E-BCS UL frame format

The E-BCS UL frame is transmitted by an eBCS non-AP STA carrying data intended for a remote destination identified in the frame. The format of E-BCS UL frame Action field is shown in Figure 9-<BC> (E-BCS UL frame Action field format).

|  | Category | Public Action | HLP Payload Length | HLP Payload | Payload Signature Length | Payload Signature | Certificate Length | STA Certificate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | variable | 2 | variable | 2 | variable |
|  | Timestamp Length | Timestamp | Timestamp Signature Length | Timestamp Signature | Destination URI | E-BCS UL Capabilities | Frame Signature Length | Frame Signature |
| Octets: | 1 | variable | 2 | variable | variable | 0 or 4 | 2 | variable |
| Figure 9-<BC> - E-BCS UL frame Action field format |  |  |  |  |  |  |  |  |

The Category field is defined in 9.4.1.11 (Action field).

The Public Action field is defined in 9.6.7.1 (Public Action frames).

The HLP Payload Length indicates the length of the HLP Payload field

The HLP Payload field carries the higher layer protocol (HLP) payload.

The Signature Length field indicates the length of the Payload Signature field.

The Payload Signature field carries the Signature of the HLP payload.

The Certificate Length field indicates the length of the STA Certificate field

The STA Certificate field carries the certificate of the transmitting STA.

The Timestamp Length field indicates the length of the Timestamp field.

The Timestamp field carries timing information to provide protection against replay attack.

The Timestamp Signature Length field indicated the length of the Timestamp Signature field.

The Timestamp Signature field carries signature of the Timestamp field.

The Destination URI element is defined in 9.4.2.89 (Destination URI element) and carries the address of the remote server.

The E-BCS UL Capabilities element is defined in 9.4.2.X (E-BCS UL Capabilities element). The element is optionally present and carries the capabilities of the transmitting non-AP STA.

The Frame Signature Length field indicated the length of the Frame Signature field.

The Frame Signature field carries signature for the contents of the E-BCS UL frame Action field except the Frame Signature field

# 11 MLME

Insert the following subclause:

### 11.X eBCS procedures

11.X.Y E-BCS UL Service

11.X.Y.1 General

The eBCS UL Service procedure allow a non-AP STA to transmit an UL frame with the expectation that one or more eBCS APs in the neighborhood might forward the contents of the frame to a remote destination specified in the frame. An eBCS non-AP STA may include a request to the forwarding AP to append additional information before forwarding to the remote server. The forwarding service is best effort with no guarantee that the contents are delivered to the remote server identified in the STA’s UL frame. Furthermore, a STAs request to embed metadata may not be fulfilled by a forwarding AP.

11.X.Y.2 E-BCS UL operation at an eBCS AP

An eBCS AP may supporting forwarding the contents of an E-BCS UL frame received from an eBCS non-AP STA to a remote server identified in the frame. An eBCS AP that supports forwarding service shall include E-BCS UL Capabilities element in the Beacon and broadcast Probe Response frames that it transmits.

An eBCS AP may support appending metadata (such as location, date/time, IP address), based a non-AP STA’s request, before forwarding the payload to the remote server identified in the STA’s E-BCS UL frame. An AP shall indicate its ability to support embedding by setting the appropriate subfield in the E-BCS UL Capabilities element to 1. An eBCS AP that is capable of embedding the requested metadata, when requested by an eBCS non-AP STA, shall append the requested metadata to the content being forwarded to the remote server. Format TBD.

An eBCS AP shall authenticate the transmitter of the packet before forwarding it to a remote server and shall provide an indication of the authentication scheme in the E-BCS Capabilities element that it transmits.

An eBCS AP may limit the amount/frequency of ULs it forwards to a remote server, shall provide an indication of the throttling scheme in the E-BCS UL Capabilities element that it transmits.

NOTE – Forwarding service is best effort and an eBCS AP, that supports forwarding service, is not required to forward a STA’s data to the destination server identified in the STA’s E-BCS UL frame if the conditions indicated by the AP are not satisfied.

11.X.Y.3 E-BCS UL operation at an eBCS non-AP

An eBCS non-AP STA may transmit an E-BCS UL frame carrying data destined for a remote server. The STA’s frame shall carry necessary fields containing security credentials necessary for authenticating the STA and protecting the contents of the frame.

An eBCS non-AP STA may include E-BCS Capabilities element to indicate its capabilities and request, if any, to a forwarding eBCS AP.

An eBCS non-AP STA may request a forwarding AP to append metadata (such as location, date/time, IP address) in the data being forwarded to the server. A forwarding eBCS AP, when capable, shall embed the requested information.

An eBCS non-AP STA may monitor the WM to gather capabilities of nearby eBCS APs and may conform to the requirements indicated by neighboring eBCS AP(s) that support forwarding service. An eBCS non-AP STA may transmit an E-BCS UL frame without obtaining any information about nearby eBCS AP(s).

NOTE – Forwarding service is best effort and an eBCS non-AP STA is not required to scan or conform to the capabilities of a neighbor eBCS AP that supports forwarding service.