IEEE P802.11
Wireless LANs

| Additional SFD text for requirements R3.6.3 & 3.6.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-01-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Abhishek Patil | Qualcomm Inc |  |  | appatil@qti.qualcomm.com |
| George Cherian |  |  |  |  |
| Jouni Malinen |  |  |  |  |
| Alfred Asterjadhi |  |  |  |  |

Abstract

This document proposes updates to the draft text in the eBCS SFD document (11-19/1429r2) for requirements R3.6.3 and R3.6.4. The contribution builds on top of document 11-19/1801r6.

- Rev0 – initial version

- Rev1 – revised based on feedback from the group when the document was presented during the Tuesday PM2 (1/14/20) session.

  - Remove E-BCS Container element and integrated the contents it into the E-BCS UL frame as a field

    - Additional fields were added based on additional (offline) feedback

  - Mandate source authentication at AP for all UL frames

  - Separately advertise AP’s capability for each embedding service

  - General text updates to align with the above changes

- Rev 2 – additional revisions based on feedback received when the doc was presented during the Thursday PM2 (1/16/20) session.

- Rev 3 – Updated the description for Timestamp field in the E-BCS UL frame based on feedback from Jouni, Morioka-san and Antonio

  - Several editorial updates throughout the document.

- Rev 4: Updated the description to Time subfield of Timestamp in clause 11.

TGbc editor, please update the SFD document to incorporate the addition and changes as shown in this document

9 Frame Formats

- Beacon frame format

Insert the following new row into Table 9-34 (Beacon frame body) in numeric order

| - Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA> | E-BCS UL Capabilities | This element is optionally present if dot11eBCSSupportImplemented is true. |

- Probe Response frame format

Insert the following new row into Table 9-41 (Probe Response frame body) in numeric order

| - Probe Response frame body |  |  |  |
| --- | --- | --- | --- |
| Order | Information | Notes |  |
| <ANA> | E-BCS UL Capabilities | This element is optionally present within a broadcast Probe Response frame if dot11eBCSSupportImplemented is true. |  |

## 9.4.2 Elements

#### 9.4.2.1 General

Insert the following new rows into Table 9-77 (Element IDs) in numeric order:

Table 9-77 - Element IDs

| Element | Element ID | Element ID Extension | Extensible | Fragmentable |
| --- | --- | --- | --- | --- |
| E-BCS UL Capabilities | 255 | <ANA> | Yes | No |

Insert the following new subclauses:

9.4.2.X E-BCS UL Capabilities element

9.4.2.X.1 General

The E-BCS UL Capabilities element contains fields that are used to advertise the capabilities of an eBCS  STA related to forwarding service to a remote destination.

An eBCS AP declares support for forwarding service and related capabilities by including the E-BCS UL Capabilities element in a Beacon and broadcast Probe Response frame.

An eBCS non-AP STA can include the E-BCS UL Capabilities element in an E-BCS UL frame if it intends to provide its capabilities to an AP that forwards its data to a remote destination.

|  | Element ID | Length | Element ID Extension | E-BCS UL Capabilities |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 9-<BC-1> - E-BCS UL Capabilities element format |  |  |  |  |

The format of E-BCS UL Capabilities element is shown in Figure 9-<BC-1> (E-BCS UL Capabilities element format).

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1 (General).

The contents of an E-BCS UL Capabilities field is defined in 9.4.2.X.2 when the element is transmitted by an eBCS AP and defined in 9.4.2.X.3 when the element is transmitted by an eBCS non-AP STA.

9.4.2.<ANA>.2 E-BCS AP UL Capabilities

The format of an E-BCS UL Capabilities field when transmitted by an eBCS AP is shown in Figure 9-<BC-2> (Format of E-BCS UL Capabilities field for an AP).

|  | UL AP Control |  |  |
| --- | --- | --- | --- |
| Octet: | 2 |  |  |
|  |  | 9-<BC-2> - Format of E-BCS UL Capabilities field for an AP |  |

The format of UL AP Control is shown in Figure 9-<BC-3> (UL AP Control field format).

|  |  | B0 B1 | B2 B3 | B4 | B5 |  | B6 | B7 B15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Authentication Mode | Limiting Mode | Location Embedding Supported |  | Data-Time Embedding Supported | IP Address Embedding Supported | Reserved |  |
| Bits: |  | 2 | 2 | 1 | 1 |  | 1 | 9 |  |
|  | 9-<BC-3> - UL AP Control field format |  |  |  |  |  |  |  |  |

The encoding of Authentication Mode subfield is shown in Table 9-<BC-4> (Encoding of Authentication Mode subfield).

| Table 9-<BC-4> - Encoding of Authentication Mode subfield |  |  |
| --- | --- | --- |
| Subfield value | Definition | Encoding |
| 0 | No Authentication | AP forwards contents of an E-BCS UL frame to the remote destination identified in the frame without authenticating the transmitter of the frame. |
| 1 | Per Destination | AP forwards contents of an E-BCS UL frame only if it is able to successfully able to authenticate the transmitter of the frame based on an established relationship with the remote destination identified in the frame. |
| 2 – 3 | Reserved |  |

The encoding of Limiting Mode subfield is shown in Table 9-<BC-5> (Encoding of Limiting Mode subfield).

| Table 9-<BC-5> - Encoding of Limiting Mode subfield |  |  |  |
| --- | --- | --- | --- |
| Subfield value | Definition | Encoding |  |
| 0 | No Throttling | AP applies no restrictions on the amount/frequency of ULs from a non-AP STA destined to a remote destination. |  |
| 1 | Per Destination | AP applies forwarding limits as specified by the remote destination with whom it has established a relationship. |  |
| 2 – 3 | Reserved |  |  |

Location Embedding Supported subfield is set to 1 if the AP supports embedding of location information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an E-BCS UL frame to the remote destination. Otherwise, the subfield is set to 0.

Date-Time Embedding Supported subfield is set to 1 if the AP supports embedding of date and time information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an E-BCS UL frame to the remote destination. Otherwise, the subfield is set to 0.

IP Address Embedding Supported subfield is set to 1 if the AP supports embedding of IP address information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an E-BCS UL frame to the remote destination. Otherwise, the subfield is set to 0.

9.4.2.X.3 E-BCS Non-AP UL Capabilities

The format of an E-BCS UL Capabilities field when transmitted by an eBCS non-AP STA is shown in Figure 9-<BC-6> (Format of E-BCS UL Capabilities field for a non-AP STA).

|  | UL Non-AP STA Control |  |
| --- | --- | --- |
| Octets: | 1 |  |
| 9-<BC-6> - Format of E-BCS UL Capabilities field for a non-AP STA |  |  |

The format of UL Non-AP STA Control is shown in Figure 9-<BC-7> (UL Non-AP STA Control field format).

|  | B0 |  |  | B1 | B2 |  | B3 | B4 B7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No forwarding Without Embedding |  |  | Location Embedding Requested | Date-Time Embedding Requested | IP Address Embedding Requested |  | Reserved |  |
| Bits: | 1 |  |  | 1 | 1 | 1 |  | 4 |  |
|  |  |  | 9-<BC-7> - UL Non-AP STA Control field format |  |  |  |  |  |  |

The No Forwarding Without Embedding subfield is set to 1 to indicate that the AP is not required to forward the contents of the frame transmitted by non-AP STA to the remote destination if it cannot append the requested information before forwarding. Otherwise the subfield is set to 0.

The Location Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append location information before forwarding. Otherwise the subfield is set to 0.

The Date-Time Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append date and time information before forwarding. Otherwise the subfield is set to 0.

The IP Address Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append AP’s IP address information before forwarding. Otherwise the subfield is set to 0.

9.6 Action frame format details

9.6.7 Public Action details

9.6.7.1 Public Action frames

Insert the following new row into Table 9-363 (Public Action field values) in numeric order:

| Table 9-363 – Public Action field values |  |  |
| --- | --- | --- |
| Public Action field value | Description |  |
| <ANA> | E-BCS UL |  |

Insert the following new subclause:

9.6.7.X E-BCS UL frame format

The E-BCS UL frame is transmitted by an eBCS non-AP STA carrying data intended for a remote destination identified in the frame. The format of E-BCS UL frame Action field is shown in Figure 9-<BC> (E-BCS UL frame Action field format).

|  | Category | Public Action | E-BCS UL Control | HLP Payload Length | HLP Payload | STA Certificate Length | STA Certificate | Timestamp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | variable | 0 or 2 | 0 or variable | 0 or 8 |
|  | Destination URI | E-BCS UL Capabilities | Frame Signature Length | Frame Signature |  |  |  |  |
| Octets: | variable | 0 or 4 | 0 or 2 | 0 or variable |  |  |  |  |
| Figure 9-<BC-8> - E-BCS UL frame Action field format |  |  |  |  |  |  |  |  |

The Category field is defined in 9.4.1.11 (Action field).

The Public Action field is defined in 9.6.7.1 (Public Action frames).

|  | B0 |  | B1 | B2 | B3 | B4 B6 | B7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STA Certificate Present |  | Packet Number Present | E-BCS UL Capabilities Present | Timestamp Present | Reserved | Frame Signature Present |  |
| Bits: | 1 |  | 1 | 1 | 1 | 3 | 1 |  |
|  |  | 9-<BC-9> - EBCS UL Control field format |  |  |  |  |  |  |

The format of E-BCS UL Control field is shown in Figure 9-<BC-9> (E-BCS UL Control field format).

The STA Certificate Present subfield is set to 1 when the STA Certificate Length and STA Certificate fields are carried in the frame. Otherwise the subfield is set to 0.

The Packet Number Present subfield is set to 1 when the Packet Number field is carried in the frame. Otherwise the subfield is set to 0.

The E-BCS UL Capabilities Present subfield is set to 1 when the E-BCS UL Capabilities element is carried in the frame. Otherwise the subfield is set to 0.

The Timestamp Present subfield is set to 1 when the Timestamp field is carried in the frame. Otherwise the subfield is set to 0.

The Frame Signature Present subfield is set to 1 when the Frame Signature Length and Frame Signature fields are carried in the element. Otherwise the subfield is set to 0.

The HLP Payload Length indicates the length of the HLP Payload field

The HLP Payload field carries the higher layer protocol (HLP) payload.

The STA Certificate Length field indicates the length of the STA Certificate field. The presence of this field is indicated by the STA Certificate Present subfield in the E-BCS UL Control field being equal to 1.

The STA Certificate field carries the certificate of the transmitting STA. The presence of this field is indicated by the STA Certificate Present subfield in the E-BCS UL Control field being equal to 1.

The Timestamp field provides protection against replay attack. The presence of this field is indicated by the Timestamp Present subfield in the E-BCS UL Control field being equal to 1.

The format of the Timestamp field is shown in Figure 9-<BC-10> - Timestamp field format.

|  | Time | Counter |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 |  | 4 |  |  |
| 9-<BC-9> - Timestamp field format |  |  |  |  |  |

The 32 bit Time subfield carries time information. See clause 11.X.Y.3 (E-BCS UL operation at an eBCS non-AP STA) for details on the value carried in this subfield.

The 32 bit Counter subfield carries a numeric value which is incremented for each packet transmission.

The Destination URI element is defined in 9.4.2.89 (Destination URI element) and carries the address of the remote destination where the packet needs to be forwarded to.

Note: The length of the Destination URI element is computed based on the value carried in the Length field in the element (value in Length field + 2 octets).

The E-BCS UL Capabilities element is defined in 9.4.2.X (E-BCS UL Capabilities element). The element is optionally present and carries the capabilities of the transmitting non-AP STA. The presence of this element is indicated by the E-BCS UL Capabilities Present subfield in the E-BCS UL Control field being equal to 1.

The Frame Signature Length field indicated the length of the Frame Signature field. The presence of this field indicated by the Frame Signature Present subfield in the E-BCS UL Control field being equal to 1.

The Frame Signature field carries signature for the contents of the E-BCS UL frame Action field except the Frame Signature field. The presence of this field indicated by the Frame Signature Present subfield in the E-BCS UL Control field being equal to 1.

# 11 MLME

Insert the following subclause:

### 11.X eBCS procedures

11.X.Y E-BCS UL Service

11.X.Y.1 General

The eBCS UL Service procedure allow a non-AP STA to transmit an UL frame with the expectation that one or more eBCS APs in the neighborhood might forward the contents of the frame to a remote destination specified in the frame. An eBCS non-AP STA may include a request to the forwarding AP to append additional information before forwarding to the remote destination. The forwarding service is best effort with no guarantee that the contents are delivered to the remote destination identified in the STA’s UL frame. Furthermore, a STAs request to embed metadata may not be fulfilled by a forwarding AP.

11.X.Y.2 E-BCS UL operation at an eBCS AP

An eBCS AP may provide forwarding service in which it can support forwarding the contents of an E-BCS UL frame received from an eBCS non-AP STA to a remote destination identified in the frame.

An eBCS AP that supports forwarding service shall declare its ability to forward by include E-BCS UL Capabilities element (see 9.4.2.xx.2 (E-BCS AP UL Capabilities)) in the Beacon and broadcast Probe Response frames that it transmits.

An eBCS AP that supports forwarding and is capable of embedding the requested metadata shall indicate its ability to support embedding by setting the corresponding subfield in the E-BCS UL Capabilities element to 1 and shall append the requested metadata to the content received from the STA before forwarding it to the remote destination. Format TBD.

An eBCS AP that supports forwarding but doesn’t support embedding of the requested metadata shall not forward the frame to the remote destination if the No Forwarding Without Embedding subfield in the E-BCS UL Capabilities element carried in the E-BCS UL frame is set to 1. Otherwise the AP shall forward the frame to the remote destination identified in the frame.

An eBCS AP may authenticate the transmitter of the packet before forwarding it to a remote destination and shall provide an indication of the authentication scheme in the E-BCS Capabilities element that it transmits. An eBCS AP that doesn’t requires authentication of the transmitter shall forward the frame to the remote destination indicated in the frame irrespective of whether the frame carries the STA Certificate field or the Packet Number field or the Frame Signature field. In order to prevent DoS or injection attacks directed towards the remote destination, it is strongly recommended that eBCS APs that support forwarding service perform source authentication and validate the frame signature.

An eBCS AP may limit the amount/frequency of ULs it forwards to a remote destination and shall provide an indication of the throttling scheme in the E-BCS UL Capabilities element that it transmits.

NOTE – Forwarding service is best effort and an eBCS AP, that supports forwarding service, is not required to forward a STA’s data to the destination identified in the STA’s E-BCS UL frame if the conditions indicated by the AP (such as authentication and/or throttling) are not satisfied or for other reasons.

11.X.Y.3 E-BCS UL operation at an eBCS non-AP STA

An eBCS non-AP STA that desires to send data to a remote destination may perform an unsolicited broadcast of an E-BCS UL frame carrying data intended for a remote destination. The URI to the remote destination would be carried in the frame. The frame may also carry additional request from the transmitting STA to the forwarding AP.

The format of the E-BCS UL frame is described in 9.6.7.x (E-BCS UL Frame Format).

The STA shall include the higher layer data intended for the remote destination in the HLP Payload field of the E-BCS UL frame.

The STA may include in the E-BCS UL frame fields (such as STA Certificate, Timestamp and Frame Signature) for authentication, preventing replay attack and protecting the contents of the frame.

The STA Certificate field when present in the frame shall carry the certificate of the transmitting STA.

The Timestamp field when present in the frame carries timing information to prevent reply attack.

When the STA has time information, the Time subfield of the Timestamp field shall carry the number of seconds since 2020-01-01 00:00:00 UTC; otherwise the subfield shall be set to 0.

NOTE – How a STA obtains time information is out of scope of this standard.

The Counter subfield of the Timestamp field shall carry a numeric value which is incremented for each packet transmission. In the rare scenario where the STA has transmitted 232 – 1 frames, the value in the field wraps around and starts from 0.

The STA shall include the Destination URI element in the E-BCS UL frame to provide the address of the remote destination where the packet needs to be forwarded to.

An eBCS non-AP STA may include E-BCS Capabilities element (see 9.4.2.xx.3 (E-BCS Non-AP UL Capabilities)) to request embedding of one or more metadata by the forwarding eBCS AP before forwarding the content to the remote destination identified in the frame.

The Frame Signature field when present in the frame shall carry the signature for the contents of the E-BCS UL frame Action field except for the field itself. The contents of the field provide protection against any attack that attempts to tamper the content of the frame.

An eBCS non-AP STA may monitor the WM to gather capabilities of nearby eBCS APs and may conform to the requirements indicated by neighboring eBCS AP(s) that support forwarding service. An eBCS non-AP STA may transmit an E-BCS UL frame without obtaining any information about nearby eBCS AP(s).

NOTE – Forwarding service is best effort and an eBCS non-AP STA is not required to scan or conform to the capabilities of a neighbor eBCS AP that supports forwarding service.