IEEE P802.11
Wireless LANs

| 802.11bc SFD text for clause 9.4.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-05-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | BlackBerry | The Pearce Building, West Street, Maidenhead, SL6 1RL, UK |  | smccann@blackberry.com |
| Antonio de la Oliva | InterDigital | Avda. de la Universidad 30, 28911, Leganes, Spain |  | aoliva@it.uc3m.es |

### Abstract

This document describes a proposed update to the ANQP-element definitions in clause 9.4.5.

9.4.5 	Access network query protocol (ANQP) elements

9.4.5.1 	General

Add the following three new entries at the base of Table 9-330, before the Vendor Specific entry.

| - ANQP-element definitionsinput.doc |  |  |
| --- | --- | --- |
| ANQP-element name | InfoID | ANQP-element (subclause) |
| Enhanced Broadcast Services | <ANA> | 9.4.5.bc |
| Enhanced Broadcast Request | <ANA+1> | 9.4.5.bc1 |
| Enhanced Broadcast Response | <ANA+2> | 9.4.5.bc2 |

Add the following three new clauses after 9.4.5.29.

9.4.5.bc. Enhanced Broadcast Services ANQP-element

The Enhanced Broadcast Services ANQP-element provides a list of one or more enhanced broadcast services that are available from a peer STA.  The format of the Enhanced Broadcast Services ANQP-element is defined in Error: Reference source not found.

|  | Info ID |  | Length | Broadcast | Enhanced Broadcast Services Tuples |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 |  | 2 | 1 | variable |
|  |  | Figure 9-838bc Enhanced Broadcast Services ANQP-element format |  |  |  |

The Info ID and Length fields are defined in Error: Reference source not found.

The Broadcast field is a 1 octet bitmap advertising information and is defined in Table 9-838bc:

Table 9-838bc – Broadcast field values

| Bit | Description (when bit set to 1) |
| --- | --- |
| 0 | No enhanced broadcast capability |
| 1 | Enhanced broadcast service indicated by each Enhanced Broadcast Services Tuple field |
| 2 | Transmit an Enhanced Broadcast Request ANQP-element to register/de-register to/from broadcast service |
| 3 | Multiple enhanced broadcast services supported, transmit an Enhanced Broadcast Request ANQP-element for more information. |
| 4 | Registration is required to received broadcast |
| 5 - 7 | Reserved |

The Enhanced Broadcast Services Tuples field contains one or more Enhanced Broadcast Services Tuple fields as shown in Figure 9-838bc1.

|  | Length | Broadcaster MAC Address (Optional) | Broadcast Stream Information (Optional) | BroadcastTime Length (Optional) | Broadcast Time (Optional) | Broadcast Duration Length (Optional) | Broadcast Duration (Optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 6 | variable | 1 | variable |  |  |

Figure 9-838bc1 - Enhanced Broadcast Services Tuple field format

The Length field is a 1-octet field whose value is set to the total length of the optional fields.

The Broadcaster MAC Address field optionally indicates the MAC Address of the STA broadcasting this channel, in the case of a multi STA setup.

The Broadcast Stream Information field is used to identify the source of the broadcast stream information (e.g. as a channel number, OFDMA sub-channel [TBD - check name] or URL), when the value of the Broadcast field bit 1 is true (see Table 9-838bc).

The Broadcast Time Length field is an optional 1-octet field who value is set to the length of the Broadcast Time field. If the Broadcast Time field is not used, the Broadcast Time Length field is also not used.

The Broadcast Time field contains timing information about when the broadcast (indicated by the Broadcast Stream Information field) is available, in terms of real time and whether the broadcast repeats. The format of the Broadcast Time field is TBD. If the Broadcast Time field is not present, then the receiving STA can assume that the broadcast stream is always available.

The Broadcast Duration Length field is an optional 1-octet field who value is set to the length of the Broadcast Duration field. If the Broadcast Duration field is not used, the Broadcast Time Duration field is also not used.

The Broadcast Duration filed contains information about the duration of a broadcast (indicated by the Broadcast Stream Information field) in terms of duration. The format of the Broadcast Duration field is TBD.  The Broadcast Duration field is only present, when there is a Broadcast Time field also present.

[Note: the Broadcast Time and Broadcast Duration fields could be merged into a single Broadcast Schedule field]

9.4.5.bc1. Enhanced Broadcast Request ANQP-element

The Enhanced Broadcast Request ANQP-element provides a list of one or more enhanced broadcast services that are available from a peer STA.  The format of the Enhanced Broadcast Request ANQP-element is defined in Error: Reference source not found.

|  | Info ID |  | Length | Broadcast Action | Enhanced Broadcast Services Tuples (Optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 |  | 2 | 1 | variable |
|  |  | Figure 9-838bc2 - Enhanced Broadcast Request ANQP-element format |  |  |  |

The Info ID and Length fields are defined in Error: Reference source not found.

The Broadcast Action field is a 1 octet bitmap and is defined in Table 9-838bc1:

Table 9-838bc1 – Broadcast Action field values

| Bit | Description (when bit set to 1) |
| --- | --- |
| 0 – 1 | Reserved |
| 2 | Register to receive broadcast |
| 3 | Unregister from receiving broadcast |
| 4 | Register with Broadcast Service(s) as indicated in the Enhanced Broadcast Services Tuples field |
| 5 | Unregister Broadcast Service(s) as indicated in the Enhanced Broadcast Services Tuple field |
| 6 - 7 | Reserved |

The Enhanced Broadcast Services Tuples field is described in 9.4.5.bc.

9.4.5.bc2. Enhanced Broadcast Response ANQP-element

The Enhanced Broadcast Response ANQP-element provides a list of one or more enhanced broadcast services that are available from a peer STA.  The format of the Enhanced Broadcast Response ANQP-element is defined in Error: Reference source not found.

|  | Info ID |  | Length | Broadcast Action Response | Enhanced Broadcast Services Tuples (Optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 |  | 2 | 1 | variable |
|  |  | Figure 9-838bc3 - Enhanced Broadcast Response ANQP-element format |  |  |  |

The Info ID and Length fields are defined in Error: Reference source not found.

The Broadcast Action Response field is a 1 octet bitmap and is defined in Table 9-838bc2:

Table 9-838bc2 – Broadcast Action Response field values

| Bit | Description (when bit set to 1) |
| --- | --- |
| 0 | Success |
| 1 | Failure |
| 2 - 7 | Reserved |

The Enhanced Broadcast Services Tuples field is described in 9.4.5.bc.