IEEE P802.11
Wireless LANs

| 802.11 SA1 - Proposed Resolutions for EDITOR adhoc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-02-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corporation |  |  | Emily.h.qi@intel.com |

Abstract

This document contains proposed resolutions to SA1 editorial comments for the EDITOR ad hoc that require TG’s feedback.

R00: Initial proposal. Provided resolutions for CID 4312(EDITOR2), 4202(EDITOR2), 4117, 4601, and 4533

R01: Discussed and approved CIDs included in R00.

R02: Provided resolutions for 3 items raised in D3.1 candidate draft review.

R03: Provided draft resolutions for more editorial comments

| 7101 | 1706.09 | 11.11.10.3 | "the reporting AP has dot11LciCivicInNeighborReport and the neighbor AP has LCI MeasurementCapability (RM Enabled Capabilities element with the LCI Measurement Capability Enabled fieldset to 1) dot11RMLCIMeasurementActivated equal to true"-- this has at least two errors "has dot11LciCivicInNeighborReport" and"Measurement Capability (...) dot11RMLCIMeasurementActivated equal to true"Note that " (RM Enabled Capabilities element with the LCI Measurement Capability Enabled field set to 1)" is also a informal way of anonymously referencing a transmission by the AP)." this can also be improved. This informality occurs in a number of places in this subclause. The proposed changes addresses two of these. | Change cited text to:"the reporting AP has dot11LciCivicInNeighborReport equal to true and the neighbor AP indicates support for LCI measurement(the neighbor AP has transmitted an RM Enabled Capabilities element with the LCI Measurement Capability Enabled field equal to true)"Make matching changes at 1706.32. |
| --- | --- | --- | --- | --- |

| CID | Clause | Page | Line | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4312 (EDITOR2) |  |  |  | The names of the following fields should have all initials upper-case, to avoid confusion: "Number of Channel Measurement Info" should be "Of". Also "Number of RX DMG Antennas", "Number of Channels", "Number of Time Blocks", "Normal Number of Frames per Channel", "Number of ANQP OIs", "OI #1 and #2 Lengths" | As it says in the comment |
| 4202 (EDITOR2) |  |  |  | The name of fields should start with uppercase letters for each word, to avoid confusion (especially with fields with "And" or "Or" in their name). | As it says in the comment |

Discussion

The commenter lists the following in CID 4312:

- Number of Channel Measurement Info

- Number of RX DMG Antennas

- Number of Channels

- Number of Time Blocks

- Normal Number of Frames per Channel

- Number of ANQP OIs

- OI #1 and #2 Lengths

I am sure there are more instances as CID 4202 comments on the same issue without the locations.

Obviously, it is a broad issue, not specific to TGmd.

We brought up this issue in the editor meeting this week.

Here is summary of the discussion from the Editor meeting.

- Moving forward, in future amendments, Editors shall use all upper case for the (sub)field names and (sub)element names, including preposition and conjunction words, e.g. Of, And, Or, etc .......  The Editor Style guide will be updated accordingly.

- The new style guide will apply to the new amendments (the amendments haven't done MDR).

- This new guidance has no intention to update the existing amendments/revisions. Due to legacy issue, changing Grandfathered terms where were widely used is considered excessive.

WG Editor updated 802.11 style guide (09/1034r16 draft) by adding the following paragraph in 2.6 Capitalization

… …

For proper names, all words in the name should be capitalized, including prepositions and conjunctions. This is to avoid ambiguity where the preposition or conjuction might be misinterpreted as part of the sentence. For example, “HT And VHT Trigger Frame RX Support subfield.” This rule does not apply to legacy text; these remain unchanged since the effort involved in changing all instances is substantial and no harm has been demonstrated.

Therefore, my proposal is to reject these two comments, or revised.

Proposed Resolution:

Rejected.

Rejec Reason: WG editor will update 802.11 Editorial Style guide 09/1034 to add a new rule including preposition and conjunction capitalization in the (sub)field and (sub)element names. This rule applies for new amendments. As stated in the style guide, “This rule does not apply to legacy text; these remain unchanged since the effort involved in changing all instances is substantial and no harm has been demonstrated.”

| 4117 (DeLaOlivaDelgado, Antonio) | 9.4.5.1 | 1473.37 | Third column of Table 9-330 shows the first letter of each ANQP Element in upper case but the ones of element Local MAC Address Policy | Change "Local MAC address policy ANQP element" in the third column of Table 9-330 to "Local MAC Address Policy ANQP element" |
| --- | --- | --- | --- | --- |

Discussion

This comment was original included in the trivial comment tab.

My proposed resolution is:

REVISED (EDITOR: 2020-01-08 20:26:03Z). At 1494.16, change "Local MAC address policy" to "Local MAC Address Policy". Note to the commenter, when the text in 1494.16 is changed, the text in 1473.37 will be changed automatically.

Mark R suggested other locations to be changed, such as: "Advice of Charge ANQP-element" and

"Network Authentication Type with Timestamp ANQP-element" should become "…Of…" and "…With…" too.

Therefore, this comment is here for discussion.

My receommendation is to reject additional changes as proposed by Mark R, with the reason:

- Additional suggestions are out of scope of the original comment.

- Preposition and conjunction capitalization is a new rule and does not apply to legacy text.

Proposed Resolution:

REVISED.

At 1494.16, change "Local MAC address policy" to "Local MAC Address Policy". Note to the commenter, when the text in 1494.16 is changed, the text in 1473.37 will be changed automatically.

Note to the commenter: this change resolved the comment in the direction suggested by the commenter.

| 4601 (RISON, Mark) | 3.2 | 193.19 | "and a temporal key, which" should be "and a temporal key that" | As it says in the comment. |
| --- | --- | --- | --- | --- |

Discussion

Cited text:

A temporal key (TK) is used to pretet information exchanged over the link.

My recommendation is to accept it.

Proposed Resolution:

Revised.

Delete “, which is used to protect information exhcnaged over the link”.

========================================================

| 4533 (RISON, Mark) | 9.4.2.21.9 | 1059.25 | The text describing the fields shown in Figure 9-236--Measurement Report field format for STA Statistics report are not described with "field" | Add missing "field"s |
| --- | --- | --- | --- | --- |

Discussion

There are multiple locations.

Here is the Figure 236:

|  | Measurement Duration | Group Identity | Statistics Group Data | Optional Subelements |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 1 | variable | variable |
| - Measurement Report field format for STA Statistics report |  |  |  |  |

Proposed Resolution:

Revised.

Change text from 1059.39 to 1059.59 as follows:

The STA Statistics report reports the change in, or current values of, the requested statistics group data measured within the measurement duration. The Measurement Duration field is set to the duration over which the change in statistics group data was measured and reported, in units of TUs. (MDR2) A Measurement Duration field set to 0 indicates a report of the current values of the statistics group data. The Group Identity field indicates the requested statistics group describing the Statistics Group Data field according to Error: Reference source not found. The Statistics Group Data field contains the requested statistics from the MIB in Annex C related to the interface on which the request was received according to Error: Reference source not found. Units used for reporting a statistic or change in statistic are the units used to define the statistic in Annex C. When the Measurement Duration field is nonzero, the reported data values for statistics that are not counters are the current values of the statistics data at the end of the measurement duration. When the Measurement Duration field is zero the current values of the requested statistics group data are reported, rather than the change.

<< Trival Editorial Corrections >>

These changes raised during TGmd Draft D3.1-0500 review.

Item D3.1-1:

|  | Related comments |  |  | Comments | Proposed Changes |
| --- | --- | --- | --- | --- | --- |
| Mark RISON | 4461 | C.3 |  | The DESCRIPTION of dot11WirelessMGTEventPeerSTATxPower and dot11WNMEventPeerRprtStaTxPower say "peer-to-peer link" but don't mention "event" anywhere, unlike similar DESCRIPTIONs | As it says in the comment |

Discussion:

For dot11WirelessMGTEventPeerSTATxPower, at 3925.40 (D3.1), it states “peer-to-peer link”.  At 3925.27, it states “peer-topeer

event report”, see:

Proposed to change to “peer-to-peer link event report” at 3925.40.

For dot11WNMEventPeerRprtStaTxPower, at 4063.32, it states “peer-to-peer

Link”. At 4063.17, it states “peer-to-peer link event report”.

Proposed to change to “peer-to-peer link event report” at 4063.32.

Proposed changes for Item D3.1-1:

Note that the changes are based on D3.1.

At 3925.40 and 4063.32, change “peer-to-peer link” to “peer-to-peer link event report”.

=======

Item D3.1-2:

|  | Related comments |  |  | Comments | Proposed Changes |
| --- | --- | --- | --- | --- | --- |
| Mark H | 4595 | 9.4.5.22 | 1491.1 | There are 12 more of these in the Draft, 7 "is a 2-octet field"s, 1 "is a 3-octet field", 1 "is a 5-octet field", and 1 "is a 6-octet field". Should we fix all of these (Editorial correction), so it doesn't come back next time as another comment? | As it says in the comment |

Discussion:

Agreed with Mark H that we need to fix them.

I searched for “octet field” through out the draft.  There are 29 instances of “octet field” in D3.1.

I believe 23 instances in clause 9 and clause 12 need to be fixed.

Proposed changes for Item D3.1-2:

Note that the changes are based on D3.1.

At 937.32, change “The Venue Info field is a 2-octet field. It contains Venue Group and Venue Type subfields.” To “The Venue Info field contains Venue Group and Venue Type subfields.”

At 1197.37 change “The Frequent Transition Count Threshold field is a 1-octet field containing the number of transitions ...” to “The Frequent Transition Count Threshold field contains the number of transitions ...”

At 1211.8, change “The BSSID field is a 6-octet field, as described in 9.2.4.3.4 (BSSID field), that identifies the BSS indicated in the AP Descriptor subelement.” To “The BSSID field identifies the BSS indicated in the AP Descriptor subelement, as described in 9.2.4.3.4 (BSSID field).”

At 1211.30, change “The Antenna Count field contains a 1-octet field indicating the number of antennas of the indicated antenna type and antenna gain pair.” To “The Antenna Count field contains the number of antennas of the indicated antenna type and antenna gain pair.”

At 1211.60, change “The Collocated Radio Type subelement contains a 1-octet field indicating the type of collocated radio” to “The Collocated Radio Type subelement indicates the type of collocated radio”

At 1212.50, change “The Device Type field is a 1-octet field indicating the category of device.” to “The Device Type field indicates the category of device.”

At 1261.14, change “The Alert Identifier Hash (AIH) is an 8-octet field. It is a unique value used to indicate an instance of an EAS message.” To “ The Alert Identifier Hash (AIH) field contains a unique value used to indicate an instance of an EAS message.”

At 1366.3, change “The Association Delay Info field(M101) is a 1-octet field whose value is an unsigned integer indicating the minimum association response delay in number of TUs.” To “The Association Delay Info field(M101) contains an unsigned integer indicating the minimum association response delay in number of TUs.”

At 1476.23, change “The Venue Info field is a 2-octet field and is defined in 9.4.1.34 (Venue Info field).” To “The Venue Info field is defined in 9.4.1.34 (Venue Info field).”

At 1476.30, change “The Length field is a 1-octet field whose value is equal to 3 plus the number of octets in the Venue Name field.” To “The Length field is equal to 3 plus the number of octets in the Venue Name field.”

At 1477.21, change “The Length of Emergency Call Number field is a 1-octet field whose value is set to the length of the Emergency Call Number field.” To “The Length of Emergency Call Number field is set to the length of the Emergency Call Number field.”

At 1478.1, change “The Network Authentication Type Indicator field is a 1-octet field and has one of the values shown in Table 9-331” to “The Network Authentication Type Indicator field is set to one of the values shown in Table 9-331.”

Also, at 1478.24, 1478.29, 1478.34, 1478.37, change “if the Network Authentication Type Indicator” to “if the Network Authentication Type Indicator field”.

At 1478.42, change “The Redirect URL Length field is a 2-octet field whose value is the length of the Redirect URL.” To “The Redirect URL Length field contains the length of the Redirect URL.”

At 1481.10, change “The NAI Realm Count field is a 2-octet field that specifies the number of NAI realms included in the NAI Realm ANQP-element.” To “The NAI Realm Count field specifies the number of NAI realms included in the NAI Realm ANQP-element.”

At 1488.33, change “The Length field is a 1-octet field whose value is set to 1 plus the number of octets in the Venue URL field.” To “The Length field is set to 1 plus the number of octets in the Venue URL field.”.

At 1489.13, change “The Length field is a 2-octet field whose value is set to 3 plus the number of octets in the NAI Realm and Plan Information Tuples fields.” to “The Length field is set to 3 plus the number of octets in the NAI Realm and Plan Information Tuples fields.”

At 1489.17, change “The Advice of Charge Type field is a 1-octet field with the values defined in Table 9-337.”. “the Advice of Charge Type field is set to one of the values defined in Table 9-337”

At 1489.52, change “The Length field is a 2-octet field whose value is set to the number of octets in the Language, Currency Code, and Plan Information fields.” to “The Length field is set to the number of octets in the Language, Currency Code, and Plan Information fields.”

At 1490.34, change “The Length field is a 1-octet field whose value is set to 2(#2203) plus the number of octets in the Local Content URL, Label Length and Label fields.” to “The Length field is set to 2(#2203) plus the number of octets in the LocalContent URL, Label Length and Label fields.”

At 1490.39, change

“The State field is a 1-octet field whose value is defined in Table 9-338” to

“The State field is defined in Table 9-338” .

At 1492.32, change

“The AP List Length (#8)field is a 1-octet field whose value indicates the length of the BSSIDs field” to

“The AP List Length (#8)field indicates the length of the BSSIDs field”.

At 1497.47, change

“The Length field is a 2-octet field that indicates the number of octets in the Information field.(#1462)” to

“The Length field indicates the number of octets in the Information field.(#1462)”.

At2606.1, change

“QoS Control field, if present, a 2-octet field that includes the MSDU priority.” To

“QoS Control field contains the MSDU priority, if present.”

Item D3.1-3:

|  | Related comments |  |  | Comments | Proposed Changes |
| --- | --- | --- | --- | --- | --- |
| Mark H | 4140 |  | 1710.33 | I know I’m not supposed to judge the Resolution at this time, just whether it was Edited correctly. However, I believe the Resolution is just wrong here. There are two parts to this sentence, for then the NDP BllockAck is used during a BlockAck session, and for when the NDP BlockAck is used during a fragment BA session. Each of needs to cite how the field is defined for that particular case, the first one was missing the reference, and that was the error in the D3.0 text. It should not have been changed to word around having the reference. This error was in the published 802.11ah, so it's hard to know what the correct cross-reference text is, but it appears that subclause 10.24.7.5 (802.11ah numbering) was intended. That subclause is now 10.25.6.5 in REVmd D3.1 | Change the text back, and add the reference, to say, "This field is defined in 10.25.6.5 when the NDP BlockAck is used during a BlockAck session …" Confirm this change with the TG, since it goes against the agreed Resolution and will require updating the Resolution. |

Discussion:

We discussed on TGmd teleconference on Jan 31st.

TGmd agreed on the following changes.

Updated the resolution CID 4140 with follows:

Revised. Change text in D3.0

"This field is defined in when the NDP BlockAck is used during a BlockAck session and is defined in 10.3.2.12 (Fragment BA procedure(11ah)) when it is used during a fragment BA session." to

"This field is defined in 10.25.6.5 (Generation and transmission of BlockAck frames by an HT STA, DMG STA, or S1G STA(11ah)) when the NDP BlockAck is used during a BlockAck session and is defined in 10.3.2.12 (Fragment BA procedure(11ah)) when it is used during a fragment BA session."

Note that the cited text in D3.0 was changed to “ This field is (#4140)used when the NDP BlockAck is used

during a BlockAck session and is defined in 10.3.2.12 (Fragment BA procedure(11ah)) when it is used

during a fragment BA session.” in D3.1

Therefore, we need to change D3.1 text instead.

Proposed resolutions for Item D3.1-3(#4140):

- Change the status of CID 4140 to “Revised”.

- At 1710.33 (D3.1), change

“This field is (#4140)used when the NDP BlockAck is used

during a BlockAck session and is defined in 10.3.2.12 (Fragment BA procedure(11ah)) when it is used

during a fragment BA session.” to

"This field is defined in 10.25.6.5 (Generation and transmission of BlockAck frames by an HT STA, DMG STA, or S1G STA(11ah)) when the NDP BlockAck is used during a BlockAck session and is defined in 10.3.2.12 (Fragment BA procedure(11ah)) when it is used during a fragment BA session."

====

| 4499 (RISON, Mark) | 9.4.2.21.9 | 1059.25 | If the term "slave" is no longer acceptable (CID 2020), is the term "master" still acceptable (other than "master key" contexts)? There are only a few such instances | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

IEEE Editor completed MEC with TGmd draft 2.1. There is no issue with the term “Master”.

206.30, 1506.3, 1506.46 (2x), 1506.47, 1507.5 (2x), 2146.34, 3773.33

At 206.30:

At 3773.33:

Above two locations are related to master WPD, which is regulatory term.

My recommendation is not change.

At 1506, 1507:  those are related a field name.  My recommendation is not change.

2146.34:

“timing master” for the TSF?

We could change it to “timing controller”. or keep as it is.

Proposed Resolution

Revised/Rejected

IEEE Editor completed MEC with TGmd draft 2.1. There is no issue with the term “Master”.

============

| 4807 (Mark H) | 9.9 | 1700.57 | This entire subclause is about PPDU headers. This is not MAC frame format, and shouldn't be in clause 9. | Move the contents of 9.9 to be at the end of 23.3.11. |
| --- | --- | --- | --- | --- |

Discussion

9.9 is for   9.9 NDP CMAC PPDUs(#2398)(11ah)

23.3.11 is for 23.3.11 S1G preamble format for NDPs

The commenter suggested to move 9.9 to the end of 23.3.11 as 23.3.12.

Proposed Resolution

Accepted.

Note to Editor: Remove 9.9 and insert the contents of 9.9 after 23.3.11 as a new clause 23.3.12.

=======

| 4800 (Mark H) | 1.3 | 150.5 | The draft is full of the terms "collocated", "co-located" and "colocated", all with apparently the same general meaning (within specific contexts, per subsequent nouns). Align all these. From a Google search: - collocated (pronounced KAHL-uh-kated) seems to be mostly an older word, that was really supposed to be a linguistic concept. - colocated (pronounced koh-LOH-kated) seems to be a somewhat deprecated spelling of co-located. - co-located (also pronounced koh-LOH-kated) seems to be a newer (since the 1960's) word for "put two or more things near each other". Recommend aligning on "co-located". | Throughout the draft, replace "collocated" (254 occurrences) and "colocated" (2 occurrences) with "co-located". |
| --- | --- | --- | --- | --- |

Discussion

Definition of “collocated”: https://www.merriam-webster.com/dictionary/collocate

Definition of “colocated” and “co-located”: https://www.merriam-webster.com/dictionary/colocate

“collocated” and “colocated” are different. “colocated” and “co-located” are same.

“Collocated” is used for “collocated interference” in most usages.

“colocated” is used for “colocated BSSID” in most usages.

According to IEEE style guid, we should try not include a hyphen. Suggest change “co-located” to “colocated” throughout the draft.

Proposed Resolution

Revised.

Change “co-located” to “co-located”, 13 instances.

Change “Co-Located” to “Colocated”, 22 instances.

Change “Co-located” to “Colocated”, 1 instance.

| 4691 (Mark R) |  |  | "Figure 9-189--Beacon Reporting data field format" should be "Beacon Reporting subelement data field format" -- but what is "data field" (several instances; see also Figure 9-232--Data field format)? | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

“Data field” is a field name of the Subelement format. See 9.4.3 subelements.

At 1022.11, change

“The Beacon Reporting subelement data field format” to

“The Beacon Reporting subelement Data field format”

At 1022.21, change

“Beacon Reporting data field format” to

“Beacon Reporting subelement Data field format”

Proposed Resolution

Revised.

Throughout the draft, change

“Reporting subelement data field format” to

“Reporting subelement Data field format”

4 instances.

Throughout the draft, change

“Reporting data field format” to

“Reporting subelement Data field format”.

12 instances including the reference locations.

| 4689 (Mark R) | 9.4.3 |  | "The optional subelements are ordered by nondecreasing subelement ID." (2x) -- they're ordered per 9.4.3 ("Subelements within an element are ordered by nondecreasing Subelement ID.") | Change to refer to 9.4.3 as for most optional subelement lists |
| --- | --- | --- | --- | --- |

Discussion

In 9.4.3, it states:

In 9.6.7.37 DCS Measurement Request frame format(11aj) and 9.6.7.38 DCS Measurement Response frame format(11aj), at 1566.5, and 1567.39,

it states:

In this two usages, sublements are included in a field, not in the element. Therefore the statement in 9.4.3 is not applied,

Proposed Resolution

Rejected.

Reason: In 9.4.3, subelements are within an element. In 9.6.7.37 and 9.6.7.38, subelements are within a field. Therefore, cannot change t o refer to 9.4.3 in 9.6.7.37 and 9.6.7.38.

| 4679 | 9.3.3.1 | 854.23 | "a) The Address 1 field of the Management frame is the RA (=DA) and is determined as the destination of the frame. b) The Address 2 field of the Management frame is the TA (=SA) and is determined as the address of the STA transmitting the frame(#2013)." arguably duplicates 9.2.4.3.1 | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

9.2.4.3.1 (at 794.3) is general. See:

The decription of Address fields in 9.3.3 are needed as they are specific to Management frames.

Proposed Resolution

Rejected.

Reason: The decription of Address fields in 9.3.3 are needed as they are specific to Management frames.

| 4661 |  |  | "section" should be "Subclause" | Fix in 9.4.2.47, 9.6.13.19, 12.4.2, 12.7.1.6.5, 12.7.2 (3x) |
| --- | --- | --- | --- | --- |

Discussion

Global search and found 62 instances.

Some of them refer to a section of in IETF draft or RFC, in IETF. “section” is used, not subclause or clause.

There is no change to those usage.

In cited locations, “section” is followed by a subclause number. There is no need to have word “section”.

Delete “section” at cited locations.

Proposed Resolution

Revised.

9.4.2.47, at 1166.47, change

“section 9.4.2.236 (OCI element(M58)(#2328)).” To

“9.4.2.236 (OCI element(M58)(#2328)).

In 9.6.13.19, at 1609.5, change

“section 9.4.2.236 (OCI element(M58)(#2328)).” To

“9.4.2.236 (OCI element(M58)(#2328)).”

In 12.4.2, at 2563.15, change

“sections 12.4.4.2.2 (Generation of the password element with ECC groups by looping(M137))” to

“12.4.4.2.2 (Generation of the password element with ECC groups by looping(M137))”

In 12.7.1.6.5, at 2657.49, delete “section”.

In 12.7.2, at 2663.14, 2666.48, 2667.39, delete “section”.

| 4615 |  |  | Since 1.4 defines x-y as being inclusive, the word "inclusive" is no longer needed for ranges | Delete "inclusive" in Clause 6 (10x), 9.2.2, 9.2.4.6.1 (2x), 9.4.2.5.1, 9.4.2.21.10, 9.4.2.45 (2x), 9.4.2.94, 9.4.2.154, 9.4.2.167 (3x), 10.19, 10.21 (3x), 11.10.14, 12.5.2.3.3, 19.3.9.3.2, 21.3.8.2.1, Table 23-1, dot11TxPowerLevelExtended in C.3 (2x). Delete "(all inclusive)" in 10.3.2.12 (x). Delete " (inclusive)" in 10.47.6 (second instance), 11.3.9.2, Table 21-23 (2x), Table 22-21 (2x), Table 23-29 (2x). Delete ", inclusive" in 12.3.3.3.6 |
| --- | --- | --- | --- | --- |

Discussion

Recommend to accept proposed changes.

Proposed Resolution

Accepted.

| 4608 | 3.1 |  | "individually addressed: When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with an individual address as the destination address (DA). When applied to a MAC protocol data unit (MPDU), it is an MPDU with an individual address in the Address 1 field. " v. "group addressed: A group addressed medium access control (MAC) service data unit (MSDU) is an MSDU that has a group address as its destination address (DA). A group addressed MAC protocol data unit (MPDU) is an MPDU that has a group address in its Address 1 field. Syn: multicast." -- the wording should be in the same form | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

At 163.19:

individually addressed: When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with an individual address as the destination address (DA). When applied to a MAC protocol data unit (MPDU), it is an MPDU with an individual address in the Address 1 field. Syn: directed, unicast.

At162.45:

group addressed: A group addressed medium access control (MAC) service data unit (MSDU) is an MSDU that has a group address as its destination address (DA). A group addressed MAC protocol data unit (MPDU) is an MPDU that has a group address in its Address 1 field. Syn: multicast.

Change group addressed definition to align with the same form of individually addressed.

Proposed Resolution

Revised.

At 162.45, change

“group addressed: A group addressed medium access control (MAC) service data unit (MSDU) is an MSDU that has a group address as its destination address (DA). A group addressed MAC protocol data unit (MPDU) is an MPDU that has a group address in its Address 1 field. Syn: multicast.”

To

“group addressed: When applied to a medium access control (MAC) service data unit (MSDU), it is an MSDU with a group address as the destination address (DA). When applied to a MAC protocol data unit (MPDU), it is an MPDU with a group address in the Address 1 field. Syn: multicast. “

| 4597 | 9.4.2.147 | 1327.55 | There is no behaviour associated with the setting of the A/C Power subfield, so this subfield is useless. CID 2345's resolution claimed that "The field is not useless. The AP or device setting up the relay benefits from information about the power characteristics of a relay device. The definition of the field is clear. An implementation can use this data for relay operation decisions." | After the referenced para add a "NOTE---A STA can use this information for relay operation decisions. How it does so is outside the scope of this standard." |
| --- | --- | --- | --- | --- |

Discussion

This is the same comment as CID 2345. In CID 2345, commenter proposed to rename “A/C Power” to “Reserved” in the figure. The comment CID 2345 was rejected with the rejection reason:

REJECTED (MAC: 2019-04-02 18:43:12Z): The commenter states: “There is no behaviour associated with the setting of the A/C Power subfield, so this subfield is useless.”

The field is not useless. The AP or device setting up the relay benefits from information about the power characteristics of a relay device. The definition of the field is clear. An implementation can use this data for relay operation decisions.

The note is not neccesory as the Relay Capability element (whole subclause) is for the STA participating in relay operation to advertise its capabilities. The A/C Power subfield is a subfield of Relay Capability Information field. Obviously, an implementation can use this data for relay operation decisions.

Proposed Resolution

Rejected.

Reason: the note is not neccesory as the Relay Capability element (whole subclause) is for the STA participating in relay operation to advertise its capabilities. The A/C Power subfield is a subfield of Relay Capability Information field. Obviously, an implementation can use this data for relay operation decisions.

| 4591 |  |  | The way UTF-8 strings are referred to is inconsistent. We have a definition of UTF-8 string (in 9.2.2) so just use that | In 9.4.2.2 change "the SSID is interpreted using UTF-8 encoding" to "the SSID is a UTF-8 string". In 9.4.2.21.14 change "The Public Identifier URI/FQDN field contains a URI encoded using UTF-8 and formatted in accordance with IETF RFC 3986" to "The Public Identifier URI/FQDN field contains a URI as a UTF-8 string, formatted in accordance with IETF RFC 3986,". In 9.4.2.26 change "The SSID in this BSS is interpreted using UTF-8 encoding" to "The SSID in this BSS is a UTF-8 string". At 1217.8 change "an UTF" to "a UTF". In 9.4.5.4 and 9.4.5.5 change "UTF-8 encoded field" to "UTF-8 string". In 9.4.5.10 change "a UTF-8 encoded character string" to "a UTF-8 string" and "in UTF-8 format" to "in UTF-8". In 9.4.5.17 change "field encoded using UTF-8 and " to "UTF-8 string,". In 9.4.5.21 change "UTF-8 formatted field " to "UTF-8 string ". In 9.4.5.22 change "a UTF-8 formatted string" to "a UTF-8 string" |
| --- | --- | --- | --- | --- |

Discussion

In 9.2.2 (781.37), we have UTF-8 string definition, see

Recommend to accept proposed changes.

Proposed Resolution

Accpeted.

| 4589 | 9 |  | Since "Extended Capabiltities element" appears zillions of times, it should be abbreviated to ECE (cf. RSNE, FTE, etc.) | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

301 instances of Extended Capabilities in TGmd D3.0

“Extended Capabilities” was introduced in IEEE802.11-2007.  It was used since then.

I don’t think it is good idea to change a well-known word to something else.

Also, “ECE” is the acronyms of “Electrical and Computer Engineering”.  😊.

Proposed Resolution

Rejected.

Reason: “Extended Capabilities” was introduced in IEEE802.11-2007.  It was used since then. It is a well-known word and should not be renamed.

| 4509 | 9.4.1.49 | 949.63 | "has the structure and order" -- everything is implicitly in the order shown | Delete " and order" at the referenced location and at 974.1 |
| --- | --- | --- | --- | --- |

Discussion

At 949.63:

At 974.1:

Recomment to accept the proposed changes.

Proposed Resolution

Accepted,

| 4482 | 9.6.3.2.1 | 1512.3 | Table 9-349--Basic ADDTS Request frame variant Action field format says "Higher Layer Stream ID element" but Table 9-350--DMG ADDTS Request frame variant Action field format, Table 9-351--Basic ADDTS Response frame variant Action field format, Table 9-352--DMG ADDTS Response frame variant Action field format, Table 9-356--ADDTS Reserve Request frame Action field format, Table 9-357--ADDTS Reserve Response frame Action field format says just "HIgher Layer Stream ID" | Delete " element" in the cited text in Table 9-349 |
| --- | --- | --- | --- | --- |

Discussion

At 1512.3

Recomment to accept the proposed changes.

Proposed Resolution

Accepted.

| 4474 |  |  | "over-the-WM" should not have hyphens, except when it's an adjective or adverb | Change hyphens to spaces in the cited text at 648.3, 649.56, 2553.31 |
| --- | --- | --- | --- | --- |

Discussion

8 instances.

5 are adjective or adverb.

3 locations need to be changed as listed by the commenter.

Proposed Resolution

Accepted.

| 4467 |  |  | "robust management frame should be "robust Management frame" | Fix at 197.57, in 11.12 caption, in dot11RSNAUnprotectedManagementFramesAllowed description (also change "frames" to "frame") |
| --- | --- | --- | --- | --- |

Discussion

### In Editorial Style guide, “2.1.2 Naming Frames” has a note:

Note, the following correct uses:

- a Management frame

My interpretion:

When it is used for a frame, it should be “Management frame”.

When it is used for other subjects, for example, protection, it doesn’t need to be “capitalized”.

For example, “management frame protection” is used with 91 instances in the draft.

At 197.45,

The subject is “service” in “robust management frame service”, not “frame”. There is no need to change “robust management frame service” to “robust Management frame service”.

At 11.12 (2328.50),

The subject is “procedures”, not “frame”. No change is needed.

At 3839.56, change

“robust management frames protection.” To

“robust management frame protection.”

Proposed Resolution

Revised.

At 3839.56, change

“robust management frames protection.” To

“robust management frame protection.”

| 4435 |  |  | Inconsistency/repetition on bit ordering wording in PHY clauses: 21.3.8.3.3 VHT-SIG-A definition NOTE--Integer fields are represented in unsigned binary format with the least significant bit in the lowest numbered bit position. 21.3.8.3.6 VHT-SIG-B definition For fields consisting of multiple bits, the LSB of the value occupies the lowest numbered bit of the field. 23.3.8.2.2.5 SIG definition / 23.3.8.2.3.2.5 SIG-A definition / 23.3.8.3.5 SIG definition Integer fields are represented in unsigned binary format with the least significant bit in the lowest numbered bit position. 20.4.3.2.1 General / 20.5.3.1.1 General / 24.4.3.2.1 General / 24.5.3.1.1 General All of the numeric fields are encoded in unsigned binary, least significant bit first. 25.3.9.1 General All the numeric fields are encoded in unsigned binary, least significant bit first. 19.3.9.4.3 HT-SIG definition NOTE 1--Integer fields are transmitted in unsigned binary format, LSB first. All of the fields in the HT-SIG are transmitted LSB first | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

Comment:

Inconsistency/repetition on bit ordering wording in PHY clauses:

21.3.8.3.3 VHT-SIG-A definition

NOTE--Integer fields are represented in unsigned binary format with the least significant bit in the lowest numbered bit position.

21.3.8.3.6 VHT-SIG-B definition

For fields consisting of multiple bits, the LSB of the value occupies the lowest numbered bit of the field.

23.3.8.2.2.5 SIG definition /

23.3.8.2.3.2.5 SIG-A definition /

23.3.8.3.5 SIG definition

Integer fields are represented in unsigned binary format with the least significant bit in the lowest numbered bit position.

20.4.3.2.1 General /

20.5.3.1.1 General /

24.4.3.2.1 General /

24.5.3.1.1 General

All of the numeric fields are encoded in unsigned binary, least significant bit first.

25.3.9.1 General

All the numeric fields are encoded in unsigned binary, least significant bit first.

19.3.9.4.3 HT-SIG definition

NOTE 1--Integer fields are transmitted in unsigned binary format, LSB first.

All of the fields in the HT-SIG are transmitted LSB first,

Discussion:

For different PHYs, we may need a sentence to describe it.

In clause19, 20, 21, 23, 24 and 25 general subclauses, add a sentence:

“All of the numeric fields are transmitted in unsigned binary format, LSB first.”

Note that there is no such sentence in clause 22 as it may refer to clause 21. For example.

At 3302.10, “The TVHT-SIG-B field for each BCU in any transmission mode is as defined in 21.3.8.3.6 (VHT-SIG-B definition) for 40 MHz bandwidth.”.  Therefore, there is no need to add a sentence in clause 22.

Proposed Resolution

Revised.

Delete cited sentences in the cited locations in the comment.

At the end of each of following subclauses:

19.3.9.4 HT portion of HT-mixed format preamble/19.3.9.4.1 Introduction

20.3.6 Common preamble/20.3.6.1 General

21.3.8.3 VHT portion of VHT format preamble/21.3.8.3.1 Introduction

23.3.8 S1G preamble/23.3.8.1 Introduction

24.3.6 Common preamble/24.3.6.1 General

25.3.9 CMMG SIG/25.3.9.1 General

add a paragraph:

“All of the numeric fields are transmitted in unsigned binary format, LSB first.”

| 4431 | 9.4.1.62 |  | Elsewhere Na has no subscripting | In Table 9-88--Order of angles in the CMMG Compressed Beamforming Report field and Table 9-90--Compressed Beamforming Report field and the para above it desubscript the a in Na |
| --- | --- | --- | --- | --- |

Discussion

Table 9-63:

Table 9-88:

Desubscript the a in Na. Should Nr and Nc should be desubscripted?

Proposed Resolution

Revised.

Desubscript the a in Na at the following locations: 973.14, 974.2, 974.18, 974.21, 974.24 and 974.29.

Desubscript the r in Nr and c in Nc at 973.14

| 4403 |  |  | NDP CMAC frames are generally referred to as such, not as S1G NDP CMAC frames | Delete "S1G " in "S1G NDP CMAC" throughout (~14x) |
| --- | --- | --- | --- | --- |

Discussion

I couldn’t find “S1G NDP CMAC frame”. I think “S1G NDP CMAC frame” has been changed to “S1G NDP CMAC PPDU”.

NDP CMAC PPDU is defined in 9.9, see:

Proposed Resolution

Accepted.

| 4355 | 9.3.1.2 | 826.20 | "In an RTS frame transmitted by a VHT STA in a non-HT or non-HT duplicate format to another VHT STA, the scrambling sequence carries the TXVECTOR parameters CH_BANDWIDTH_IN_NON_HT and DYN_BANDWIDTH_IN_NON_HT (see 10.3.2.7 (CMMG RTS procedure(11aj)))" -- clearly a broken xref | As it says in the comment |
| --- | --- | --- | --- | --- |
| 4354 | 9.3.1.2 | 826.20 | "In an RTS frame transmitted by a VHT STA in a non-HT or non-HT duplicate format to another VHT STA, the scrambling sequence carries the TXVECTOR parameters CH_BANDWIDTH_IN_NON_HT and DYN_BANDWIDTH_IN_NON_HT (see 10.3.2.7 (CMMG RTS procedure(11aj)))" -- clearly a broken xref | Delete the parenthetical |

Discussion

In 802.11-2016, the reference in  the cited paragrapy is 10.3.2.6:

10.3.2.6 in 802.11-2016 is:

In TGmd D3.0, 10.3.2.6 becomes 10.3.2.8:

So the correct reference is 10.3.2.8.

Are CID 4355 and CID 4354 the same comment?

Proposed Resolution for CID 4355 and 4354.

Revised.

Change “10.3.2.7 (CMMG  RTS procedure(11aj))” to “10.3.2.8 (VHT and S1G RTS procedure(11ah))”

| 4233 | 17.3.10.6 | 2959.38 | "The start of (#2601)an OFDM transmission at a receive level greater than or equal to the minimum modulation and coding rate sensitivity (-82 dBm for 20 MHz channel spacing, -85 dBm for 10 MHz channel spacing, and -88 dBm for 5 MHz channel spacing) shall " -- the parenthesis is duplication | Replace the material in the parenthesis with "see 17.3.10.2 Receiver minimum input sensitivity". Since the same requirement applies to any OFDM modulation and coding, change the cited text to start "The start of (#2601)an OFDM transmission at a receive level greater than or equal to the minimum modulation and coding rate sensitivity across all modulations and codings (". Make similar changes in other places where similar duplication occurs: "rate sensitivity (-82 + 20 = -62 dBm) in the 20 MHz channel.", "10 dB (-72 dBm for 20 MHz, -69 dBm for 40 MHz).", "The start of a DMG SC mode transmission at a receive level greater than the minimum sensitivity for MCS 1 (-68 dBm)", "coding rate sensitivity (-82 + 20 = -62 dBm)", "coding rate sensitivity (-88 + 20 = -68 dBm in", "a threshold (-68 dBm for 6 MHz," + 4 more in same subclause, "the minimum sensitivity for MCS 1 (-71 dBm)", "sensitivity for CMMG control mode (-78 dBm)", "The start of a CMMG SC mode transmission at a receive level greater than the minimum sensitivity for MCS 0 (-74 dBm for 540 MHz, -71 dBm for 1080 MHz)", "The start of a CMMG OFDM mode or CMMG SC mode transmission at a receive level greater than the minimum sensitivity for MCS 0 (-74 dBm for 540 MHz" |
| --- | --- | --- | --- | --- |
| 4232 | 17.3.10.6 | 2959.38 | "The start of (#2601)an OFDM transmission at a receive level greater than or equal to the minimum modulation and coding rate sensitivity (-82 dBm for 20 MHz channel spacing, -85 dBm for 10 MHz channel spacing, and -88 dBm for 5 MHz channel spacing) shall " -- the parenthesis is duplication | Replace the material in the parenthesis with "see 17.3.10.2 Receiver minimum input sensitivity". Since the same requirement applies to any OFDM modulation and coding, change the cited text to start "The start of (#2601)an OFDM transmission at a receive level greater than or equal to the minimum modulation and coding rate sensitivity across all modulations and codings (" |

Discussion

The proposed change from commenter for CID 4233:

Replace the material in the parenthesis with "see 17.3.10.2 Receiver minimum input sensitivity".  Since the same requirement applies to any OFDM modulation and coding, change the cited text to start "The  start  of  (#2601)an  OFDM  transmission  at  a  receive  level  greater  than  or  equal  to  the  minimum

modulation  and  coding  rate  sensitivity across all modulations and codings (".

Make similar changes in other places where similar duplication occurs:

"rate sensitivity (-82 + 20 = -62 dBm) in the 20 MHz channel.", "10 dB (-72 dBm for 20 MHz, -69 dBm for 40 MHz).", "The start of a DMG SC mode transmission at a receive level greater than the minimum sensitivity for MCS 1 (-68 dBm)", "coding rate sensitivity (-82 + 20 = -62 dBm)", "coding rate sensitivity (-88 + 20 = -68 dBm in", "a threshold (-68 dBm for 6 MHz," + 4 more in same subclause, "the minimum sensitivity for MCS 1 (-71 dBm)", "sensitivity for CMMG control mode (-78 dBm)",

"The start of a CMMG SC mode transmission at a receive level greater than the minimum sensitivity for

MCS  0  (-74  dBm  for  540  MHz,  -71  dBm  for  1080  MHz)", "The start of a CMMG OFDM mode or CMMG SC mode transmission at a receive level greater than

the minimum sensitivity for MCS 0 (-74 dBm for 540 MHz"

The proposed change from commenter for CID 4232:

Replace the material in the parenthesis with "see 17.3.10.2 Receiver minimum input sensitivity".  Since the same requirement applies to any OFDM modulation and coding, change the cited text to start "The  start  of  (#2601)an  OFDM  transmission  at  a  receive  level  greater  than  or  equal  to  the  minimum

modulation  and  coding  rate  sensitivity across all modulations and codings (".

Reassign them to  PHY .

Proposed Resolution

| 4208 | 152.45 | 2 | Some months in Clause 2 are abbreviated, others not | Abbreviate all months (to their 3-letter form) except May |
| --- | --- | --- | --- | --- |

Discussion

In  Draft Recommended Practice for Preparing an IEEE Standards Draft, no abbreciated month  is used, for example “November  2013”.

So, change abbreviated name of a month to the full name of a month. For example, change “Sept” to “September”.

Proposed Resolution

Revised.

Throughout Clause 2:

change “Sept.” or “Sep” to “September” .

change “Nov.” to “November”.

change “Feb.” to “Febuary” .

Change “Jan.” to “January”.

Change “Apr.” to “April”.

Change “Mar.” to “March”.

| 4203 | 9.4.2.229.2 | 1442.22 | The name of fields should start with uppercase letters for each word, to avoid confusion (especially with fields with "And" or "Or" in their name) | In Figure 9-754--CMMG Capabilities Info field format and Table 9-313--Subfields of the CMMG Capabilities Info field format change "Short GI for 540 MHz" to "Short GI For 540 MHz" |
| --- | --- | --- | --- | --- |

Discussion

See CID 4312.

Proposed Resolution

Rejected.

Rejec Reason: WG editor will update 802.11 Editorial Style guide 09/1034 to add a new rule including preposition and conjunction capitalization in the (sub)field and (sub)element names. This rule applies for new amendments. As stated in the style guide, “This rule does not apply to legacy text; these remain unchanged since the effort involved in changing all instances is substantial and no harm has been demonstrated.”

| 4198 | 9.4.2.24.3 |  | Table 9-151--AKM suite selectors is missing a row for 18. This is confusing, given the presence of a row for 0 | Delete the row for 0 in the table, and change the cell that says "(11ai )18, (#17 1)21 -255" to "0, (11ai )18, (#17 1)21 - |
| --- | --- | --- | --- | --- |

Discussion

The  location is 1106.21.

Proposed Resolution

Accepted.

| 4037 | 9.4.2.26 | 1116.22 | THIS COMMENT SUBMITTED ON BEHALF OF ROGER MARKS. The reserved subfields row is not correct. In Rev2.0, the penultimate row was 82, and 83-n were marked Reserved in the final row. Comment #2685 requested inserting a new row and incrementing the reserved row by 1. Instead, the new row was marked 87 and the prior row shows 86 as reserved. The remaining values are no longer marked as Reserved. | Change "Local MAC Address Policy" from 87 to 86. Move the reserved row (currently 86) to the end, and change it to "87-n". |
| --- | --- | --- | --- | --- |

Discussion

The value of Extended Capabilities field is assigned by ANA.

Value “86” is assigned to another task group.

At the bottom of the table, Add a row “88-n”/ “Reserved”

Proposed Resolution

Revised.

At the bottom of the table, Add a row “88-n”/ “Reserved”

| 4026 | 9.2.4.2 | 793.20 | The Duration/ID field also contains the association identifier for broadcast transmissions of S1G PPDUs. This is only mentioned in Table 9-9, but should be included in the text, which purportedly lists all definitions for the contents of the Duration field. | Change "In Control frames of subtype PS-Poll other than PS-Poll+BDT frames" to "In Control frames of subtype PS-Poll other than PS-Poll+BDT frames, and for broadcast transmissions of S1G PPDUs" |
| --- | --- | --- | --- | --- |

Discussion

Proposed changes from the commenter:

Change "In Control frames of subtype PS-Poll other than PS-Poll+BDT frames"

to

"In Control frames of subtype PS-Poll other than PS-Poll+BDT frames, and for broadcast transmissions of S1G PPDUs"

Cited text:

Feedback?

Proposed Resolution

??

| 4024 | 9.4.2.1 | 978.36 | The column "Element ID Extension" has a title the doesn't match how it is described in the text (see lines 20-22 above). | Change "Element ID Extension" to "Element ID Extension present" |
| --- | --- | --- | --- | --- |

Discussion

Cited text;

The “Element ID Extension” at line 35 is for a value of “Element ID Extension”, not for “present”.

At line 22, we should change “Element ID Extension present” to “Element ID Extension”.

Proposed Resolution

Revised.

At 978.22, change “Element ID Extension present” to “Element ID Extension”.

| 4688 |  |  | "multicast MSDU"/"Multicast MSDU" should be "group addressed MSDU"/"group addressed MSDU"; similarly unicast -> individually addressed | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

8 instances for “multicast MSDU”.

3 instances for “unicast MSDU”.

Proposed Resolution

Accepted.

Note to editor: 8 instances for “multicast MSDU”. 3 instances for “unicast MSDU”.

| 4593 |  |  | "forty MHz" should be "40 MHz" | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

17 instances.

Proposed Resolution

Accepted. Note to Editor: 17 instances.

| 4592 |  |  | CID 2096 asked for "that had received" to be changed to "that receives", and was accepted, but there are still some left | Change in 9.2.2 and 10.3.2.9 (not the other 3) |
| --- | --- | --- | --- | --- |

Discussion

I couldn’t find “that had received”.

I found 5 instances of “that has received”.

Proposed Resolution

Revised.

At 781.50 and 1741.31:

Chage “that has received” to “that receives”.

| 4501 |  |  | "ACM flag" and "ACM bit" should be "ACM subfield", as should "ACM field", for consistency | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

Trivial to search for the terms (2x ACM flag, 18x ACM bit, 8x ACM field (note not QACM))

Proposed Resolution

Accepted. Note to editor: 2x ACM flag, 18x ACM bit, 8x ACM field (note not QACM)

| 4469 | 3.4 |  | definitions with spaces shouldn't be in 3.4 (the components need definition only). So no 802.x LAN, HWMP SN, PP A-MSDU, PTP TSPEC or SA Query SPP A-MSDU | As it says in the comment |
| --- | --- | --- | --- | --- |

Discussion

802.x  LAN,

“LAN” is already defined. So , remove definition of “802.x LAN” in 3.4

HWMP SN;

“HWMP” is already defined. Add definition for “SN/ sequence number” in 3.4

PP A-MSDU:

“A-MSDU” is already defined.

PP” is already defined for “polling period”.

Add definition for “PPD/payload protected”

Change “PP A-MSDU” to “PPD A-MSDU”  throughout. 12 instances.

PTP TSPEC:

“TSPEC” is already defined.

Add definition in 3.4: “PTP/peer-to-peer”.

SA Query:

SA is defined for “Source Address”.

SA Query is for “security association query”, 113 instances.

Change “SA Query” to “SAQ” thoughout.

???

SPP A-MSDU:

“A-MSDU” is already defined.

Add definition “SPP/signaling and payload protected”.

Proposed Resolution

Delete following definitions in 3.4:

802.x  LAN,

HWMP SN,

PP A-MSDU,

PTP TSPEC

SA Query,

SPP A-MSDU

Add following definitions in 3.4:

“SN	 sequence number”

“PPD	 payload protected”

“PTP	 peer-to-peer”

“SAQ	security association query”.

“SPP	signaling and payload protected”

Change “PP A-MSDU” to “PPD A-MSDU”  throughout. 12 instances.

Change “SA Query” to “SAQ” thoughout, 113 instances. ??

|  | “ |  |  |  |
| --- | --- | --- | --- | --- |

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution

Discussion

Proposed Resolution