IEEE P802.11
Wireless LANs

| Misc CR on D6.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020.03.17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on D6.0.

3 comments, CID 24105, 24106, 24107 are resolved in this document.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Modification based on some offline comments |
| R2 | Modification based on the discussion during the CC |
| R3 | Modification based on Mark’s Opt C. Opt C makes no additional technical changes to the current spec except make some clarifications on the confusing part of the spec. |
| R4 | Add at least one 20 MHz subchannel shall be punctured. |
| R5 | Corrected typo in the primary to in the preamble |
| R6 | Further resolve Mark’s comment adding MIB part, and correcting the idle channels descriptions |
| R7 | Further resolve Mark’s comment |
| R8&R9 | Revisit CID 24194 per Song An’s comment |

## CID 24105

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 24105 | 561.25 | 27.3.11.7.2 | "where in the primary 80 MHz ... at least one 20 MHz subchannel that is not in the primary 40 MHz is punctured". Does this mean that in 160 MHz preamble puncturing using bandwidth field value 7, at least one 20 MHz within the Secondary 40 MHz need to be punctured? Probably that is not what was intended. The phrase "where in the primary 80 MHz" is unnecessary and may cause confusion. | Delete "the primary 80 MHz of". | REVISED Agree with the commenter that is not what was intended. Furthr clarify how many subchannels can be punctured. Instructions to the editor, plase make the changes as shown under CID 24105 in doc 11-20/0497r9 |

Instructions to the editor

Please make the changes to L21, P561 as shown below:

Table 27-20—HE-SIG-A field of an HE MU PPDU, Bandwidth field

Set to 4 for preamble puncturing in 80 MHz, where in the preamble the only punctured subchannel is the secondary 20 MHz channel.

Set to 5 for preamble puncturing in 80 MHz, where in the preamble the only punctured subchannel is one of the two 20 MHz subchannels in the secondary 40 MHz channel.

Set to 6 for preamble puncturing in 160 MHz or 80+80 MHz, where in the preamble the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel.

Set to 7 for preamble puncturing in 160 MHz or 80+80 MHz, where in the preamble  the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured.

## CID 24106

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 24106 | 192.57 | 9.4.2.247.3 | As written, B2 means that in 160 MHz, only one 20 MHz can be punctured, and that 20 MHz is the Secondary20. Is this the correct intention of this capability bit? | Please clarify. | Revised The changes are specified, follow the description in HE-SIG-A. Instructions to the editor, plase make the changes as shown under CID 24106 in doc 11-20/0497r9 |

Discussion

For non-HT duplicate, the logic is CBW80 cannot be punctured to CBW40 or lower; CBW160/80+80 cannot be punctured to CBW80 or lower. And at least one subchannel is punctured.

For punctured CBW80, the following modes are supported if assuming the first bit is P20, the second bit is S20

111x

11x1

1x11

1x1x

1xx1

Note that 1111, 1?xx cannot be supported.

For CBW160 or 80+80, 1111 1111, 1??? xxxx cannot be supported

Where 1 means not punctured, x means punctured, ? means either punctured or not

Instructions to the editor

Please make the changes to L57, P192 as shown below:

Table 9-321b—Subfields of the HE PHY Capabilities Information field

B0 indicates support for the reception of an 80 MHz preamble where the only punctured subchannel is the secondary 20 MHz channel.

B1 indicates support for the reception of an 80 MHz preamble where the only punctured subchannel is one of the two 20 MHz subchannels in the secondary 40 MHz channel.

B2 indicates support for the reception of a 160 MHz or 80+80 MHz preamble where the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel.

B3 indicates support for the reception of a 160 MHz or 80+80 MHz preamble where the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured.

Please make the changes to L43, P274 as shown below:

10.23.2.5 EDCA channel access in a VHT, HE or TVHT BSS

i) Transmit an 80 MHz HE MU PPDU where in the preamble the only punctured subchannel is the secondary 20 MHz channel, if all of the 20 MHz subchannels that are not punctured were idle during an interval of PIFS immediately preceding the start of the TXOP.

j) Transmit an 80 MHz HE MU PPDU where in the preamble the only punctured subchannel is one of the two 20 MHz subchannels in the secondary 40 MHz channel, if all of the 20 MHz subchannels that are not punctured were idle during an interval of PIFS immediately preceding the start of the TXOP.

k) Transmit an 160 MHz or 80+80 MHz HE MU PPDU where in the preamble the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel, if all of the 20 MHz subchannels that are not punctured were idle during an interval of PIFS immediately preceding the start of the TXOP.

l) Transmit an 160 MHz or 80+80 MHz HE MU PPDU where in the preamble the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel, if all of the 20 MHz subchannels that are not punctured were idle during an interval of PIFS immediately preceding the start of the TXOP; at least one 20 MHz subchannel is punctured.

NOTE 3—In the case of rule j), there is only one idle 20 MHz subchannel in the secondary 40 MHz channel and the other 20 MHz subchannel in the secondary 40 MHz channel is preamble punctured.

Please make the changes to L46, P484 as shown below:

Table 27-1—TXVECTOR and RXVECTOR parameters (continued)

HE-CBW-PUNC80-PRI for preamble puncturing in 80 MHz, where in the preamble the only punctured subchannel is the secondary 20 MHz channel.

HE-CBW-PUNC80-SEC for preamble puncturing in 80 MHz, where in the preamble the only punctured subchannel is one of the two 20 MHz subchannels in the secondary 40 MHz channel.

HE-CBW-PUNC160-PRI20 for preamble puncturing in 160 MHz, where in the preamble the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel.

HE-CBW-PUNC80+80-PRI20 for preamble puncturing in 80+80 MHz, where in preamble the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel.

HE-CBW-PUNC160-SEC40 for preamble puncturing in 160 MHz, where in the preamble the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured.

HE-CBW-PUNC80+80-SEC40 for preamble puncturing in 80+80 MHz, where in the preamble the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured.

Please make the changes to L8, P498 as shown below:

Table 27-3— Interpretation of FORMAT, NON_HT Modulation and CH_BANDWIDTH parame-ters (continued)

| FORMAT | NON_HT_ MODULATION | CH_BANDWIDTH | CH_OFFSET | PPDU format |
| --- | --- | --- | --- | --- |
|  |  | HE-CBW-PUNC80- PRI |  | The STA transmits an 80 MHz HE PPDU where the only punctured subchannel is the secondary 20 MHzchannel. |
|  |  | HE-CBW-PUNC80- SEC |  | The STA transmits an 80 MHz HE PPDU where the only punctured subchannel is one of the two 20 MHz subchannels in the secondary 40 MHz channel. |
|  |  | HE-CBWPUNC160- PRI20 |  | The STA transmits an 160 MHz HE PPDU where the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel. |
|  |  | HE-CBWPUNC80+ 80-PRI20 |  | The STA transmits an 80+80 MHz HE PPDU o where the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel |
|  |  | HE-CBWPUNC160- SEC40 |  | The STA transmits an 160 MHz HE PPDU where the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured. |
|  |  | HE-CBWPUNC80+ 80-SEC40 |  | The STA transmits an 80+80 MHz HE PPDU where the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured. |

Please make the changes to L50, P754 as shown below:

C.3 MIB Detail

dot11HEPuncturedPreambleRxImplemented OBJECT-TYPE

SYNTAX OCTET STRING(SIZE(1))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute is a bitmap that indicates the preamble puncturing support, where bit 0 is set to 1 to indicate support for the reception of an 80 MHz preamble where the only punctured subchannel is the secondary 20 MHz channel, bit 1 is set to 1 to indicate support for the reception of an 80 MHz preamble where the only punctured subchannel is one of the two 20 MHz subchannels in the secondary 40 MHz channel, bit 2 is set to 1 to indicate support for the reception of a 160 MHz or 80+80 MHz preamble where the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel, and  bit 3 is set to 1 to indicate support for the reception of a 160 MHz or 80+80 MHz preamble where the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured.."

::= { dot11PhyHEEntry 2 }

Please make the changes to L14, P499 as shown below:

Table 27-4—Interpretation of CH_BANDWIDTH and INACTIVE_SUBCHANNELS parameters

when FORMAT is equal to NON_HT and NON_HT_MODULATION is equal to NON_HT_DUP_

OFDM

| CH_BANDWIDTH | INACTIVE_SUBCHANNELS | CH_OFFSET | PPDU format |
| --- | --- | --- | --- |
| CBW80 | All bits set to 1 except for the four bits corresponding to the primary 80 MHz channel, which are set to 0 |  |  |
| CBW80 | The bit corresponding to the primary 20 MHz channel set to 0 and one or two other bits set to 0 that correspond to any other subchannels in the primary 80 MHz, except that the two other bits corresponding to secondary 40MHz channel shall not be both set to 0, all other bits set to 1 |  |  |
| CBW160 | All eight bits set to 0 |  |  |
| CBW160 | The bit corresponding to the primary 20 MHz channel set to 0 and one to six other bits set to 0 that correspond to any other subchannels in the 160 MHz, except that the four other bits corresponding to secondary 80MHz channel shall not be all set to 0, all other bits set to 1 |  |  |
| CBW80+80 | All eight bits set to 0 |  |  |
| CBW80+80 | The bit corresponding to the primary 20 MHz channel set to 0 and one to six other bits set to 0 that correspond to any other subchannels in the 80+80 MHz, except that the four other bits corresponding to secondary 80MHz channel shall not be all set to 0, all other bits set to 1 |  |  |

## CID 24107

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 24107 | 192.62 | 9.4.2.247.3 | Does this capability bit match the operation for value 7 of the Bandwidth field in HE-SIG-A of an HE MU PPDU? | Please clarify. | Revised. The changes are specified, follow the description in HE-SIG-A. The spec doesn’t have any restrictions on S80. The restrictions are for P80 only. Instructions to the editor, plase make the changes as shown under CID 24107 in doc 11-20/0497r9 |

Instructions to the editor

Please make the changes to L21, P561 as shown below:

Table 27-20—HE-SIG-A field of an HE MU PPDU, Bandwidth field

Set to 4 for preamble puncturing in 80 MHz, where in the preamble the only punctured subchannel is the secondary 20 MHz channel.

Set to 5 for preamble puncturing in 80 MHz, where in the preamble the only punctured subchannel is one of the two 20 MHz subchannels in the secondary 40 MHz channel.

Set to 6 for preamble puncturing in 160 MHz or 80+80 MHz, where in the preamble the only punctured subchannels are the secondary 20 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel.

Set to 7 for preamble puncturing in 160 MHz or 80+80 MHz, where in the preamble  the only punctured subchannels are zero, one or both of the 20 MHz subchannels in the secondary 40 MHz channel and zero to three of the 20 MHz subchannels in the secondary 80 MHz channel; at least one 20 MHz subchannel is punctured.

## CID 24194 (Revisit)

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 24194 | 586.36 | 27.3.11.8.5 | "The number of OFDM symbols in the HE-SIG-B field, denoted by NSYM,HE-SIG-B, shall be signaled by the Number Of HE-SIG-B Symbols Or MU-MIMO Users field in the HE-SIG-A field of an HE MU PPDU". Only true if No Compression is used. | Add "If the HE-SIG-B Compression field in the HE-SIG-A field is 0" | Revised Agreed with the commenter. Specify the changes. Instructions to the editor, plase make the changes as shown under CID 24194 in doc 11-20/0497r9 |

Discussion:

How to calculate the number of OFDM symbols in the HE-SIG-B field of compression mode is missing.

Instructions to the editor

Please make the changes to L36, P586 as shown below:

If the HE-SIG-B Compression field in the HE-SIG-A field is 0, the number of OFDM symbols in the HE-SIG-B field, denoted by NSYM,HE-SIG-B, shall be signaled by the Number Of HE-SIG-B Symbols Or MU-MIMO Users field in the HE-SIG-A field of an HE MU PPDU (see 27.3.11.7.2 (Content)).

If the HE-SIG-B Compression field in the HE-SIG-A field is 1, the number of OFDM symbols in the HE-SIG-B field is calculated based on the number of User fields in the HE-SIG-B content channel, the HE-MCS of the HE-SIG-B field and if DCM is applied to HE-SIG-B, which are indicated by the Number Of HE-SIG-B Symbols Or MU-MIMO Users field, HE-SIG-B-MCS field and HE-SIG-B-DCM field respectively in the HE-SIG-A field of an HE MU PPDU (see 27.3.11.7.2 (Content)).