IEEE P802.11
Wireless LANs

| GAS and RLQP comments - proposed resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-06-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes resolutions to REVmd D3.0 for the following CIDs:

4096, 4097, 4131 and 4596 regarding GAS and RLQP

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4096 | 11.23.3.4 | 2432 | 57 | What is a "protected Action frame"? Should this be a "Protected Dual of Public Action frame"? If that is true, then isn't the decision about whether RLQP is protected or not up to the regulatory domain, as opposed to the protocol (i.e. RLQP) itself? | Change the sentence "When RLQP is transmitted between the GDD dependent STA and its GDD enabling STA, it uses protected Action frames, but does not use Ethertype 89-0d frames"to"When RLQP is transmitted between the GDD dependent STA and its GDD enabling STA, it does not use Ethertype 89-0d frames. In some regulatory domains, RLQP may use Protected Dual of Public Action frames" |

Discussion

The “protected Action frame” appears to be a typo.

Regarding the phrase “When RLQP is transmitted between the GDD dependent STA and its

GDD enabling STA, it uses protected Action frames….”; it does appears to be out of line with the next couple of sentences in the same clause:

“In some regulatory domains, the GDD enabling STA may be required to have secured connection with the RLSS. When security is required by regulation, the Ethertype 89-0d frame body may employ corresponding security features.”

However, an improvement can be made by changing “may” to “might” in the cited and following sentence.

Proposed Resolution

Set the resolution for CID 4096 to the following:

Revised

Change the following two sentences:

"When RLQP is transmitted between the GDD dependent STA and its GDD enabling STA, it uses protected Action frames, but does not use Ethertype 89-0d frames.

In some regulatory domains, the GDD enabling STA may be required to have secured connection with the RLSS.”

to

"When RLQP is transmitted between the GDD dependent STA and its GDD enabling STA, it does not use Ethertype 89-0d frames. In some regulatory domains, RLQP might use Protected Dual of Public Action frames.

In some regulatory domains, the GDD enabling STA might be required to have secured connection with the RLSS.”

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4097 | 11.23.3.4 | 2432 | 53 | The start of this sentence is incorrect "The RLQP payload contains RLQP-elements", as it is GAS that contains RLQP-elements. | Change the start of the cited sentence to "The GAS payload contains RLQP-elements" |

Discussion

Appears to be a typo.

Proposed Resolution

Set the resolution for CID 4097 to the following:

Accept

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4131 | 11.43.4.3 | 2519 | 4 | Within the note, I think that the "Query Response" is a GAS Query Reponse, although it's really hard to tell. It seems to be mixing an Action Frame request "channel query request" with a RLQP-element response "Query Response". Indeed other parts of the TV whitespace procedures seem to be Action frames (CAQ frame) and GAS frames (e.g. CAQ RLQP-element), which I think is redundant. GAS can operate in both an unprotected and protected mode (with Protected Dual of Public Action frame) so why have this redundancy? | Change "Query Response" to "GAS Query Response" in two locations within the note. However, it's likely that a debate will be required to capture some other similar cases in the draft. |

Discussion

Within the cited note, the request and responses are Channel Availability Query RLQP-element request and Channel Availability Query RLQP-element response, as used in the earlier part of clause 11.43.4.3.

There doesn’t appear to be any redundancy in the note.

Proposed Resolution

Set the resolution for CID 4097 to the following:

Revised

Change:

“NOTE—For the CAQ process, the RLSS provides the list of available channels for the location(s) provided by the requesting STA in the Query Response. In some regulatory domains, the RLSS can perform access to a GDB to derive its response for channel query request and the associated information it provides in its Query Response.”

to

“NOTE—For the CAQ process, the RLSS provides the list of available channels for the location(s) provided by the requesting STA in the Channel Availability Query RLQP-element response. In some regulatory domains, the RLSS can perform access to a GDB to derive its response for the Channel Availability Query RLQP-element request and the associated information it provides in its Channel Availability Query RLQP-element response.”

| CID | Clause Number(C) | Page(C) | Line(C) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4596 | 9.4.2.92 | 1256 | 21 | Per 18/1371r0 Advertisement protocol ID 1 does not have a well-defined format so needs to be deprecated (probably also ID 2 has the same problem) | In Table 9-237---Advertisement protocol ID definitions at the end of the leftmost cell for values 1 and 2 append "(deprecated)". 19/0574 confirms this is the correct thing to do technically, and in IEEE technical correctness trumps political correctness |

Discussion

Submission 11-19-0574r6 discusses a similar CID 2402 and the resolution to that CID was:

“Rejected

These Advertisements Protocols are defined, and therefore still exist within IEEE 802.21-2017 -  they should not be deprecated.  A liaison or request could be sent to the IEEE 802.21 WG asking for assistance and clarification with these Advertisement Protocols format if one wished to pursue deprecation.”

This resolution equally applies to this CID.

Proposed Resolution

Set the resolution for CID 4596 to the following:

Rejected

These Advertisements Protocols are defined, and therefore still exist within IEEE 802.21-2017 -  they should not be deprecated.  A liaison or request could be sent to the IEEE 802.21 WG asking for assistance and clarification with these Advertisement Protocols format if one wished to pursue deprecation.