IEEE P802.11
Wireless LANs

| MAC CR Fragmentation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei | F3-06, Huawei Industrial Base, Bantian, Longgang District, Shenzhen, China | +86 15889743667 | ming.gan@huawei.com |

Abstract

This submission proposes resolutions of comments received for TGax draft 6.0. (The proposed change is based on TGax Draft 6.0)

- 24364 24553 24554 24557  (4 CIDs)

Revisions:

- Rev 0: Initial version of the document.

- Rev 4: Resolution change for CID 24557 and some editorial change with yellow color

- Rev 5: Correct the reference for the resolution change for CID 24557 based on Mark’s comment

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGax Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGax Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGax Editor: Editing instructions preceded by “TGax Editor” are instructions to the TGax editor to modify existing material in the TGax draft.  As a result of adopting the changes, the TGax editor will execute the instructions rather than copy them to the TGax Draft.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 24364 | 326.20 | 26.3.1 | The dynamic fragmentation levels are not not clear. "Level 1: support for one dynamic fragment that is a non-A-MPDU" -- support where? Within a given PSDU? For the lifetime of the MSDU/A-MSDU/MMPDU being fragmented? "Level 3: support for up to 4 dynamic fragments for each MSDU and each A-MSDU (if supported by the recipient) in an A-MPDU and up to one dynamic fragment for one MMPDU (see 26.6.3 (Multi-TID A-MPDU and ack-enabled single-TID A-MPDU)) in an A-MPDU that does not contain an S-MPDU" -- again support where/in what context/within what scope? | As it says in the comment | Revised- Agree with commenter in principle. Propose resolution to address this issue. TGax editor please make the changes as shown in 11-20/0981 r3 under CID 24364 |
| 24553 | 326.30 | 26.3.1 | A statement is needed in the General subclause specifying that, under level 3 fragmentation, the maximum number of fragments that can be sent across A-MPDUs for this TID, is 4 (i.e., FN cannot be set to a value greater than 3). This rule of course supersedes the Maximum Number of Fragments Exponent value. | As in comment. | Revised- No motivation to supersede the Maximum Number Of Fragmented MSDUs/A-MSDUs Exponent. Now the maximum number of dynamic fragments in an A-MPDU is calculated based on the Maximum Number Of Fragmented MSDUs/A-MSDUs Exponent. But to make the field name correct, propose resolution to change “Maximum Number of Fragments Exponent” to “Maximum Number Of Fragmented MSDUs/A-MSDUs Exponent”. TGax editor please make the changes as shown in 11-20/0981 r3 under CID 24553 |
| 24554 | 329.22 | 26.3.2.4 | Rule that limits WinEndO-WinStartO to be within BL/4 is missing for fragmentation level 3. Please add here that a level 3 fragmentation BA session uses a WinSize that does not exceed BL/4 for the BA session that corresponds to that TID. | As in comment. | Revised- The parameter BitmapLength is derived by dividing by 4, the length represented in the Block Ack Bitmap subfield in the Compressed BlockAck frame and Multi-STA BlockAck frame for a particular TID. And the WinSizeR is set to the smaller of BitmapLength and the value of the Buffer Size field of the associated ADDBA Response frame. Change the WinSizeO setting as per the comment TGax editor please make the changes as shown in 11-20/0981 r3 under CID 24554 |
| 24557 | 327.49 | 26.3.2.1 | PN setting rules for dynamic fragmentation with different size retransmissions are unclear. Please clarify them. | As in comment. | Revised- Agree with the commenter in principle, propose resolution to clarify the PN settting in this case. TGax editor please make the changes as shown in 11-20/0981 r5 under CID 24557 |

Discussion: …

26.3 Fragmentation and defragmentation

26.3.1 General

An HE STA shall follow the fragmentation procedures defined in 10.2.6 (Fragmentation/defragmentation overview), 10.4 (MSDU, A-MSDU, and MMPDU fragmentation), and 10.5 (MSDU, A-MSDU, and MMPDU defragmentation) except that some of the rules are relaxed as defined in the subclauses below. Dynamic fragmentation provides further flexibility in aggregating data to fit in a constrained PPDU duration (see 26.5 (MU operation)).

Subclauses 10.2.6 (Fragmentation/defragmentation overview) and 10.4 (MSDU, A-MSDU, and MMPDU fragmentation) define the procedure to generate uniformly fragmented MSDUs or MMPDUs, where the length of each fragment is the same, except for the last fragment. These fragments are not included in an A-MPDU under HT-immediate block ack agreements.

An HE STA can negotiate the use of different levels of dynamic fragmentation:

— Level 1: support for one dynamic fragment that is a non-A-MPDU, no support for dynamic fragments in an A-MPDU that does not contain an S-MPDU.

— Level 2: support for dynamic fragment(s) in an A-MPDU that does not contain an S-MPDU, subject to the following conditions

— No more than one dynamic fragment of any given MSDU or A-MSDU in the A-MPDU, and the MSDU or A-MSDU shall be under a block ack agreement.

— No more than one dynamic fragment of an MMPDU in the A-MPDU (see 26.6.2 (Ack-enabled single-TID A-MPDU operation) and 26.6.3 (Multi-TID A-MPDU and ack-enabled single-TID A-MPDU)).

— Level 3: support for dynamic fragment(s) in an A-MPDU that does not contain an S-MPDU, subject to the following conditions

— No more than four dynamic fragments of any given MSDU or A-MSDU in the A-MPDU, and the MSDU or A-MSDU shall be under a block ack agreement.

— No more than one dynamic fragment of an MMPDU in the A-MPDU (see 26.6.2 (Ack-enabled single-TID A-MPDU operation) and 26.6.3 (Multi-TID A-MPDU and ack-enabled single-TID A-MPDU)). (#24364).

NOTE—An HE STA that is operating in Level 2 or Level 3 can still transmit and receive the dynamic fragments under Level 1.

9.4.2.247.2 HE MAC Capabilities Information field

Table 9-262z—Subfields of the HE MAC Capabilities Information field

| Subfield | Definition | Encoding |
| --- | --- | --- |
| … | … | … |
| Dynamic Fragmentation Support | Indicates the level of dynamic fragmentation that is supported by a STA as a recipient. | Set to 0 if the STA does not support dynamic fragmentation. Set to 1 if the STA supports level 1 dynamic fragmentation (see 26.3.1 General). Set to 2 if the STA supports level 2 dynamic fragmentation (see 26.3.1 General). Set to 3 if the STA supports level 3 dynamic fragmentation (see 26.3.1 General). |

26.3.2 Dynamic fragmentation

26.3.2.1 General

With level 1 and level 2 dynamic fragmentation, the following apply:

— An originator STA transmitting one or more dynamic fragments shall solicit an immediate response from the recipient STA for each of the fragments.

— The originator STA shall transmit the fragments in order as defined in 10.4 (MSDU, A-MSDU, and MMPDU fragmentation).

With level 3 dynamic fragmentation, not all dynamic fragments require an immediate response and dynamic fragments are not required to be sent in order.

An HE STA may transmit dynamic fragments of an A-MSDU provided the A-MSDU Fragmentation Sup-port subfield of the HE Capabilities element transmitted by the recipient is 1.

The originator STA shall not transmit concurrently the dynamic fragments of a number of outstanding MSDUs and A-MSDUs (if supported) to the same recipient STA that is greater than Nmax, where Nmax for MSDUs and A-MSDUs (if supported) is calculated based on the Maximum Number Of Fragmented MSDUs/A-MSDUs Exponent (#24553) subfield of the HE Capabilities element transmitted by the recipient STA. The term outstanding refers to an MPDU containing all or part of an MSDU or A-MSDU for which transmission has been started, and for which delivery of the MSDU or A-MSDU has not yet been completed (i.e., an acknowledgment of the final fragment has not been received and the MSDU, A-MSDU or MMPDU has not been discarded due to retries, lifetime, or for some other reason).

An originator STA may retransmit the full MSDU, A-MSDU or MMPDU if all the previously transmitted dynamic fragments of that MSDU, A-MSDU or MMPDU have explicitly failed at the receiving STA. An originator STA may retransmit a failed fragment if one or more of the previously transmitted fragments of that MSDU, A-MSDU, or MMPDU have explicitly failed at the receiving STA (see below for a definition of this term). The frame body length and contents of the retransmitted fragment shall be the same as the initially transmitted fragment and shall remain fixed for the lifetime of the MSDU, A-MSDU or MMPDU at that STA except when all the fragments preceding the initial transmitted fragment were received and all the fragments following the initial transmitted fragment have either explicitly failed or have not been transmitted, in which case the frame body length and contents of the retransmitted fragment may be different from the initially transmitted fragment and the PN assignment for the retransmitted fragment shall follow the rules defined in 12.5.3.3.2 (PN processing) and 12.5.5.3.2 (PN processing) except that the PN shall be incremented for the retransmitted fragment if it has a different body length from the previously transmitted fragment and is encrypted.(# 24557)

26.3.2.4 Level 3 dynamic fragmentation

…

Level 3 fragmentation allows multiple fragments of an MSDU or A-MSDU included in the same A-MPDU, reducing the fragments’ transmission delay. An originator STA may transmit to a recipient STA the following:

— One dynamic fragment of an MSDU, A-MSDU (if supported by the recipient), or MMPDU that is a non-A-MPDU using level 1 dynamic fragmentation.

• The originator STA shall follow the rules defined in 10.12.8 (Transport of S-MPDUs) for gener-ating the S-MPDU.

— Up to four dynamic fragments of an MSDU or A-MSDU (if supported by the recipient) for each MSDU or A-MSDU, and up to one dynamic fragment of an MMPDU in an A-MPDU, where the A-MPDU contains at least one dynamic fragment using level 3 dynamic fragmentation under block ack agreement.

• The originator STA shall set the Fragment Number subfield of each MPDU to a value less than 4.

• The originator STA shall follow the rules defined in 10.25.6.7 (Originator’s behavior) for gener-ating the A-MPDU with the exception that the A-MPDU shall contain MPDUs whose range of the Sequence Number subfields does not exceed BL/4, where BL is the length of the Block Ack Bitmap field of the Compressed (#) BlockAck or Multi-STA BlockAck frame that corresponds to a TID of a transmitted fragment (see 10.25.6 (HT-immediate block ack extensions) and 26.4 (HE acknowl-edgment procedure)).

• The originator STA shall ensure that WinSizeO does not exceed BL /4 , where BL is the length of the Block Ack Bitmap field of the Compressed  BlockAck or Multi-STA BlockAck frame that corresponds to a TID of a transmitted fragment (see 10.25.6 (HT-immediate block ack extensions) and 26.4 (HE acknowl-edgment procedure)). (#24554)